3GPP TSG-SA4 Meeting #129-e	S4-251484

# Online, 2024-08-19- 2024-08-23

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.997 | CR | 0001 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to TR 26.997 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Dolby Sweden AB, Ericsson LM, Fraunhofer IIS, Huawei Technologies Co Ltd., Nokia, NTT, Orange, Panasonic Holdings Corporation, Philips International B.V., Qualcomm Incorporated, VoiceAge Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IVAS_Codec, ISAR, TEI18 |  |  |  |  |  | Date: |  |  | 2024-08-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Corrections of erroneous information, addition of missing information |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correction of references - Correction and harmonization of upper-/lower-case usage of statistical parameter “t” in clauses 9.11.4.4, 9.11.6.4, 9.11.7.4 - Addition of missing information on low-bitrate ISAR operation points. This information has been provided to SA4#128 and it was agreed in the Audio SWG at SA4#128 to include this information into TR 26.997, however the inclusion was omitted for version 18.0.0. This information is considered an important part of TR 26.997, since it characterizes the split rendering solution at bitrates of 384 and 512 kbit/s. Otherwise, these characterization at these bitrates would be completely missing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Unclear or wrong text, which might lead to misunderstandings. - Missing important information on split rendering feature of IVAS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 9.11.2, 9.11.4.4, 9.11.6.4, 9.11.7.4, 9.11.8, 9.11.9, 9.11.10, 9.11.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE 1

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.250: "Codec for Immersive Voice and Audio Services (IVAS); General overview".

[3]	void

[4]	3GPP TS 26.252: "Codec for Immersive Voice and Audio Services (IVAS); Test Sequences".

[5]	3GPP TS 26.253: "Codec for Immersive Voice and Audio Services (IVAS); Detailed Algorithmic Description including RTP payload format and SDP parameter definitions".

[6]	3GPP TS 26.254: "Codec for Immersive Voice and Audio Services (IVAS); Rendering".

[7]	3GPP TS 26.255: "Codec for Immersive Voice and Audio Services (IVAS); Error concealment of lost packets".

[8]	3GPP TS 26.256: "Codec for Immersive Voice and Audio Services (IVAS); Jitter buffer management".

[9]	3GPP TS 26.258: "Codec for Immersive Voice and Audio Services (IVAS); C code (floating-point)".

[10]	Recommendation ITU-T P.800: "Methods for subjective determination of transmission quality".

[11]	Supplement ITU-T P.Suppl29: "ITU-T P.800 – Use Cases".

[12]	Recommendation ITU-R BS.1534-3: "Method of the subjective assessment of intermediate quality level of audio systems".

[13]	ITU-T Handbook of subjective testing practical procedures, 2011.

[14]	Audio File Format Specifications: WAVE, https://www-mmsp.ece.mcgill.ca/Documents/AudioFormats/WAVE/WAVE.html.

[15]	AFsp Package https://www-mmsp.ece.mcgill.ca/Documents/Downloads/AFsp/.

[16]	IEEE Recommended Practice for Speech Quality Measurements, in IEEE Transactions on Audio and Electroacoustics, vol. 17, no. 3, pp. 225-246, September 1969, doi: 10.1109/TAU.1969.1162058.a.

[17]	Recommendation ITU-R BS.2051-3 (05/2022): Advanced sound system for programme production.

[18]	3GPP TR 26.952: "Codec for Enhanced Voice Services (EVS); Performance characterization".

[19]	3GPP TS 26.445: "Codec for Enhanced Voice Services (EVS); Detailed Algorithmic Description".

[20]	3GPP TR 26.996: "Immersive Audio for Split Rendering Scenarios; Performance characterization".

[21]	Recommendation ITU-T P.191 (03/2023): Software tools for speech and audio coding standardization.

[22]	Recommendation ITU-R BS.1770-4 (10/2015): Algorithms to measure audio programme loudness and true-peak audio level.

CHANGE 2

### 9.11.2	Overview

In the ISAR Selection phase, four experiments have been conducted to evaluate the performance of the IVAS specific ISAR solution. Table ‎9.11-1shows a high-level overview of the experiments. Each experiment was carried out twice (in experiments a and b), once by the solution proponent and once by a cross-checker (XC).

Table ‎9.11-1: High-level overview of ISAR selection experiments

| Exp | Input format | Source material | Listening environment | Bitrates (kbps) | Listening Lab |
| --- | --- | --- | --- | --- | --- |
| ISAR_BS1534-1a ISAR_BS1534-1b | SBA (HOA3) | Generic Audio | Headphones | IVAS: 512, CuT: 768 | Dolby Qualcomm (XC) |
| ISAR_BS1534-2a ISAR_BS1534-2b | Multi-channel 7.1+4 | Generic Audio | Headphones | IVAS: 512, CuT: 768 | Fraunhofer Ittiam (XC) |
| ISAR_BS1534-3a ISAR_BS1534-3b | Objects (ISM-4) | Generic Audio | Headphones | IVAS: 512, CuT: 768 | Fraunhofer Nokia (XC) |
| ISAR_BS1534-4a ISAR_BS1534-4b | MASA (2 TC) | Generic Audio | Headphones | IVAS: 512, CuT: 768 | Dolby Bytedance (XC) |

In addition to the selection tests evaluating the performance of the single ISAR candidate solution for IVAS described above, analogous experiments were carried out with the selected ISAR solution operated at 384 and 512 kbps. These experiments were provided by a company contribution and are summarized in Table 9.11-1a.

Table 9.11-1a: High-level overview of ISAR low-rate experiments

| Exp | Input format | Source material | Listening environment | Bitrates kbps | Listening Lab |
| --- | --- | --- | --- | --- | --- |
| ISAR_BS1534-1-LR | SBA (HOA3) | Generic Audio | Headphones | IVAS: 512, CuT: 384 IVAS: 512, CuT: 512 | Dolby |
| ISAR_BS1534-2-LR | Multi-channel 7.1+4 | Generic Audio | Headphones | IVAS: 512, CuT: 384 IVAS: 512, CuT: 512 | Dolby |
| ISAR_BS1534-3-LR | Objects (ISM-4) | Generic Audio | Headphones | IVAS: 512, CuT: 384 IVAS: 512, CuT: 512 | Dolby |
| ISAR_BS1534-4-LR | MASA (2 TC) | Generic Audio | Headphones | IVAS: 512, CuT: 384 IVAS: 512, CuT: 512 | Dolby |

CHANGE 3

#### 9.11.4.4	Statistical analysis

Table ‎9.11-2: Result of statistical analysis of ISAR_BS1534-1a test checking CuT NWT 0-DOF Reference

| Mean Diff. (c03 - c04) | Stdev Diff. | SEMD | t | Prob. | ToR |
| --- | --- | --- | --- | --- | --- |
| -17.9917 | 5.7941 | 0.5289 | -34.0156 | 1.0000 | Pass |

Table ‎9.11-3: Result of statistical analysis of ISAR_BS1534-1b test checking CuT NWT 0-DOF Reference

| Mean Diff. (c03 - c04) | Stdev Diff. | SEMD | t | Prob. | ToR |
| --- | --- | --- | --- | --- | --- |
| -36.6583 | 20.3336 | 1.8562 | -19.7492 | 1.0000 | Pass |

CHANGE 4

#### 9.11.6.4	Statistical analysis

Table ‎9.11-6: Result of statistical analysis of ISAR_BS1534-3a test checking CuT NWT 0-DOF Reference

| Mean Diff. (c03 - c04) | Stdev Diff. | SEMD | t | Prob. | ToR |
| --- | --- | --- | --- | --- | --- |
| -26.6500 | 14.7310 | 1.3448 | -19.8178 | 1.0000 | Pass |

Table ‎9.11-7: Result of statistical analysis of ISAR_BS1534-3b test checking CuT NWT 0-DOF Reference

| Mean Diff. (c03 - c04) | Stdev Diff. | SEMD | t | Prob. | ToR |
| --- | --- | --- | --- | --- | --- |
| -32.4083 | 22.0761 | 2.0153 | -16.0814 | 1.0000 | Pass |

CHANGE 5

#### 9.11.7.4	Statistical analysis

Table ‎9.11-8: Result of statistical analysis of ISAR_BS1534-4a test checking CuT NWT 0-DOF Reference

| Mean Diff. (c03 - c04) | Stdev Diff. | SEMD | t | Prob. | ToR |
| --- | --- | --- | --- | --- | --- |
| -14.0167 | 5.0493 | 0.4609 | -30.4091 | 1.0000 | Pass |

Table ‎9.11-9: Result of statistical analysis of ISAR_BS1534-4b test checking CuT NWT 0-DOF Reference

| Mean Diff. (c03 - c04) | Stdev Diff. | SEMD | t | Prob. | ToR |
| --- | --- | --- | --- | --- | --- |
| -13.7000 | 22.1500 | 2.0220 | -6.7754 | 1.0000 | Pass |

CHANGE 6

### 9.11.8	ISAR Experiment ISAR_BS1534-1-LR (HOA3, Generic Audio, 384 and 512 kbps Split Rendering Link, Headphone Presentation)

#### 9.11.8.1	Overview

The experiment ISAR_BS1534-1 evaluates Split Rendering for IVAS HOA3 audio at 384 and 512 kbps.

#### 9.11.8.2	Test conditions

The test conditions in all experiments are as follows:

-	c01 (REF): Hidden reference: Native coding system (IVAS at 512kbps rendered to post renderer pose)

-	c02 (LP7): LP7 anchor: Hidden reference, 7 kHz LP filtered

-	c03 (0-DOF): 0-DOF native transcoding reference (IVAS at 512kbps binaurally rendered to pre-renderer pose, IVAS stereo coded@256kbps)

-	c04 (CuT1): 3-DOF system 1 under test (IVAS at512 kbps split-rendered with ISAR operating at 512 kbps)

-	c05 (CuT2): 3-DOF system 1 under test (IVAS at 512 kbps split-rendered with ISAR operating at 384 kbps)

#### 9.11.8.3	Result plots

Provided below are the result plots and tables with statistical analysis result for the ISAR_BS1534-1-LR experiment.

Figure ‎9.11-9: Results of ISAR_BS1534-1-LR test for SBA input audio

#### 9.11.8.4	Statistical analysis

Table 9.11-10: Result of statistical analysis of BS1534-1-LR test checking CuT1 NWT 0-DOF Reference

| Mean Diff. (c03 - c04) | Stdev Diff. | SEMD | t | Prob. | ToR |
| --- | --- | --- | --- | --- | --- |
| -18.3021 | 14.2564 | 1.4550 | -12.5784 | 1.0000 | Pass |

Table 9.11-11: Result of statistical analysis of BS1534-1-LR test checking CuT2 NWT 0-DOF Reference

| Mean Diff. (c03 - c05) | Stdev Diff. | SEMD | t | Prob. | ToR |
| --- | --- | --- | --- | --- | --- |
| -18.6354 | 14.3455 | 1.4641 | -12.7280 | 1.0000 | Pass |

#### 9.11.8.5	Experimental conclusions

Conclusion of the low-rate experiment ISAR_BS1534-1-LR is that the ISAR solution for IVAS, when operated at 384 and 512 kbps, respectively, still clearly meets the requirement that it shall be no worse than the 0-DOF transcoding reference system. It can be observed that the achievable quality is even clearly better whereby a quality level in the ‘excellent’ range close to the quality of the native IVAS coding reference system is achieved. In contrast, the 0-DOF transcoding alternative offers substantially lower quality.

The objective to provide a quality level as close as possible to the native coding reference system is met in the sense that the quality score of the split rendering system is in the high ‘excellent’ range, which indicates only very minor audible differences.

Despite the lower bit rates, this conclusion is the same as for the highest tested bit rate of 784 kbps.

### 9.11.9	ISAR Experiment ISAR_BS1534-2-LR (MC 7.1+4, Generic Audio, 384 and 512 kbps Split Rendering Link, Headphone Presentation)

#### 9.11.9.1	Overview

The experiment ISAR_BS1534-2-LR evaluates Split Rendering for IVAS 7.1+4 audio at 384 and 512 kbps.

#### 9.11.9.2	Test conditions

The test conditions in all experiments are as follows:

-	c01 (REF): Hidden reference: Native coding system (IVAS at 512kbps rendered to post renderer pose)

-	c02 (LP7): LP7 anchor: Hidden reference, 7 kHz LP filtered

-	c03 (0-DOF): 0-DOF native transcoding reference (IVAS at 512kbps binaurally rendered to pre-renderer pose, IVAS stereo coded@256kbps)

-	c04 (CuT1): 3-DOF system 1 under test (IVAS at512 kbps split-rendered with ISAR operating at 512 kbps)

-	c05 (CuT2): 3-DOF system 1 under test (IVAS at 512 kbps split-rendered with ISAR operating at 384 kbps)

#### 9.11.9.3	Result plots

Provided below are the result plots and tables with statistical analysis result for the ISAR_BS1534-2 experiment.

Figure ‎9.11-10: Results of ISAR_BS1534-2-LR test for MC 7.1.4 input audio

#### 9.11.9.4	Statistical analysis

Table 9.11-12: Result of statistical analysis of BS1534-2-LR test checking CuT1 NWT 0-DOF Reference

| Mean Diff. (c03 - c04) | Stdev Diff. | SEMD | t | Prob. | ToR |
| --- | --- | --- | --- | --- | --- |
| -18.4479 | 4.4506 | 0.4542 | -40.6130 | 1.0000 | Pass |

Table 9.11-13: Result of statistical analysis of BS1534-2-LR test checking CuT2 NWT 0-DOF Reference

| Mean Diff. (c03 - c05) | Stdev Diff. | SEMD | t | Prob. | ToR |
| --- | --- | --- | --- | --- | --- |
| -17.3542 | 5.0304 | 0.5134 | -33.8016 | 1.0000 | Pass |

#### 9.11.9.5	Experimental conclusions

Conclusion of the low-rate experiment ISAR_BS1534-2-LR is that the ISAR solution for IVAS, when operated at 384 and 512 kbps, respectively, still clearly meets the requirement that it shall be no worse than the 0-DOF transcoding reference system. It can be observed that the achievable quality is even clearly better whereby a quality level in the ‘excellent’ range close to the quality of the native IVAS coding reference system is achieved. In contrast, the 0-DOF transcoding alternative offers substantially lower quality.

The objective to provide a quality level as close as possible to the native coding reference system is met in the sense that the quality score of the split rendering system is in the high ‘excellent’ range, which indicates only very minor audible differences.

Despite the lower bit rates, this conclusion is the same as for the highest tested bit rate of 784 kbps.

### 9.11.10	ISAR Experiment ISAR_BS1534-3-LR (4 Objects, Generic Audio, 384 and 512 kbps Split Rendering Link, Headphone Presentation)

#### 9.11.10.1	Overview

The experiment ISAR_BS1534-3-LR evaluates Split Rendering for 4 object audio (ISM4) at 384 and 512 kbps.

#### 9.11.10.2	Test conditions

The test conditions in all experiments are as follows:

The test conditions in all experiments are as follows:

-	c01 (REF): Hidden reference: Native coding system (IVAS at 512kbps rendered to post renderer pose)

-	c02 (LP7): LP7 anchor: Hidden reference, 7 kHz LP filtered

-	c03 (0-DOF): 0-DOF native transcoding reference (IVAS at 512kbps binaurally rendered to pre-renderer pose, IVAS stereo coded@256kbps)

-	c04 (CuT1): 3-DOF system 1 under test (IVAS at512 kbps split-rendered with ISAR operating at 512 kbps)

-	c05 (CuT2): 3-DOF system 1 under test (IVAS at 512 kbps split-rendered with ISAR operating at 384 kbps)

#### 9.11.10.3	Result plots

Provided below are the result plots and tables with statistical analysis result for the ISAR_BS1534-3-LR experiment.

Figure ‎9.11-11: Results of ISAR_BS1534-3a test for ISM-4 input audio

#### 9.11.6.4	Statistical analysis

Table 9.11-14: Result of statistical analysis of BS1534-4-LR test checking CuT1 NWT 0-DOF Reference

| Mean Diff. (c03 - c04) | Stdev Diff. | SEMD | t | Prob. | ToR |
| --- | --- | --- | --- | --- | --- |
| -15.0938 | 10.4419 | 1.0657 | -14.1629 | 1.0000 | Pass |

Table 9.11-15: Result of statistical analysis of BS1534-3-LR test checking CuT2 NWT 0-DOF Reference

| Mean Diff. (c03 - c05) | Stdev Diff. | SEMD | t | Prob. | ToR |
| --- | --- | --- | --- | --- | --- |
| -13.6771 | 9.9202 | 1.0125 | -13.5085 | 1.0000 | Pass |

#### 9.11.8.5	Experimental conclusions

Conclusion of the low-rate experiment ISAR_BS1534-3-LR is that the ISAR solution for IVAS, when operated at 384 and 512 kbps, respectively, still clearly meets the requirement that it shall be no worse than the 0-DOF transcoding reference system. It can be observed that the achievable quality is even clearly better whereby a quality level in the ‘excellent’ range close to the quality of the native IVAS coding reference system is achieved. In contrast, the 0-DOF transcoding alternative offers substantially lower quality.

The objective to provide a quality level as close as possible to the native coding reference system is met in the sense that the quality score of the split rendering system is in the high ‘excellent’ range, which indicates only very minor audible differences.

Despite the lower bit rates, this conclusion is the same as for the highest tested bit rate of 784 kbps.

### 9.11.11	ISAR Experiment ISAR_BS1534-4-LR (MASA 2TC, Generic Audio, 384 and 512 kbps Split Rendering Link, Headphone Presentation)

#### 9.11.11.1	Overview

The experiment ISAR_BS1534-4-LR evaluates Split Rendering for IVAS MASA audio using 2 transport channels (TC) at 384 and 512 kbps.

#### 9.11.11.2	Test conditions

The test conditions in all experiments are as follows:

-	c01 (REF): Hidden reference: Native coding system (IVAS at 512kbps rendered to post renderer pose)

-	c02 (LP7): LP7 anchor: Hidden reference, 7 kHz LP filtered

-	c03 (0-DOF): 0-DOF native transcoding reference (IVAS at 512kbps binaurally rendered to pre-renderer pose, IVAS stereo coded@256kbps)

-	c04 (CuT1): 3-DOF system 1 under test (IVAS at512 kbps split-rendered with ISAR operating at 512 kbps)

-	c05 (CuT2): 3-DOF system 1 under test (IVAS at 512 kbps split-rendered with ISAR operating at 384 kbps)

#### 9.11.11.3	Result plots

Provided below are the result plots and tables with statistical analysis result for the ISAR_BS1534-4-LR experiment.

Figure ‎9.11-12: Results of ISAR_BS1534-4a test for MASA input audio

#### 9.11.11.4	Statistical analysis

Table 9.11-16: Result of statistical analysis of BS1534-4-LR test checking CuT1 NWT 0-DOF Reference

| Mean Diff. (c03 - c04) | Stdev Diff. | SEMD | t | Prob. | ToR |
| --- | --- | --- | --- | --- | --- |
| -13.0104 | 5.7647 | 0.5884 | -22.1132 | 1.0000 | Pass |

Table 9.11-17: Result of statistical analysis of BS1534-4-LR test checking CuT2 NWT 0-DOF Reference

| Mean Diff. (c03 - c05) | Stdev Diff. | SEMD | t | Prob. | ToR |
| --- | --- | --- | --- | --- | --- |
| -11.8958 | 5.6819 | 0.5799 | -20.5134 | 1.0000 | Pass |

#### 9.11.11.5	Experimental conclusions

Conclusion of the low-rate experiment ISAR_BS1534-4-LR is that the ISAR solution for IVAS, when operated at 384 and 512 kbps, respectively, still clearly meets the requirement that it shall be no worse than the 0-DOF transcoding reference system. It can be observed that the achievable quality is even clearly better whereby a quality level in the ‘excellent’ range close to the quality of the native IVAS coding reference system is achieved. In contrast, the 0-DOF transcoding alternative offers substantially lower quality.

The objective to provide a quality level as close as possible to the native coding reference system is met in the sense that the quality score of the split rendering system is in the high ‘excellent’ range, which indicates only very minor audible differences.

Despite the lower bit rates, this conclusion is the same as for the highest tested bit rate of 784 kbps.

END OF CHANGES