Agenda item: 	9.8

Source: 	Qualcomm Inc.

Title: 	[FS_AVATAR] On the Message Format and Carriages for Skeletal Information for Animation

Document for	Agreement

# Introduction

For Avatar animation, it is important to convey the skeletal pose information to the animator.

There are skeletal representations defined in OpenXR (XR_FB_body_tracking) and Unity (SkeletonBone), and MPEG is also developing an Avatar Representation Format with support for skinning. These representations are good for local animation. But for remote animation, the size of the representation may be too big and pose a strain on the communication network. For example, the skeleton representation in XR_FB_body_tracking comprises 70 joints, each with 4 floating-point (32-bit) numbers for the orientation and 3 floating-point numbers for the position. This means a total size of 1960 bytes. It is important to have a compact representation of the skeletal information and choose the appropriate carriage for transport.

# Compact Skeletal representation

The joints (and bones) are not independent and rather follow a hierarchical structure. The hierarchy may be leveraged to reduce the size of the representation. The hierarchy has been used in OpenXR and Unity. Additionally, differential encoding, i.e., encoding an offset from a reference for a joint (e.g., in the T-pose).

Proposal 1: FS_AVATAR to study compact skeletal representation and fill any gap in the current representations produced by other standards bodies.

# Carriage mechanism

Potential carriages of the skeletal information include the RTP header extension, the RTCP packet, and the metadata data channel message.

TS 26.522 defined an RTP header extension to carry a single pose (the so called XR pose) and actions. The size of the pose is 28 bytes and is appropriate for the RTP header extension. For the skeletal information, even with size reduction, the size may be still too large to fit into an RTP header extension.

RTCP can accommodate a larger payload. The skeletal information needs to be sent frequently, on a per frame basis, but RTCP is subject to bandwidth limitation (e.g., not more than 5% of the total bandwidth) and may not be able to support such frequent and large-size transmissions.

TS26.565 defined the metadata data channel messages. The data channel can easily support large-size data and has inherent mechanisms to handle reordering, loss recovery and latency. Given the large size of the skeletal information and the frequent transmissions, a metadata data channel message is the best carriage. Currently TS26.565 does not support skeletal information. A new metadata channel message is needed for skeletal information.

Proposal 2: FS_AVATAR to define a new metadata channel message for skeletal information.

# Proposal

We propose to add clauses 1, 2 and 3 to the FS_AVATA TR.