3GPP SA4 #130	S4-241900

Orlando, FL, US, November 18-22, 2024

Work item: FS_5G_RTP_Ph2

Title: [FS_5G_RTP_Ph2] On the PSSN for the lone PDU

Qualcomm Incorporated

- Introduction

According to clause 5.37.5 of TS23.501 [1], on handling the lone PDU in the DL, the UPF still maps a lone PDU to a PDU Set without specifying how to set the PSSN, and what is specified is to ‘determine’ the PDU Set Information which includes the PSSN. Specifically, clause 5.37.5.1 of TS23.501 [3] states:

If the PSA UPF receives a PDU that does not belong to a PDU Set based on Protocol Description for PDU Set identification, then the PSA UPF still maps it to a PDU Set and determines the PDU Set Information as described in clause 5.37.5.2.

- Analysis

In [2], it is pointed out that if the UPF assigns consecutive PSSNs to the lone PDUs and PDU Sets as they arrive in the downlink, the UPF will have to translate the PSSNs of PDU Sets, resulting in the following issues

- Unnecessary PSSN translation at the UPF

- The UPF complexity for maintaining state information to convey correct PDU Set correlation information for PDU Sets within the same QoS flow

- Breaking media dependence information in the case of out-of-order delivery

The above in turn may result in added jitter or reduced troughput to the media.

This PSSN assignment method also has a problem for multi-modality synchronization currently under consideration in RAN2 and SA2 (see R2-2409272 [3]), as the assingment may break the dependence information among PDU Sets in different QoS flows of modalities.

Observation: To avoid added jitter or reduced troughput and not to corrupt PDU Set dependence information, and to reduce the burden of UPF complexity, it is desirable that the UPF does not translate the PSSNs of arriving PDU Sets.

- Potential solutions

Solution #15 (clause 6.15) proposed two solutions to assigning a PSSN to a lone PDU:

PSSN is trickier since it depends on the transmission order of the PDU Set by the sender and is monotonically increased by the sender by 1 for each subsequent PDU Set. Since there is no gap between the PSSNs assigned by the sender for the RTP PDUs, the UPF would have to either assign an existing PSSN (i.e., a PSSN that is already used by another PDU Set) to the new PDU Set that contains the lone PDU, or a predetermined value that indicates that PSSN is undefined for that PDU Set.

Between the two potential solutions, the second solution (a predetermined value for lone PDUs) is better because the first solution (reusing a PSSN) leads to two PDU Sets having the same PSSN.

A third solution, proposed in [4], is to divide the PSSN space into two sub-spaces, one for (marked) PDU Sets and the other for lone PDUs. This allows for sequencing the lone PDUs as well.

- Proposal

### It is proposed to incorporate the following proposal to the conclusion clause of TR 26.822.

### Proposal: consider guidelines on using a pre-determined PSSN (e.g., 0x3FF) for lone PDUs or dividing the PSSN sequence space into sub-spaces for PDU Sets and lone PDUs.

NOTE: the above proposal avoids the potential discrepancy between the PSSN signaled in the control plane and the PSSN in the user plane discussed in [5].

### References

### [1] 3GPP TS 23.501: "System architecture for the 5G System (5GS)".

### [2] S4aR240097, “On the PSSN for the lone PDU”, 3GPP SA4-e (AH) RTC SWG post 129-e, Huawei, Oct 23, 2024.

### [3] R2-2409272, “Reply to SA2 LS on multi-modality awareness”, 3GPP TSG-RAN WG2 Meeting #127bis, Hefei, China, Oct 14-18, 2024.

### [4] S4aR240088, “Solution KI#2 lone/unmarked PDU without SSRC packet filter in 5G System”, 3GPP SA4-e (AH) RTC SWG post 129-e, Huawei, Oct 23, 2024.

### [5] S4-241901, “Control-Plane Solution to the Key Issue on Enhancements of Data Burst Marking”, 3GPP SA4 Meeting #130, Orlando, FL, Nov 18-22, 2024.