3GPP TSG-SA WG4 Meeting post 130	S4-242027

Orlando,  18th - 22nd November 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | pCR |  | rev | - | Current version: | 0.3.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ### Potential solution to Key Issue #3: Evaluation framework |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_MediaEnergyGREEN |  |  |  |  |  | Date: |  |  | 2024-11-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The latest draft of 3GPP TR 26.942 contains Key Issue #3: Evaluation Framework under clause 6.3. Add a potential solution for KI#3 Evaluation Framework |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes new solution based on ETSI ES 203 228 V1.3.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Proposed objectives will not be met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2, 7.2.1, 7.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 1st Change

# 2	References

[ES 203 228]	ETSI ES 203 228 V1.3.1: " Environmental Engineering (EE); Assessment of mobile network energy efficiency, 10/2020.

[ES 202 336] 	ETSI ES 202 336-12: "Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model".

## 2nd Change

## 7	Potential solutions

Editor’s note: Description of potential solutions.

## 7.1	Mapping of solutions to Key Issues

Table 7.1-1: Mapping of solutions to Key Issues

| Solutions |  |  |  |
| --- | --- | --- | --- |
|  | KI#1 | KI#2 | KI#3 |
| #1 |  |  |  |
| #2 |  |  | X |
| #3 |  |  |  |
| #4 |  |  |  |
| #5 |  |  |  |
| #6 |  |  |  |
| #7 |  |  |  |
| #8 |  |  |  |
| #9 |  |  |  |

## 3rd Change (all new text)

## 7.2	Solution #2: Evaluation Framework to measure energy efficiency of a UE

### 7.2.1	Key Issue mapping

This solution addresses Key Issue #3 (Evaluation Framework) described in clause 6.3.

### 7.2.2	Functional description

#### 7.2.2.1	Definition of Energy Efficiency

#### In ITU-T L 1310 [L1310], energy efficiency will be defined as the relationship between the specific functional unit for a piece of equipment (i.e., the useful work of telecommunications) and the energy consumption of that equipment. For example, when transmission time and frequency bandwidth are fixed, a telecommunication system that can transport more data (in bits) with less energy (in Joules) is considered to be more energy efficient. For this reason, metrics that can evaluate the performance of a piece of equipment against its energy consumption are to be defined.

#### From Rel-15 onwards, the definition of Energy Efficiency is clarified in 3GPP. The definition does not come from 3GPP but from ETSI Technical Committee Environmental Engineering, in the ETSI ES 203 228 [ES203228]. It aims at defining the topology and level of analysis to assess the energy efficiency of mobile networks. In particular it defines metrics for mobile network energy efficiency and methods for assessing (and measuring) energy efficiency in operational networks.

#### As per ETSI ES 203 228 V1.3.1 [ES203228], Energy Efficiency (EE) is defined as the relation between the useful output and power consumption, where power consumption is defined as the power consumed by a device to achieve an intended application performance.

#### Mobile Network data Energy Efficiency EENE is the ratio between the performance indicator Data Volume (DVMN) and the Energy Consumption (ECMN) when assessed during the same time frame (T) as as defined in clause 7.1 of ITU-T recommendation L. 1331 [L1331]. This is also shown by the formula:

#### EENE= DVMNECMN

#### where DV is the Data Volume, expressed in bit, transported across a network element. The Data Volume measurements are collected via OAM. EC is the Energy Consumption, expressed in Joule, of the same network element. MN stands for Mobile Network. Clause 8.2 of ITU-T L.1331 [L.1331] illustrates how to measure/collect the information about data volume (for capacity), coverage area (for coverage) as well as energy consumption over a measurement period called T, that can span over one week, one month, or longer periods.

#### This formula is reminded in several 3GPP TSs and TRs dealing with energy efficiency (EE).

#### 7.2.2.2	UE Energy Efficiency

Similar to the definition of network EE KPI, one possible example to represent the UE energy efficiency (EE) is bit per Joule. A UE is considered to be more energy efficient when the data usage (in bits) in high with less energy consumption (in Joules), when battery capacity and consumption rate are fixed when assessed during the time frame (T). The metric is defined as below:

EEUE=Data Volumei=power stateEnergy Consumption (i)

- Data Volume [bit] represents the traffic volume for the UE for a given duration.

- Energy Consumption [estimated Joule] represents the total energy consumption of the UE for a given duration, which is the summation of power consumption.

The data volume consumed by k number of sub-applications is defined as the summation of the data volume consumed by the user equipment under investigation during the time frame T of the energy consumption assessment for k sub-applications. This could be shown as:

DVk,app=i=1kDV

where DVk,app, measured in bit, is the data volume over the measurement period T of the UE for a given duration.

The overall UE EE is then computed as follows:

EEUE=DVk,appi=power stateEnergy Consumption (i)

### 7.2.3	Procedures

The following methodology is proposed to measure energy efficiency of the UE:

1.	A test scenario is defined, and test conditions described:

a.	Network (connection type, upload and download bandwith, latency).

b.	User devices (type, model, SoC, OS version, video player). At least 2 devices should be used.

c.	Test environment (number of devices, number of iterations, etc.).

d. 	Anchor against which the specific features will be evaluated (i.e., 5GMS service delivering a 720p video at 2 Mbps in HEVC).

e.	Reference sequence(s) used.

2.	Launch the application under test which implements the reporting of energy-related information, including either battery discharge rate or else total energy discharged and duration.

3.	The test is done for the anchor and the implementation including the feature evaluated.

4.	Extract data from the data collector for non-real-time analysis. Characterization is documented in terms of expected energy savings, and may include additional comparison parameters such as impact on the end user’s Quality of Experience, etc.

5. The time duration of the measurement, denoted as T, shall be one of the alternatives:

- Weekly measurement: T equal to 7 days.

- Monthly measurement: T equal to 30 days.

- Yearly measurement: T equal to 365 days.

The minimum duration is therefore one week: monthly and yearly measurements are extensions of the basic week test as per ETSI ES 203 228 V1.3.1 [ES202336] guidelines.

6. The Energy Consumption of the MN can be measured by means of metering information provided by utility suppliers or by mobile network integrated measurement systems. Moreover, sensors can be used to measure site and equipment energy consumption, following the requirements set by ETSI ES 202 336-12 [ES202336].

7. The data volume shall be measured using network counters for data volume in the UE under test. For PS traffic, the data volume is considered as the overall amount of data transferred to and from the UE under test. Data volume shall be measured in an aggregated way per each application present in the UE. For CS traffic (e.g. CS voice or VoLTE), the data volume is considered as the number of minutes of communications during the time T multiplied by the data rate of the corresponding service and the call success rate as per ETSI ES 203 228 V1.3.1 [ES203228].

### 7.2.4	Impacts on existing services, entities and interfaces

7.2.4.1	Direct Data Collection Client

-	Collect and report per-application energy-related information as per the method described to the Data Collection AF.

7.2.4.2	Data Collection AF

-	Expose energy-related information reporting for a given UE as events.

## End of change