Agenda item: 	10.6

Source: 	Lenovo

Title: 	[FS_5G_RTP_Ph2] Views on way forward for KI#1

Document for	Discussion and Agreement

# Introduction

In TR 26.822 [1] KI#1, Inaccuracy of the PDU Set Size (PSSize) information, is an issue carried over from Rel-18 when it was deemed beneficial to further study its solution space.

# Background and Analysis Review

The potential causes for KI#1 are listed in TR 26.822 and can be summarized by on-path network layer effects in the path to N6 which may change the indicated PSSize value by an RTP sender. The PSSize is optionally included in the RTP header extension (HE) for PDU Set marking.

Across Rel-18 and current FS_5G_RTP_Ph2 study phase multiple solutions were proposed:

- Rel-18 solution based on indicating additionally the Number of PDUs in the PDU Set (NPDS) in the RTP HE for PDU Set marking and on signaling the origin IP address type to the UPF to allow for accurate PSSize correction mainly due to NAT46/64 operations in the N6 path. See for details agreed pCR [2] to TS 26.522. The solution was partially replaced by an Editor’s Note in TS 26.522 that it is yet to be resolved.

- Sol#4 of TR 26.822: Feedback-based correction of PSSize at the RTP sender using feedback indication of a correction factor originating from the RTP receiver (i.e., its peer). One caveat not well treated in Sol#4 is how the feedback should happen in practice. SDP-based feedback is mentioned as the signaling feedback channel for the correction factor indication. Yet, SDP is primarily used for RTC session establishment and media delivery happens only after the SDP procedure is completed. As such, this approach implies reissuing SDP offer-answer messages and reconfiguring the session to include the correction factor some time after media delivery has already started. It is advisable to use a better approach for the correction factor indication. A viable and good option is RTCP feedback, e.g., through RFC 4585 (i.e., RTCP-RTPFB, RTCP-PSFB) or RFC 3611 (i.e., RTCP-XR) which are well-suited for feedback extensions with minimal overheads. Further pros and cons are provided in TR 26.822 Clause 6.4, but the main advantage of this solution (assuming practical, RTCP-based, low-overhead feedback) is the lack of impact on network infrastructure, i.e., UPF.

- Sol#7 of TR 26.822: Replacement of PSSize semantics to remaining PSSize and differential processing of remaining PDU Set Size at UPF to determine inaccuracies due to network effects in N6 path. UPF complexity and changes to the PSSize semantics are major drawbacks in this case as provided in TR 26.822 Clause 6.7.

- UPF PDU Set Size overprovisioning [3]: This solution (not yet formally) discussed proposes the UPF modifying the PSSize indicated in the RTP HE PDU Set marking by a correction factor (e.g., 5%) over GTP-U. The main observation in [3] is that in practice PSSize may not always be byte-accurate and RAN should handle any inaccuracies anyways. Yet, handling underestimated PSSize values is harder from an optimized resource allocation perspective rather than an overestimated PSSize, where remaining resources could be early identified and returned to the pool of available radio resources. Despite the potential overhead, this takes a simple and potentially practical approach to UPF processing of PSSize.

However, SA4 did not explicitly cover and clarify following KI#1 objectives:

- Study what type of accuracy is needed from a RAN scheduling perspective, as the PDU Set Size is mainly used to optimize the scheduling at the RAN, take their feedback into account.

NOTE 1:	This point may require coordination with RAN2.

- Study and develop solutions to improve 5G RTP workflows using PDU Set Marking taking such feedback into account.

# Way forward and conclusions

The PSSize general design principle in Rel-18 was to aid the network with fast and accurate size estimates of incoming PDU Sets while enabling low complexity processing at the UPF.

Based on all of the above the following way forward is advisable:

- Maintain Rel-18 general principle and verify (by means of LS to: RAN2, cc: SA2) the SA4 assumptions regarding the RAN accuracy needs regarding the PDU Set size values.

- Consider towards normative work KI#1 candidate solutions from Rel-18 and Sol#4 from TR 26.822, conditionally based on RAN2 feedback as:

  - If exact PDU Set Size accuracy is required by RAN consider Rel-18 towards normative work effectively solving the NAT46/64.

  - If PDU Set Size accuracy may slightly vary at RAN consider Sol#4 from TR 26.822 towards normative work based on RTCP feedback mechanisms to indicate the correction factor

The solution on UPF PDU Set Size overprovisioning may be generally regarded as a UPF implementation aspect, as specified by TS 23.501, in determining the PDU Set size in the GTP-U header for PDU Set Information. Based on RAN2 feedback, UPF PDU Set Size overprovisioning informative guidelines may be considered upon SA2 request.

# 4	 Proposal

Discuss and agree on the way forward and conclusions as of Clause 3. If agreeable, capture conclusions in the corresponding TR 26.822 Clause 8, and LS to RAN2 on the matter of PDU Set Size accuracy requirements.

# References

[1] TR 26.822 v1.0.0, Study of 5G Real-time Transport Protocol Configurations, Phase 2.

[2] Tdoc S4-231758: pCR - [5G_RTP] Number of PDUs in PDU Set within RTP header extension, Source: Ericsson, TSG SA4#126, Chicago, Status: Agreed.

[3] Tdoc S4-241800: pCR – [FS_5G_RTP_Ph2] Solution KI#1 PDU Set Size overprovisioning, Source: Huawei, TSG SA4 #130, Status: Not Yet Treated. Draft available offline during FS_5G_RTP_Ph2 preparatory meeting: https://www.3gpp.org/ftp/TSG_SA/WG4_CODEC/3GPP_SA4_AHOC_MTGs/SA4_RTC/Inbox/Drafts/S4-241800v1.docx