3GPP TSG-SA4 Meeting #130	S4-242229

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024          (Revision of S4-242206)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.250 | CR | 0002 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IVAS codec levels |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Dolby Sweden AB, Fraunhofer IIS, Nokia, Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IVAS_Codec |  |  |  |  |  | Date: |  |  | 2024-11-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current IVAS codec specification lacks a definition of IVAS codec levels. |  |  |  |  |  |  |  |  |
| Summary of change: |  | IVAS codec levels are defined and requirements for conformant encoder and decoder implementations of these levels are given. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Implementers might decide to implement only a subset of features to meet their hardware constraints thereby impacting interoperability. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE 1

# 4	General

The codec for Immersive Voice and Audio Services is part of a framework comprising besides encoder and decoder, renderer and a number of auxiliary functions associated with the support of stereo and immersive audio formats.

The IVAS codec is an extension of the 3GPP Enhanced Voice Services (EVS) codec; it provides full and bit exact EVS codec functionality for mono speech/audio signal input.

On top of that the IVAS codec is optimized for encoding and decoding of stereo and immersive audio formats, using tools such as Single Channel Element (SCE) coding, Channel Pair Element (CPE) coding and multi-channel coding by means of the Multi-channel Coding Tool (MCT). The stereo modes comprise a hybrid time-domain/DFT-domain/MDCT-domain coding scheme including inter channel alignment (ICA). Immersive audio formats comprise multi-channel audio (5.1, 5.1.2, 5.1.4, 7.1, 7.1.4 setups), scene-based audio (Ambisonics up to order 3), metadata-assisted spatial audio (MASA), and object-based audio (Independent Stream with Metadata (ISM) up to 4 ISMs). In addition, the following combined immersive audio formats are supported: object-based audio with scene-based audio (OSBA, up to 4 ISMs with Ambisonics) and object-based audio with metadata-assisted spatial audio (OMASA, up to 4 ISMs with MASA).

The codec features VAD/DTX/CNG for rate efficient stereo and immersive conversational voice transmissions, an error concealment mechanism to combat the effects of transmission errors and lost packets. Jitter buffer management is also provided.

The IVAS codec operates on 20 ms audio frames. It is capable of switching its bit rate upon command instantly at (active) frame boundaries.

A reference configuration where relevant interface signals and various relevant send side processing functions are identified is given in Figure 1. A corresponding reference configuration for receive side identifying relevant interface signals and processing functions is given in Figure 2. In the figures, the relevant specifications for each function are also indicated.

In Figures 1 & 2, the UE Send and Receive Audio processing are included, to show the complete path between the audio input/output in the User Equipment (UE) and a possible digital interface in network (all excluding A/D or D/A conversion). The detailed specification of the audio parts is not within the scope of the present document. These aspects are only considered to the extent to highlight that the function of the audio parts and the operation of the IVAS codec are closely dependent on each other.

IVAS codec complexity and memory requirements generally scale with bit rate and the number of input / output audio channels. To ease IVAS deployments on a large range of UEs with different capabilities while maximising interoperability, three IVAS codec functionality levels are defined with increasing bit rate and commensurately increasing complexity/memory requirements.

The following levels are defined:

- Level 1: all bit rates up to 80 kbps

- Level 2: all bit rates up to 192 kbps

- Level 3: all bit rates up to 512 kbps

Requirements

- A compliant IVAS encoder of a given level shall support all IVAS coding modes/coded formats at all bit rates of that level.

- A compliant IVAS decoder of a given level shall support decoding of all IVAS coding modes/coded formats at all bit rates of that level to an immersive output format and shall support:

  - Integrated rendering supporting all the output formats as specified in Table 4.4-1 of [3], and

  - Pass-through operation as external (EXT) processing output (as described in clause 4.3 of [3]) for further consumption by an external renderer

- A compliant IVAS decoder of level 3 shall support integrated rendering to all IVAS output formats as specified in clause 4.3.2 of [3].

- A compliant IVAS decoder of level 1 or 2 shall support decoding of an IVAS bitstream corresponding to a higher level to at least mono and stereo.

NOTE 1: Immersive output format implies any output format other than mono.

NOTE 2: While the definition of IVAS levels ensures support for immersive output formats at the decoder output, specific device requirements to ensure immersive audio presentation are present in other specifications such as [12].

NOTE 3: Compliant IVAS encoder means that an encoder implementation meets the encoder compliance criteria defined in clause 8 and [7]. Compliant IVAS decoder means that a decoder implementation meets the decoder compliance criteria defined in clause 8 and [7] for the supported output formats.

Recommendations

For the immersive coding formats other than stereo, the supported output formats of an IVAS decoder should include at least one immersive output format other than stereo.

Figure 1: Overview of audio processing functions - Transmit Side

Figure 2: Overview of audio processing functions - Receive Side

Interfaces:

1: Audio input channels (16-bit linear PCM, sampled at 8 (only EVS), 16, 32, or 48 kHz)

2: Metadata associated with input audio

3: Encoded audio frames (50 frames/s), number of bits depending on IVAS codec mode

4: Encoded Silence Insertion Descriptor (SID) frames

5: RTP Payload packets

6: Lost Frame Indicator (BFI)

7: Renderer config data

8: Head-tracker pose information and scene orientation control data

9: Audio output channels (16-bit linear PCM, sampled at 8 (only EVS), 16, 32, or 48 kHz)

10: Metadata associated with output audio

END OF CHANGES