3GPP TSG-SA4 Meeting #130	S4-242244

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024					revision S4-241887

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.802 | CR | 0004 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AMD] Updated Conclusions for TR 26.802 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD |  |  |  |  |  | Date: |  |  | 2024-11-22 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Conclusions are missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds skeleton for conclusions |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No instructions for next steps |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TR 26.802 CR 0001, 0002, 0003 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR may aggregate all CRs from this meeting that are sent to SA plenary |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## =====  CHANGE =====

## 8.4	Recommended follow-up work from this version

In a second phase of this feasibility study, additional Key Issues have been documented.

Based on the study of these, the following next steps are recommended:

1.	Provide relevant extensions to TS 26.502 [26502] to extend the MBS User Service architecture based on the updated conclusions in clause 5. Candidates for these extensions are:

-	for Key Issue #8: In-session unicast repair for MBS Object Distribution as introduced in clause 5.9 based on the conclusions in clause 5.9.7 to address Gap#1 in clause 5.9.5 by the candidate solution in clause 5.9.6:

a.	Defining a new reference point in TS 26.502 [26502].

b.	Documenting call flows and procedures for both post-session and in-session unicast repair.

-	for Key Issue #9: MBS User Service and Delivery Protocols for eMBMS as introduced in clause 5.10 based on the conclusions in clause 5.10.6:

-	Fully specify support for the Joint BM-SC and MBSF Functionality. For this purpose, the gap identified in clause 5.10.4.1 of the present document needs to be addressed by documenting additional procedures and baseline parameters as required in TS 26.502 [26502] and permitting the signalling of MBMS sessions.

-	Document in an informative annex to TS 26.502 [26502] the deployment architectures, client architectures and high-level call flows in clauses 5.10.2.3 and 5.10.2.4.

2.	Provide relevant extensions to MBS User service protocols and formats specified in TS 26.517 [26517] based on the conclusions in clause 5 and the stage-2 extensions above, if applicable. Candidates for these extensions are:

-	for Key Issue #8: In-session unicast repair for MBS Object Distribution as introduced in clause 5.9 based on the conclusions in clause 5.9.7 to address Gap#2, #3, #4, and #5 in clause 5.9.5 by the candidate solution in clause 5.9.6:

-	On gap #2 identified in clause 5.9.5, both of the following signalling options are expected to be supported:

-	Using FDT parameters to signal the time when repairs can be requested using the Expires attribute).

-	Using LCT header information to signal the time when repairs can be requested using the B-Flag.

-	On gap #3 identified in clause 5.9.5, the following signalling options exist in the FLUTE File Delivery Table (FDT):

-	Defining a new FDT extensions parameter to signal the availability time when the object needs to be released.

-	On gap #4 identified in clause 5.9.5, the execution of MBS object delivery and in-session unicast repair can run in parallel in the MBS Client. However, this should be validated if there are cases this is not the case and whether these cases need to be explicitly stated, for example RedCAP UEs.

-	On gap #5 identified in clause 5.9.5, time synchronization can reuse functionalities defined in TS 26.346 [16], but tighter synchronization that 1 second. This work is aligned with the findings and work in clause 5.11.3.6.

-	for Key Issue #9: MBS User Service and Delivery Protocols for eMBMS as introduced in clause 5.10 based on the conclusions in clause 5.10.6:

-	address the relevant stage-3 aspects based on stage-2 work.

3.	Continue the study of additional extensions to MBS User Services. Candidate topics based on the present document are:

-	for Key Issue #9: MBS User Service and Delivery Protocols for eMBMS as introduced in clause 5.10 based on the conclusions in clause 5.10.6:

-	Validate the approaches by implementation, for example in 5G-MAG Reference Tools, and identify if the functionality is fully supported or any further specification updates are needed.

-	Going forward, ensure that enhancements to the MBSTF and delivery methods in MBS can also be leveraged and deployed for eMBMS.

4.	Coordinate work with other working groups and organizations:

-	for Key Issue #9: MBS User Service and Delivery Protocols for eMBMS as introduced in clause 5.10 based on the conclusions in clause 5.10.6:

-	Validate the approaches by implementation, for example in 5G-MAG Reference Tools.

-	validate whether the TMGI allocation in the MBS US Announcement is achievable via MB-SMF with other working groups.

Editor”s Note: Implementation of 26802-0003rev6 still to be done