3GPP TSG-SA WG4 Meeting #130	S4-241834

USA, Orlando, 18 – 22 November 2024									  Revision of S4aR240092

Source:	Nokia, Qualcomm

Title:	[FS_5G_RTP_Ph2] Proposed conclusions for TR 26.822

Agenda item:	4.5

Document for:	Agreement

1. Introduction

SA4 is planning to conclude FS_5G_RTP_Ph2 in the upcoming Orlando meeting in November. Therefore, SA4 needs to start working on conclusions and recommendations for normative work.

This document is a draft and work-in-progress. It is proposed as a basis for conclusions for all key issues in the TR. The intention is to document the conclusions for the KIs that are stable and for which no further input is expected, and document the interim conclusions or insert placeholders for the other KIs and complete when the solution discussions are completed.

We note that some KIs were not progressed, and for some of the other KIs no potential normative work has been identified.

Update rev1:

Discussion from the RTC SWG telco on October 23

| S4aR240092 | [FS_5G_RTP_Ph2] Proposed conclusions for TR 26.822 | Nokia | FS_5G_RTP_Ph2 (Study of 5G Real-time Transport Protocol Configurations, Phase 2) |
| --- | --- | --- | --- |

Presenter: Serhan

Comments:

- Liangping: KI#8 RTX is E2E, no need to SA2/RAN involvement, KI#12 conclusions are premature, same for KI#14.

- Rufael: Suggestion is to have a table rather than text to collect all conclusions.

- Serhan: The format is not very important, but critical to identify what to do in normative work

- Rufael: Reason is to have a systematic approach like other SIs.

- Serhan: Conclusions intended on normative work, but we can discuss further on the format

- Serhan: Okay to note - but I encourage colleagues to contribute to this for Nov. meeting.

Decision: Noted

Update offline on October 30:

Due to time constraints, the group could only discuss the conclusions of the first three key issues. This document is provided as an input contribution to provide the basis for further progressing the conclusions during the SA4#130 meeting.

2. Reason for Change

Add conclusions for key issues in TR 26.822 and recommendations for normative work.

3. Proposal

It is proposed to take the interim conclusions below as basis and keep progressing them to complete the study item in time in SA4#130 Orlando meeting.

* * * First Change (all new text) * * * *

# 8	Conclusions

## 8.1	Conclusions for Key Issue #1

Inaccuracy of the PDU Set Size (PSSize) information

[Normative work for Solution #4 (measurement-based pre-compensation) is recommended.]

## 8.2	Conclusions for Key Issue #2

QoS handling requirements for lone PDU

The following aspects are concluded as principles for normative work:

-	Extend the RTC provisioning feature in TS 26.510 and TS 26.113 to include PDU Set Importance values for PDUs of protocols that may be treated as lone PDUs in the UPF.

-	Coordinate with SA2 on whether Protocol Description needs to be extended with the lone PDU information.

-	Consider guidelines for handling lone PDU in TS 26.522

Editor’s Note: Some new solutions are still being proposed for this KI. Guidelines for PSSN handling in the UPF may be considered based on these.

## 8.3	Conclusions for Key Issue #3

Enhancements for application-layer FEC support

Existing AL-FEC and congestion control schemes were studied in this KI.

No recommendation for normative work to include new AL-FEC schemes in the 3GPP specifications was identified.

## 8.4	Conclusions for Key Issue #4

AL-FEC awareness for PDU Set handling

The following topics are recommended for normative work:

- a generic mechanism to improve congestion control algorithms for AL-FEC encoded traffic considering intentional packet discarding by the network,

- packet loss rate indication for the RAN or the UE to determine how much redundancy needs to be delivered to meet the upstream and/or downstream packet losses.

- Develop Guidelines for carriage of AL-FEC when using RTP HE in TS 26.522

## 8.5	Conclusions for Key Issue #5

RTP transport of XR metadata

This key issue was not progressed, hence no recommendation for normative work.

## 8.6	Conclusions for Key Issue #6

PDU Set marking for XR streams with RTP end-to-end encryption

- Extend the guidelines for support SRTP in such cases in TS 26.522

No additional normative work has been identified for Rel-19 since SRTP usage is already supported in Rel-18.

## 8.7	Conclusions for Key Issue #7

RTCP messages to better support XR services in 5G

Existing RTCP messages and RTP HEs were studied in this KI.

No potential normative work has been identified for Rel-19.

## 8.8	Conclusions for Key Issue #8

RTP retransmission in supporting XR services in 5G

The following aspects are concluded as principles for normative work:

-	[Consider extending TS 26.522 to define a compact/modified RTP HE for retransmitted PDUs to optimize the header size while preserving essential information for network decisions.

NOTE: it is not clear about the benefit of such optimization given that retransmission should happen with a small probability and the header size reduction is negligible relative to a typical whole PDU size.]

-	Consider sending information from the application to the 5G Core Network to make it aware of end-to-end retransmissions.

-	Coordinate with SA2 and RAN2 on network awareness of retransmitted PDUs as well as core network and RAN handling of retransmitted PDUs based on the information provided by the application.

## 8.9	Conclusions for Key Issue #9

Feasibility of RTP multiplexing options for transport of XR media streams

TBD

Editor’s Note: Some new solutions are still being proposed for this KI.

## 8.10	Conclusions for Key Issue #10

Use cases and intended deployment scenarios for enhancements of RTP header extension for PDU Set marking

This key issue was not progressed, hence no recommendation for normative work.

## 8.11	Conclusions for Key Issue #11

Enhancements of RTP header extension for PDU Set marking

This key issue was not progressed, hence no recommendation for normative work.

## 8.12	Conclusions for Key Issue #12

Enhancements of Data Burst Marking

The following aspects are concluded as principles for normative work:

-	Extend the PDU Set information sent to the network to include time to next burst (TTNB)

-	Extend the PDU Set information sent to the network to include burst size

Editor’s Note: Whether a new HE needs to be defined for the above extensions or the existing RTP HE for PDU Set making is extended is FFS.

NOTE: the feasibility of predicting the TTNB and the burst size in relation to potentially added delay needs to be demonstrated for conversational video and XR traffic before normative work starts.

NOTE: Other potential solutions will not be excluded in the normative phase if their merit can be justified.

## 8.13	Conclusions for Key Issue #13

Applicability of the RTP header extension for PDU Set marking to different PDU Set types

The following aspects are concluded as principles for normative work:

-	Consider extending the PSI guidelines in TS 26.522 for the case when a PDU Set is defined as a tile (as opposed to a video frame or slice).

-	Coordinate with SA2 and RAN2 on potential benefits of signaling PDU Set type to the 5G network.

## 8.14	Conclusions for Key Issue #14

Traffic detection and QoS flow mapping for multiplexed media stream data flows

The following aspects are concluded as principles for normative work:

-	Coordinate with SA2 on potential usage of MID for packet filtering in the UPF.

## 8.15	Conclusions for Key Issue #15

Media and metadata delivery over multiple sessions

TBD