3GPP TSG-SA WG4 Meeting #130		S4-241861

Orlando, USA, 18th – 22nd Nov. 2024

Source:	vivo

Title:	Discussion on LS from RAN2 on XR multi-modality

Agenda Item:	10.3

Document for:	Discussion and Decision

- Introduction

In SA4#130 meeting, a replied LS from RAN2 is received on multi-modality awareness at RAN [1]:

For the usage of multi-modality information, RAN2 has discussed and reached the following agreements during the previous RAN2 meetings.

First, RAN2 agreed that the multi-modality information (e.g., MMSID), can be used for some RAN handling, e.g. QoS flow to DRB mapping as below.

RAN2 considers that based on multi-modal information: - The gNB may perform joint admission control. Details can be left up to RAN3 in potential WI phase. - The gNB may consider this information during QoS flow to DRB mapping (up to gNB implementation)

Furthermore, RAN2 also agreed that traffic synchronization may be achieved by packet level synchronization information, if it can be provided from CN to RAN, which is up to SA2/4 to discuss.

For DL, whether traffic synchronization (on a per packet basis) can be achieved depends on whether packet level synchronization information can be provided from CN to RAN. RAN2 thinks PDU Set discard across QoS flows of the same multi-modal service based on the dependency information between the multi-modal flows can only be achieved in case the synchronization information can be available at the UE which is up to SA2/SA4.

In this contribution, we will discuss the synchronization information of multi-modality services related issue and provide the draft response from SA4 point of view.

- Discussion

  - Previous discussions on Multi-Modal service information awareness in RAN

In previous meetings, SA2 has indicated that policy control enhancements to support multi-modal services has been specified in Rel-18, with the use of Multi-modal Service ID (MMSID) in AF and PCF and forwarding MMSID to RAN via control plane is in principle technically feasible to SA2 but they also want to further understand the usage of such information [2]. Thus, RAN2 clarified the agreements in the previous RAN2 meeting the MMSID could be used for some RAN operations, e.g. QoS flow to DRB mapping. Furthermore, RAN2 also discussed and agreed SDU discard across QoS flows would be also performed based on traffic synchronization information which up to SA2/SA4 discussion [1].

  - Scenario and benefit of Multi-Modal service information awareness in RAN

From previous RAN discussion, RAN could handle the QoS flow belonging to one Multi-Modal service as a integrity based on information that which QoS flows belong to one Multi-Modal service. In detail, once RAN gets QoS flow and Multi-modal Service mapping relation, it can accept or deny all of the QoS flows established to avoid partial QoS flow setup in RAN in case of RAN congestion, which may degrade user experience. Once several QoS flows for on Multi-modal service have been accepted by RAN, it could also use the flow correlation to perform QoS flow to RB mapping. For example, gNB could map different QoS flows into one DRB to guarantee the coordination transmission in RAN.

The synchronization information defines certain synchronization threshold as per QoS flow or per packets basis across different QoS flows. Once certain flow or some packets exceeds the defined synchronization threshold, the network or UE may consider such data are outdated that could be discarded to save radio resources and avoid introducing delay to the following data transmission. It is typically useful in case of radio congestion. Since in the congestion scenario, gNB anyway needs to discard PDUs stored in the buffer, if synchronization information can be known to gNB, it could choose more proper packet to discard. It is similar as PSI based SDU discard defined in Rel-18 that low importance data could be discarded.

  - Discussion on PDU Set Discard based on synchronization information

From the above LS from RAN2, we analyzed there are two use cases for PDU Set discard across QoS flows of the same multi-modal service based on the correlation information and synchronization information between the multi-modal flows. The first one is packet discarding due to one of the correlated PDU set in another QoS flow been discarded. It is similar as Rel-18 PDU set based SDU discard, which implies the loss of one packet of all packets within the PDU set may result in discarding the whole PDU set. According to TS22.261, in case of multi-modality, for example, the sensor information would be correlated to certain video/media fragment. Once the media fragmentation packets have been discarded, it would be straightforward that the sensor information is also useless to the application layer in the receiver side. The second case is discarding packets due to transmission delay or network congestion since synchronization threshold between different media components is considered more critical to user experience for Multi-modal services according to TS22.261. For both of the use cases, UE may need to obtain the correlation and synchronization information either from CN PCF/AF or from UE application layer.

Through analysis of open-source XR streaming solutions, we found that correlation information between different media components (such as tactile information and video stream) can be obtained through stream identifiers (including stream IDs and corresponding frame indices). Besides, packet level synchronization information can be obtained via tracking frame index which is included in the video frame header to associate with a specific motion control haptic frame. Furthermore, most of the flows have their sent timestamp and some of the delay-critical flows may also have target timestamp, so that synchronization threshold of some pair of flows may be derived from above parameters.

Observation: some of the application codec schemes may allow UE to obtain correlation information and synchronization information of different flows within certain service.

Furthermore, in Rel-18, SA4 has analyzed dropping packets in transport network if congestion happened. As a result, it is suggested to follow the same principle as Rel-18 PSI based packet dropping which is specified in 26.522 clause 4.2.6.2. This implies PDU set with higher PSI value could be considered to be discarded in case of network congestion also considering the correlation and synchronization information between different flows.

In summary, UE application layer could obtain correlation information and synchronization threshold based on some media codec scheme, which implies UE AS could further get such information via cross layer implementation. For packet discarding, it is suggested to reuse Rel-18 PSI based packet discard concept after obtain above correlation information and synchronization threshold.

Proposal: SA4 to conclude UE could obtain correlation information and synchronization threshold of PDU sets cross QoS flows from its APP layer by UE implementation. Packet discarding could follow PSI rules defined in Rel-18.

- Conclusion

In this contribution, we discuss the issues in the LS from RAN2 on multi-modality [1]. Based on the discussion, we have the following observation and proposal and a draft reply LS has been attached as the Annex:

Observation: some of the application codec schemes may allow UE to obtain correlation information and synchronization information of different flows within certain service.

Proposal: SA4 to conclude UE could obtain correlation information and synchronization threshold of PDU sets cross QoS flows from its APP layer by UE implementation. Packet discarding could follow PSI rules defined in Rel-18.

- References

- S4-241802/R2-2409272, Reply to SA2 LS on multi-modality awareness, RAN2

- S2-2409444, LS reply on multi-modality awareness at RAN, SA2#164

- Annex

Title:	[Draft] reply LS on multi-modality services

Response to:	S4-241802/R2-2409272

Release:	Rel-19

Work Item:	NR_XR_Ph3-Core

Source:	TBD [to be SA4]

To:	RAN2

Cc: 	SA2, RAN3

# Contact Person:

# Name:	              TBD

E-mail Address:	 TBD

# 1. Overall Description:

SA4 discussed the synchronization information obtaining at the UE and PDU Set discard across QoS flows of the same multi-modal service based on the correlation information and synchronization threshold between the multi-modal flows and drew the conclusion from SA4 point of view in below:

UE could obtain correlation information and synchronization threshold of PDU sets cross QoS flows within one Multi-Modal service from its APP layer by UE implementation. Packet discarding could follow PSI rules defined in Rel-18.

# 2. Actions:

To RAN WG2:

SA4 kindly asks RAN2 to take the above information into account and provide answers to the questions above.

# 3. Date of Next SA4 Meetings:

TSG SA WG4 Meeting #131		17th Feb. – 21st Feb. 2025		Geneva, Swiss

TSG SA WG4 Meeting #131bis		7th April. – 11th April. 2025		Online