3GPP TSG-SA WG4 Meeting #130	S4-241871

# USA, Orlando, 18 – 22 November 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.956 | CR | pseudo | rev |  | Current version: | 0.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Beyond2D] Representation Format - Light Fields Video |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile Com. Corporation, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Beyond2D |  |  |  |  |  | Date: |  |  | 2024-10-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The study item description in SP-240479 addresses the following objectives - Identify and document beyond 2D formats, that are market-relevant within the next years, generated from established and emerging capturing systems (including cameras for spatial video capturing), contribution, and usable on display technologies (smartphones, VR HMDs, AR glasses, autostereoscopic and multiscopic displays). During SA4#128, several scenarios were defined, that are considered to address the distribution scenarios and evaluation frameworks. However, some of the scenarios already assume a specific Representation Format that seems to be of less relevance initially. The evaluation framework is important, once Representation formats are defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document focuses on light fields video. It's a starting point. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE ===== (add to References)

[LF-1]	Gershun A (1939) The light field. Moscow, 1936. Translated by Moon P, Timoshenko G in J Math Phys XVIII:51–151

[LF-2]	Marc Levoy and Pat Hanrahan. 1996. Light field rendering. In Proceedings of the 23rd annual conference on Computer graphics and interactive techniques (SIGGRAPH '96).

[LF-3]	Google Light Stage X4, https://www.fxguide.com/fxfeatured/exclusive-paul-debevec-and-the-light-stage-research-at-google/

[LF-4]	USC Lightstage X6, https://vgl.ict.usc.edu/Data/LightStage/

[LF-5]	Zhou, Taotao, et al. "Relightable neural human assets from multi-view gradient illuminations." Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition. 2023.

[LF-6]	Michael Broxton, John Flynn, Ryan Overbeck, Daniel Erickson, Peter Hedman, Matthew Duvall, Jason Dourgarian, Jay Busch, Matt Whalen, and Paul Debevec. 2020. Immersive light field video with a layered mesh representation. ACM Trans. Graph. 39, 4, Article 86 (August 2020), 15 pages. https://doi.org/10.1145/3386569.3392485

[LF-7]	M. B. de Carvalho et al., "A 4D DCT-Based Lenslet Light Field Codec," 2018 25th IEEE International Conference on Image Processing (ICIP), Athens, Greece, 2018, pp. 435-439, doi: 10.1109/ICIP.2018.8451684.

[LF-8]	ISO/IEC JTC 1/SC 29/WG 04, w24268, “Overview of lenslet video coding activities”, MPEG 147, Sapporo.

[LF-9]	Daniel, Jamison R. et al. “Initial work on development of an open Streaming Media Standard for Field of Light Displays (SMFoLD).” SD&A (2018).

[LF-10]	X. Min, J. Zhou, G. Zhai, P. Le Callet, X. Yang and X. Guan, "A Metric for Light Field Reconstruction, Compression, and Display Quality Evaluation," in IEEE Transactions on Image Processing, vol. 29, pp. 3790-3804, 2020, doi: 10.1109/TIP.2020.2966081.

[LF-11]	ISO/IEC JTC 1/SC29/WG1 N100306, REQ "Use Cases and Requirements for Light Field Quality Assessment v5.0", 97th JPEG Meeting, Online, October 2022.

[LF-12]	What you should know about light-field, https://www.digitalmedia.fraunhofer.de/en/mediainformation/trendbrochures/trendbrochure-2021/what-you-should-know-about-light-field-.html

[LF-13]	"A Visual Introduction to the Past, Present, and Future of Light Field Technology", https://cubicleninjas.com/light-field-technology/

[LF-14]	Ruben Verhack, "AI-Driven Breakthroughs in Image-Based Rendering: Light Fields, SMoE, Gaussian Splatting, NeRFs and beyond", https://cubicleninjas.com/light-field-technology/, Tech Posts Computer Graphics, https://blog.datameister.ai/ai-driven-breakthroughs-image-based-rendering, February 2024.

=====  CHANGE =====

4.3.X	Light Fields Video

4.3.X.1	Definition

A light field, or lightfield, is a vector function that describes the amount of light flowing in every direction through every point in a space. The term light field was first coined in 1936 by Andrey Gershun [LF-1], for describing the radiometric properties of light in three-dimensional space. Scientifically, light rays are described by the 5-dimensional plenoptic function, which each ray is defined by three coordinates in 3D space (3 dimensions) and two angles to specify their direction in 3D space (2 dimensions).

Light fields were introduced into computer graphics in 1996 for image-based rendering applications, i.e. to compute new views of a scene from pre-existing views without the need for scene geometry [LF-2].

Note that also the term "radiance field" may be used to refer to similar, or identical concepts such as light fields. In an extension, neural radiance fields (NeRFs) a a subject of latest research, for details on NeRFs refer to clause 4.3.Y.1.

Light field representations can be divided into two types:

-	the plenoptic function: as shown in Figure 4.3.X.1-1(a), the plenoptic function represents the light field in a 7-dimensional function, which includes the position of viewer’s eye (x, y, z), direction (Θ, Φ in polar coordinates), wavelength (λ), and time (t). However, in practical applications, the 4D light field function L (u, v, s, t) based on two planes, as shown in Figure 4,3,X,1-1(b), is more commonly used. In 4D light field function, (u, v) are the coordinates of the first plane, representing the angular information of the light field, while (s, t) are the coordinates of the second plane, representing the spatial information of the light field.

Figure 4,3,X,1-1(a), 7D plenoptic function

Figure 4,3,X,1-1(b), 4D plenoptic function

-	the reflectance field: as shown in Figure 4,3,X.2, the reflectance field corresponds to the plenoptic function but focuses on the emission surface. It describes the transport of light between the incident light on an object and the light exiting from it.

Figure 4,3,X,2, reflectance field

A light field can capture multiple-dimensional data, including spatial, angular, spectral, and temporal domains. Compared to traditional 2D video, the light field provides more 3D information, offering viewers an immersive visual experience. It also enables intelligent system to better understand the 3D environment or objects.Therefore, light field-related technologies are gradually being applied in fields such as industrial inspection, embodied intelligence, virtual/augmented reality, glasses-free 3D displays and so on, demonstrating vast application potential. For example, in the field of industrial inspection, data captured by light field cameras can be used to compute and render multi-view images, synthesize focus stacks, flexibly control depth of field, and achieve full volumetric reconstruction, thereby improving the accuracy of industrial inspection and quality control. Light field video can also be used in high-precision 3D reconstruction of people, objects, and environments, reducing the cost of digital content production, which can be applied in the metaverse and digital twin cities.

Table 4.3.X.1-1 provides an overview of existing lightfield technologies and their properties.

Table 4.3.X.1-1 Existing lightfield technologies and their properties

| Properties | Google Lightstage X4 [LF-3] | USC Lightstage X6 [LF-4] | PlenOptic Stage [LF-5] |
| --- | --- | --- | --- |
| Type | Color gradient, polarized light, directional light, OLAT | White gradient, OLAT | Gradient light, polarized light, directional light, color gradient, OLAT |
| Number of Lights | 331 | 1111 | 162~460 |
| Number of Cameras | 90~100+ | 3 | 6~32 |
| Acquisition speed | 60fps | 990fps | 5fps |
| Resolution | 4K | 2K | 6K/16K |
| Output frame rate | 30fps | 30fps | 5fps |

Editor’s Note: other resolution may include 1536 * 2048, 3840 * 2160, 7680*4320 (45-100 views)

4.3.X.2 	Production and Capturing Systems

Light fields are typically produced either by rendering a 3D model or by photographing a real scene. In either case, to produce a light field, views must be obtained for a large collection of viewpoints. Depending on the parameterization, this collection typically spans some portion of a line, circle, plane, sphere, or other shape, although unstructured collections are possible. Devices for capturing light fields photographically may include a moving handheld camera or a robotically controlled camera, an arc of cameras (as in the bullet time effect used in The Matrix), a dense array of

Figure 4,3,X,2-1, Sequential Imaging

-	Camera Array or camera modules: To improve the efficiency of light field acquisition, multiple cameras can be combined into a camera array, capturing the scene from different angles and positions to obtain light field video. Camera array-based light field video acquisition systems are highly time-efficient and can capture dynamic objects. However, due to the typically large size of the camera array and the high hardware costs, along with the challenges of synchronization and data transmission between multiple cameras, its practical application has certain limitations.

Figure 4,3,X,2-2(a), Camera Array: China Mobile’s "Metaverse Light Field Capture Center"

Figure 4,3,X,2-2(b), An embodyment of the Stanford Multi-Camera Array, using 96 cameras with telephoto lenses

Figure 4,3,X,2-2(c), Light Field Smartphone: Pelican Imaging

-	Plenoptic Camera: To balance system cost and acquisition time, some researchers have proposed using a microlens array combined with a single camera for light field acquisition. A plenoptic camera places the microlens array between the main lens and the image sensor, offering the advantages of portability and low cost. This device can efficiently capture dynamic scenes, therefore they are increasingly being used in light field acquisition. PlenoptiCam is an open-source, cross-platform software tool that processes raw plenoptic camera images into light field data through 4D image alignment and calibration: https://github.com/hahnec/plenopticam.

Figure 4,3,X,2-3, Plenoptic Camera

4.3.X.3 	Rendering and Display Systems

Light field video can be viewed on different rendering and display systems, including:

-	AR glasses: CREALTM light-field AR glasses, https://creal.com/ar/

Figure 4,3,X,3-1,CREALTM light-field AR glasses

-	VR HMD: Meta Quest, CREALTM light-field VR HMD, https://creal.com/vr/

-	Light Field Display: Looking Glass (https://www.holoxica.com/looking-glass-4k), Leia

Figure 4,3,X,3-2, Looking Glass

For rendering light-field video, it takes image rendering toward a “no-geometry-required” solution but use multiple image views. For example, there is an open-source interactive light field renderer using dynamically reparameterized light fields in GitHub:

https://github.com/linusmossberg/light-field-renderer.

4.3.X.4 	Supporting Information

The following is the end-to-end Light Field Video System proposed by Google [LF-6], including capturing, reconstructing, compressing, and rendering high quality immersive light field video. The immersive light fields are recorded using a custom array of 46 time-synchronized cameras distributed on the surface of a hemispherical, 92cm diameter dome. From this data it produces 6DOF volumetric videos with a wide 80-cm viewing baseline, 10 pixels per degree angular resolution, and a wide field of view (>220 degrees), at 30fps video frame rates. The resulting RGB, alpha, and depth channels in these layers are then compressed using conventional texture atlasing and video compression techniques. The final compressed representation is lightweight and can be rendered on mobile VR/AR platforms or in a web browser.

Figure 4,3,X,4, End-to-end Light Field Video System

Multiple compression techniques have been proposed for the light field video, they are mainly categorized into three groups:

-	Transform-based methods: these approaches typically involving using a transformer, such as the discrete cosine transform (DCT) or the discrete wavelet transform (DWT). For example, the MuLE [LF-7], which has been adopted by the JPEG pleno standardization committee.

-	Prediction-based methods: some studies interpret light fields as multi-view sequences and use multi-view extension such as MV-HEVC for compression.

-	Learning-based methods: these are recent development in light field compression. They leverage the power of machine learning techniques (e.g., GAN or CNN) to improve encoding efficiency,

MPEG is currently working on Lenslet Video Coding (LVC), which focuses on dense light field representations, use cases and requirements, and dedicated codecs .[LF-8].

The content can be delivered with a DASH-compliant framework, an open streaming media standard for field of light displays (SMFoLD) [LF-9] had also been developed for streaming dynamic scene. However, the work appears to have been discontinued, as there have been no updates since 2018.

Typical quality criteria for evaluating light field video includes:

-	3D IQA, PSNR and SSIM Quality Metrics

- 	FR light field metric [LF-10], which considers global spatial quality metrics based on viewing structure matching, local spatial quality based on near-edge MSE, and angular quality based on multiview quality analysis.

- 	JPEG is also preparing standardization activities [LF-11] in the domains of objective and subjective quality assessment for light fields, improved light field coding modes, and learning-based light field coding.

4.3.X.5 	Benefits and Limitations

4.3.X.5.1	Benefits

The light field videos offer the following main benefit:

-	Immersive visual experience, “cover all the different perspectives so the user can choose which one he wants to have and of course different users can choose different perspectives.” [LF-12]

Some advantages and benefits of lightfields are provided in [LF-13]:

-	the light field content production approach has the benefit that it is a full representation of what is captured, is high resolution, one can observe in stereo and can move within this content, to a predefined degree.

-	Content can be captured by a micro lens array or produced by computer graphics. Light fields allow the high fidelity of models, textures, lighting, and reflections.

-	Lightfield captures are holographic, i.e. they contain all possible views within a preset range that can be defined.

-	Light field captures have parallax, an overlap of object depth based on head movement. This increases immersion and presence, which is not available in 3DOF omnidirectional videos.

-	Captures can have higher quality reflections, complex lighting, or realistic physics which would not be possible in real-time 3D graphics.

-	Light fields focus or aperture can be adjusted on the fly based on depth versus a 2D video which needs to be in focus at the time of filming. This is because light fields can estimate depth based on the data acquired.

-	Just like 3D models, multiple light field captures can be incorporated together and can be manipulated.

-	Most of the captured data is not needed and can be removed. For example, thousands of high quality photos of small movements can be compressed to be a fraction of this size by removing this redundant visual info.

4.3.X.5.2	Limitations

The large number of views and captures in light field videos generate massive amounts of data, leading to significant storage and bandwidth demands. Even when content is delivered promptly, current handheld devices struggle to load these resources in real-time.

Some disadvantages and limitations of lightfields are provided in [LF-13]:

-	Lightfields require a massive amount of high quality pixel based information, regardless whether it is from a real camera or from a virtual camera. In addition, the more movement in a scene means more individual captures of this information.

-	Lightfields have restrictions in the volume they can capture, so they really only work for content where edges are not visible. Light fields are not useful for everything, but very targeted use cases.

-	Lightfields are not truly volumetric (aka 3D), but fake 3D. If you try to go past the window of the capture area, the illusion disappears.

-	Though light field technology had been on computer visionaries’ radar since the early 1990’s, even the biggest players struggled to launch a tangible version to market. Often light fields have been relegated to white papers, rarely finding their way into a commercial project.

[With the emergence of AI-based rendering technologies [LF-14], NeRFs are the successors of light fields, addressing some of the weaknesses of light fields. NeRFs are a scene methods in which a single neural network captures the entire scene. The neural network maps the viewing angle onto the color output, providing a continuous representation of the entire scene. The downside of scene-based approaches is that the whole set of weights is required to reconstruct even portions of the scene. This leads to the main disadvantages of whole-scene NeRF methods: encoding and decoding complexity and memory requirements. While NeRFs can be utilized as 3D assets in virtual productions, Gaussian splats can serve the same purpose just as effectively. Additionally, Gaussian splats offer the advantage of being loosely connected to the underlying geometry. This opens up possibilities for editing these 3D assets in various ways. Hence, more effective methods for distributing light field and radiance fields are anticipated.]

Editor’s NOET: The above paragraph could be relocated to a separate clause under 4.3.X as part of the conclusion.