3GPP TSG-SA4 Meeting #130	S4-241905

# Orlando, USA, 18th – 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.253 | CR | 0005 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to TS 26.253 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei Technologies Co Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IVAS_Codec_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Errors and omissions identified in the detailed algorithmic description. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrections and insertion of additional description to correct the algorithmic description. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS 26.253 describing the IVAS codec algorithm will be incorrect and missing description of certain operations, which can be misleading. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.4.12.2.1, 5.3.2.3.2, 5.3.2.3.3, 5.3.2.4.12.2.3, 5.3.2.3.4.2, 5.3.2.3.4.3, 5.3.2.4.13, 5.3.2.4.10, 5.3.2.3.1.2.8, 5.3.2.4.7.2, 5.3.2.4.4.5, 5.3.2.3.10, 5.3.2.4.12.3, 6.3.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE 1

###### 5.3.2.4.12.2	Adaptive residual signal encoding

5.3.2.4.12.2.1	Adaptive residual signal encoding parameter

For the 32 kbps WB coding mode, an adaptive residual signal encoding parameter is used to determine whether to encode the residual signals of the M sub-bands in the current frame. The adaptive residual signal encoding parameter res_dmx_ratio_lt is calculated based on downmixed signal energy and residual signal energy of each of M sub-bands in the current frame, wherein spectral coefficients of the current frame are divided to obtain N sub-bands, the M sub-bands are at least some of the N sub-bands, N is a positive integer greater than 1, M ≤ N, and M is a positive integer. The parameter res_dmx_ratio_lt of the current frame is specifically determined based on the dmx_res_all and frame_nrg_ratio. The res_cod_mode_flag is calculated according to

res_cod_mode_flag=1,res_dmx_ratio_lt > 0.01 0,else

when res_cod_mode_flag is equal to 1 means encode the residual signals of the M subbands, otherwise do not encode the residual signals of the M subbands. The res_dmx_ratio is a parameter described relationship between the downmixed signal energy and the residual signal energy of each of the M sub-bands, the res_dmx_ratio is the energy parameter with a largest value in the M energy parameters. The res_dmx_ratio_lt is a parameter described a long-term smoothing parameter of the current frame. The long-term smoothing parameter (long-term smoothing residual signal encoding parameter) res_dmx_ratio_lt is calculated according to

res_dmx_ratio_lt=res_dmx_ratio* + res_dmx_ratio_lt_prev * 1-

wherein res_dmx_ratio_lt_prev represents the long-term smoothing parameter of the previous frame of the current frame, wherein

α=0.2,  frame_nrg_ratio> 3.2  and  res_dmx_ratio < 0.1)  or  (frame_nrg_ratio< 0.21 and res_dmx_ratio > 0.4) 0.05,       else

The res_dmx_ratio is calculated according to

res_dmx_ratio[b] = res_cod_NRG_S[b]/(res_cod_NRG_S[b] + (1 – g(b))∙(1 – g(b)) res_cod_NRG_M[b] + 1)

wherein res_dmx_ratio[b] represents the energy parameter of the sub-band whose sub-band index number is b,b=0,1,2,…M-1, b is greater than or equal to 0 and is less than or equal to a preset maximum sub-band index number, res_cod_NRG_S[b] represents residual signal energy of the sub-band whose sub-band index number is b, res_cod_NRG_M[b] represents downmixed signal energy of the sub-band whose sub-band index number is b, and g(b) represents a function of a side gain side_gain[b] of the sub-band whose sub-band index number is b. The frame_nrg_ratio is a parameter described relationship between a sum of residual signal energy and downmixed signal energy of the M sub-bands, and a sum of residual signal energy and downmixed signal energy of M sub-bands in a frequency-domain signal of a previous frame of the current frame.

CHANGE 2

##### 5.3.2.3.2	TD stereo coder overview

The TD stereo coder is a low-rate low-complexity parametric stereo technology where the stereo input signal is encoded in time domain. It is especially effective for scenes where the correlation between the left and the right channel of the input audio signal is low, when the background noise is fluctuating a when an interfering talker is present. The left and the right channel of the input audio signal obtained after preprocessing and delay alignment. The classifier of uncorrelated input signals and the cross-talk detector, described in 5.3.2.5, are used to select frames suitable for the TD stereo coder.The TD stereo coder comprises a method for combining the left and the right channel of the input stereo signal to obtain the primary and the secondary channel based on a downmix matrix , the channel combination ratio factor is used to construct the downmix matrix. The bitrate is adaptively distributed between the primary and the secondary channel to maximize the quality of the encoded stereo signal. The secondary channel usually requires lower amount of bits due to leveraging some parameters from the the primary channel.

CHANGE 3

##### 5.3.2.3.3	Time-domain stereo downmix

The correlated signal channel combination scheme is a channel combination scheme corresponding to a near in phase signal, wherein the near in phase signal is a stereo signal whose phase difference between a left channel signal and a right channel signal falls within[-θ, +θ]. The TD stereo downmix converts the signals in the left channel and the right channel into primary channel and secondary channel. The functionality of the TD stereo downmix is schematically shown in Figure 5.3-19.

CHANGE 4

5.3.2.4.12.2.3	Calculation of downmixed signal and residual signal during transitional frame

In stereo coding, obtaining an initial downmixed signal and an initial residual signal of a sub-band corresponding to a preset frequency band in a current frame. If the previous/current frame is a switching frame, the downmixed signal and the residual signal of the sub-band corresponding to the preset frequency band in the current frame are calculated based on a switch fade-in/fade-out factor of a current/previous frame, the initial downmixed signal, and the initial residual signal. The switch fade-in/fade-out factor of the current/previous frame is determined based on a residual signal coding parameter and an inter-frame energy fluctuation parameter. The residual signal coding parameter is used to represent an energy relationship between a downmixed signal and a residual signal of the current/previous frame, and the inter-frame energy fluctuation parameter is used to represent an energy or amplitude relationship between the current/previous frame and a frame previous to the current/previous frame. The switch fade-in/fade-out factor of the current/previous frame is determined according to

when frame_nrg_ratio>3.2 and res_dmx_ratio<0.1, switch_fade_factor=0.75; when frame_nrg_ratio<0.21 and res_dmx_ratio>0.4, switch_fade_factor=0.25;  in another case, switch_fade_factor=0.5; wherein

frame_nrg_ratio represents the inter-frame energy fluctuation parameter of the current frame/previous which is defined as a ratio of total energy of the downmixed signal and the residual signal of the current/previous frame to total energy of a downmixed signal of a frame previous to the current/previous frame and a residual signal of the frame previous to the current/previous frame; res_dmx_ratio represents the residual signal coding parameter of the current frame; switch_fade_factor represents the switch fade-in/fade-out factor of the current/previous frame.

CHANGE 5

###### 5.3.2.3.4.2	NOOP sub-mode selection

The final selection of the NOOP sub-mode is then performed based on the initial selection of the NOOP sub-mode and a modification flag related to the NOOP-specific mixing ratio, fmod_SM . The modification flag fmod_SM indicates that the NOOP-specific mixing ratio βSM needs or does not need to be modified.  If the modification flag in the previous frame indicates that the mixing ratio needs to be modified, then the final selection of the NOOP sub-mode is set to 0, i.e. NOOPfinal=0. If the modification flag of the previous frame indicates that the mixing ratio does not need to be modified, then the final selection of the NOOP sub-mode is set based on a series of conditions involving the signal classification of the primary and the secondary channel.

The channel combination scheme decision for the current frame is determined based on the signal type of in/out of phase of the stereo signal in the current frame and the channel combination scheme for the previous frame.

When the signal type of in/out of phase of the stereo signal is the near in phase signal and the channel combination scheme for the previous frame is the correlated signal channel combination scheme, determining that the initial channel combination scheme for the current frame is the correlated signal channel combination scheme.

When the signal type of in/out of phase of the stereo signal in the current frame is the near out of phase signal and the channel combination scheme for the previous frame is the anticorrelated signal channel combination scheme, determining that the initial channel combination scheme for the current frame is the anticorrelated signal channel combination scheme.

When the signal type of in/out of phase of the stereo signal in the current frame is the near in phase signal and the channel combination scheme for the previous frame is the anticorrelated signal channel combination scheme, and if signal-to-noise ratios of the left and right channel signals in the current frame are both less than a threshold, determining that the initial channel combination scheme for the current frame is the correlated signal channel combination scheme; or if the signal-to-noise ratio of the left channel signal and/or the signal-to-noise ratio of the right channel signal in the current frame are/is greater than or equal to the threshold, determining that the initial channel combination scheme for the current frame is the anticorrelated signal channel combination scheme.

When the signal type of in/out of phase of the stereo signal in the current frame is the near out of phase signal and the channel combination scheme for the previous frame is the correlated signal channel combination scheme, if the signal-to-noise ratios of the left and right channel signals in the current frame are both less than the threshold, determining that the initial channel combination scheme for the current frame is the anticorrelated signal channel combination scheme; or if the signal-to-noise ratio of the left channel signal and/or the signal-to-noise ratio of the right channel signal in the current frame are/is greater than or equal to the second threshold, determining that the initial channel combination scheme for the current frame is the correlated signal channel combination scheme.

A channel combination scheme modification decision for the current frame is performed based on the initial channel combination scheme for the current frame. A channel combination ratio factor modification flag of the previous frame indicates that a channel combination ratio factor needs or does not need to be modified, determining that the channel combination scheme for the current frame is the anticorrelated signal channel combination scheme. If a channel combination ratio factor modification flag of the previous frame indicates that a channel combination ratio factor needs to be modified, determining that the channel combination scheme for the current frame is the anticorrelated signal channel combination scheme. If the channel combination ratio factor modification flag of the previous frame indicates that the channel combination ratio factor does not need to be modified, determining whether the current frame meets a switching condition, and determining the channel combination scheme for the current frame based on a result of determining whether the current frame meets the switching condition; wherein the channel combination ratio factor modification flag is used to indicate whether the channel combination ratio factor needs to be modified. The channel combination ratio factor modification flag is used to indicate whether the channel combination ratio factor needs to be modified.

The switching condition is determined based on a frame type of a primary channel signal in the previous frame and a frame type of a secondary channel signal in the previous frame.

CHANGE 6

###### 5.3.2.3.4.3	NOOP signal coding

When operating in the NOOP sub-mode, the TD stereo coder modifies the TD stereo downmix described in clause 5.3.2.3.3. In this mode, it is the anticorrelated signal channel combination scheme corresponding to a near out of phase signal, wherein the near out of phase signal is a stereo signal whose phase difference between the left channel signal and the right channel signal falls within   and where   [180°-θ, 180°+θ]is an angle between 0° and 90°. The left and right channels of the input stereo signal are combined using a NOOP-specific mixing factor, incorporating fade-in and fade-out smoothing techniques.

……

To obtain the primary channel signal and the secondary channel signal in the TD stereo mode for the NOOP signal, a segmented time-domain downmix process is performed based on the channel combination scheme for the current frame and the channel combination scheme in the previous frame when the channel combination scheme for the current frame is different from the channel combination scheme for the previous frame. The channel combination scheme for the previous frame is the correlated signal channel combination scheme, and the channel combination scheme for the current frame is the anticorrelated signal channel combination scheme. To obtain the start/first middle segments of the primary and secondary channel signals, time-domain downmix processing on the start/first middle segments is performed by using the mixing ratio corresponding to the correlated signal channel combination scheme for the previous frame and a time-domain downmix processing manner corresponding to the correlated signal channel combination scheme for the previous frame. To obtain the end segments of the primary and secondary channel signals, time-domain downmix processing on the end segments is performed by using the mixing ratio corresponding to the anticorrelated signal channel combination scheme for the previous frame and a time-domain downmix processing manner corresponding to the anticorrelated signal channel combination scheme for the current frame. To obtain the middle segments of the primary and secondary channel signals, a weighted summation processing on the first middle segments and the second middle segments is performed. The segments can be calculated according to

CHANGE 7

##### 5.3.2.4.13	Reverberation gain parameter determination

The left channel signal and the right channel signal are treated as the first channel signal and the second channel signal. Encoder quantizes the downmixed signal, the target reverberation gain parameter, and the identification information which can reconstruct the first channel signal and the second channel signal and writes the quantized parameters into the bitstream.

The reverberation gain parameters correspond to different sub-bands of the first channel signal and the second channel signal. The target reverberation gain parameter indicates those reverberation gain parameters that needs to be encoded. The target reverberation gain parameter is determined based on at least one of coherence between energy of the first channel signal and energy of the downmixed signal and coherence between energy of the second channel signal and the energy of the downmixed signal, wherein each of the first channel signal and the second channel signal comprises a plurality of frequency bins. The identification information is used to indicate a sub-band corresponding to the target reverberation gain parameter and whether the initial reverberation gain parameter needs to be adjusted. The identification information is determined based on the target difference value which is the larger difference value in the first difference value and the second difference value. The identification information uses 1bit to indicate the first frequency band. A target attenuation factor used to adjust initial reverberation gain parameter of a target channel signal is calculated based on the first difference value and the second difference value. Each of the plurality of attenuation factors corresponds to at least one sub-band of the target channel signal, and any sub-band corresponds to only one attenuation factor. The target attenuation factor needs to be quantized and writes the quantized target attenuation factor into the bitstream. The first difference value is a sum of absolute values of difference values between energy of the first channel signal and energy of the downmixed signal at a plurality of frequency bins, and the second difference value is a sum of absolute values of difference values between energy of the second channel signal and energy of the downmixed signal at the plurality of frequency bins. When the first difference value or the second difference value is greater than 120, the reverberation gain parameter corresponding to a sub-band of a first frequency band is the target reverberation gain parameter, wherein the first frequency band is a part of all frequency bands of each of the first channel signal and the second channel signal, wherein a frequency of the first frequency band is less than a frequency of another frequency band different from the first frequency band in the first channel signal and the second channel signal. The plurality of frequency bins are in a second frequency band of each of the first channel signal and the second channel signal, and a frequency of the second frequency band is greater than a frequency of another frequency band, different from the second frequency band, in the first channel signal and the second channel signal.

CHANGE 8

##### 5.3.2.4.10	Active Downmix

The phase of a target frequency signal is linearly correlated with the IPD of the audio signal. The target frequency signal is transformed to a target time domain signal. An ITD of the audio signal is calculated based on the target time domain signal and a preset time domain peak condition. The target frequency signal is calculated according to the IPD of the audio signal and the amplitude of the target frequency signal, wherein

XM_real(k)=AM(k)*cos(IPD(k))XM_iamge(k)=AM(k)*sin(IPD(k))

XM_real(k) represents the real part of the target frequency signal, XM_iamge(k) represents the imaginary part of the target frequency signal, AM(k) represents the amplitude of the target frequency signal, IPD(k) represents the IPD of the audio signal, k represents the frequency bin.

The global IPD if used is first compensated by rotating the Left channel and leave untouched the Right channel. It is achieved as follows:

……

CHANGE 9

5.3.2.3.1.2.8	Delay estimation

In TD stereo coding, ITD is estimated by a cross-correlation coefficient of the current frame, a delay track estimation value of the current frame based on buffered ITD information of at least one past frame, an adaptive window function of the current frame. Specifically, the ITD is calculated based on a weighted cross-correlation coefficient which is obtained by the delay track estimation value and the adaptive window function.

CHANGE 10

5.3.2.4.7.2	IPD encoding scheme

In addition, a reference parameter is used to determine the IPD parameter encoding scheme. The reference parameter includes at least one of the signal characteristic parameter and the signal characteristic parameters of previous frame. The signal characteristic parameter comprises the correlation between left channel and right channel of the current frame. The signal characteristic parameters of previous frame include at least one of the correlations between left channel and right channel of previous frame, an ITD parameter of previous frame, a signal type of previous frame.

CHANGE 11

###### 5.3.2.4.4.5 	Refined ITD control mechanism

The ITD is critical to keep the stereo image stable. Sometimes ITD values calculated by the frequency domain correlation are not continuous. A difference parameter is used to represent a difference between the ITD of the current frame ITDm and the ITD of the previous frame ITDm-1. The refined ITD is determined based on the difference parameter and a characteristic parameter of the current frame. The characteristic parameter is calculated as follows.

Divide a low frequency part of the left-channel frequency-domain signal of the current frame into M sub-bands, where each sub-band includes N frequency domain amplitude values. Calculate a correlation parameter of the current frame and a previous frame according to

cori=j=0N-1L(i*N+j)∙L(-1)(i*N+j)j=0N-1L(i*N+j)∙L(i*N+j)∙j=0N-1L(-1)(i*N+j)∙L(-1)(i*N+j)

i=0,1,…,M-1, where L(i*N+j) represents a jth frequency domain amplitude value of an ith sub-band in the low frequency part of the left-channel frequency-domain signal of the current frame, L(-1)(i*N+j) represents a jth frequency domain amplitude value of an ith sub-band in a low frequency part of a left-channel frequency-domain signal of the previous frame, and cori represents a normalized cross-correlation value corresponding to an ith sub-band in the M sub-bands. And, the correlation parameter represents the degree of correlation between the current frame and the previous frame.

Calculate a peak-to-average ratio of each sub-band of the current frame par_Li.

If the ITD value of the current frame and an ITD value of the previous frame meet at least of one of the preset conditions, determine whether to reuse the ITD value of the previous frame for the current frame. The preset conditions set to

…

-	condition_6_c: an absolute value of a difference between the ITD value of the previous frame and the ITD value of the current frame is greater than half of a target value, where the target value is an ITD value whose absolute value is larger in the ITD value of the previous frame and the ITD value of the current frame,

condition6c=itd, itd - previtd> 0.5 * maxitd, previtditd> previtdprevitd,itd - previtd≤ 0.5 *maxitd, previtditd> previtd

The ITD fine control result itd_fine sets to

itd_fine=previtd, conditiontotal is true)itd,others

where

conditiontotal=(condition_1234 and ((condition_5 and condition_6_b) or condition_6_c)) or (condition_1234 and condition_6_a), condition_1234 = condition_1 and (condition_2 or condition_3 or condition_4)

In addition, the ITD of the current frame could reuse the ITD of the previous frame ITD(m-1) based on the hangover counter and the initial ITD of the current frame The hangover counter is the quantity of target frames that are allowed to appear consecutively. A characteristic information which is described by the signal-to-noise ratio and the peak feature of cross correlation coefficients of the stereo signal is used to control the hangover counter. When the signal-to-noise ratio and the peak feature of the cross-correlation coefficients meet at least one of preset conditions, the hangover counter will be reduced by adjusting at least one of a target frame count representing the quantity of target frame that appears consecutively and a threshold of the target frame count which allows the target frame to appear consecutively. And, the hangover counter will be reduced by increasing the target frame count or reducing the threshold of the target frame count.

When the signal-to-noise ratio meets the signal-to-noise ratio condition, stopping reusing the ITD value of the previous frame as the ITD value of the current frame. The signal-to-noise ratio condition is set as the SNR value is less than 0.006 or the SNR value is greater than 2 000 000.

CHANGE 12

##### 6.3.2.2.3	Reusing LP filter coefficients of the primary channel and dequantization of LSF parameters for the secondary channel

…

The quantized LSF parameter of the secondary channel signal is obtained by using a sum of the predicted LSF parameter and the prediction residual, and the prediction residual is obtained by performing two-stage prediction on the LSF parameter of the secondary channel signal based on the spectrum-broadened LSF parameter of the primary channel signal.

Decoding the spread LSF parameters of the primary channel signal comprises obtaining a quantized LSF parameter of a primary channel signal, obtaining a target adaptive spreading factor of a stereo signal, spreading the quantized LSF parameter of the primary channel signal based on the target adaptive spreading factor. The spread LSF parameter of the primary channel signal is a quantized LSF parameter of a secondary channel signal in the current frame, or it is used to determine a quantized LSF parameter of a secondary channel signal in the current frame.

CHANGE 13

##### 5.3.2.10 Reusing LP filter coefficients of the primary channel and quantization of LSF parameters for the secondary channel

In TD stereo coding a prediction residual of an LSF parameter of a secondary channel signal in the current frame is determined based on an original LSF parameter of the secondary channel signal and the spectrum-broadened LSF parameter of the primary channel signal. The spectrum-broadened LSF parameter is performed by spectrum broadening on a quantized LSF parameter of a primary channel signal in a current frame. And then the prediction residual is quantitated. The spectrum-broadened LSF parameter of the primary channel signal is calculated by performing pull-to-average processing on the quantized LSF parameter of the primary channel signal, wherein the pull-to-average processing is performed according to the following formula:

LSFSB(i)=β⋅LSFP(i)+(1-β)⋅LSFS(i)

wherein LSFSB represents a vector of the spectrum-broadened LSF parameter of the primary channel signal, LSFP(i) represents a vector of the quantized LSF parameter of the primary channel signal, i represents a vector index, β represents a broadening factor, 0 < β < 1, for example β = 0.91, LSFS represents a mean vector of the original LSF parameter of the secondary channel signal, 1 ≤ i ≤ M, i is an integer, and M represents a linear prediction parameter.

When the LSF parameter of the secondary channel signal does not meet a reusing condition, the prediction residual of an LSF parameter of the secondary channel signal is obtained by using a difference between the original LSF parameter of the secondary channel signal and a predicted LSF parameter that is obtained by performing two-stage prediction on the LSF parameter of the secondary channel signal based on the spectrum-broadened LSF parameter of the primary channel signal, wherein reusing means that the quantized LSF parameter of the secondary channel signal may be obtained based on the quantized LSF parameter of the primary channel signal.

When the LSF parameter of the secondary channel signal meets a reusing condition, Target adaptive spreading factor is determined based on quantized LSF parameter of a primary channel signal and an LSF parameter of a secondary channel signal in the current frame. The quantized LSF parameter of a primary channel signal and the target adaptive spreading factor are written into the bitstream. The target adaptive spreading factor is minimizing a weighted distance between a spectrum-spread LSF parameter of the primary channel signal and the LSF parameter of the secondary channel signal. The quantized LSF parameter of the primary channel signal, the LSF parameter of the secondary channel signal, and the adaptive spreading factor β satisfy the following formula:

β=i=1Mwi-LSFS2i+LSFSiLSFSi-LSFSiLSFPi+LSFSiLSFPii=1Mwi-LSFS2i-LSFP2i+2LSFSiLSFPi, wherein

LSFS is a vector of the LSF parameter of the secondary channel signal, LSFP is a vector of the quantized LSF parameter of the primary channel signal, LSFS is a mean vector of the LSF parameter of the secondary channel signal, i is a vector index, 1 ≤ i ≤ M, i is an integer, M is a linear prediction order, and wi is an ith weighting coefficient.

The quantized LSF parameter of the secondary channel signal is obtained based on the target adaptive spreading factor and the quantized LSF parameter of the primary channel signal. The adaptive spreading factor is quantized to obtain the target adaptive spreading factor.

The weighting factor is a parameter value greater than 0 and less than 1. It is determined based on coding mode of a to-be-encoded signal in a stereo signal and preset correspondence between the mode and the parameter value. The coding mode comprises at least one of the following modes: bitrate, bandwidth, channel number, manner of obtaining a target line spectral frequency parameter of the to-be-encoded signal. The manner of obtaining the target line spectral frequency parameter of the to-be-encoded signal comprises at least one of the methods: quantizing an original line spectral frequency parameter or obtaining the target line spectral frequency parameter through prediction. The weighting factor is calculated based on the parameter value and an energy spectrum of a linear prediction filter that is corresponding to the original line spectral frequency parameter, wherein the weighting factor is usable in calculating a distance representative of spectral distortion between the original line spectral frequency parameter and the target line spectral frequency parameter.

CHANGE 14

###### 5.3.2.4.12.3	Residual signal encoding mode information determination

The encoding mode indication information of the residual signal of the current frame is obtained by at least one of the following information: residual signal encoding status of previous frame, updating manner flag for a long-term smooth parameter of the current frame, status change parameter relative to previous frame. The encoding status of the previous frame is used to indicate at least one of the following cases: the quantity of consecutive frames whose residual signals are encoded before the current frame, a quantity of consecutive frames whose residual signals are not encoded before the current frame, and encoding modes of residual signals of the previous frame. The status change parameter is a ratio of energy of the current frame to energy of the previous frame. The encoding mode that used to indicate whether to encode the residual signal of the current frame is determined based on the encoding mode indication information and the initial encoding mode of the residual signal of the current frame. The initial encoding mode of the residual signal of the current frame is determined based on the downmixed signal energy of the current frame and the residual signal energy of the current frame.

If the following conditions are met, the encoding mode of the residual signal of the current frame is the encoding mode of the residual signal of the previous frame. The conditions include that the initial encoding mode is different from the encoding mode of the previous frame which is closely adjacent to the current frame, and the encoding mode of the previous frame indicates to encode the residual signal of the previous frame, and an additional condition. The additional condition is the quantity of consecutive frames whose residual signals are encoded before the current frame is less than a threshold or the updating manner flag for the long-term smooth parameter is 0, and the encoding mode of the residual signal of the previous frame is not modified. If the additional condition is not met, the encoding mode of the current frame is the initial encoding mode. If the initial encoding mode is the same as the encoding mode of the previous frame, the encoding mode of the residual signal of the current frame is the initial encoding mode.

If the following conditions are met, the encoding mode of the residual signal of the current frame is the encoding mode of the residual signal of the previous frame. The conditions include that the initial encoding mode is different from the encoding mode of previous frame which is closely adjacent to the current frame, and the encoding mode of previous frame indicates not to encode the residual signal of the previous frame, and a second additional condition. The second additional condition is the quantity of consecutive frames whose residual signals are not encoded before the current frame is less than a threshold or the value of the status change parameter is not less than a second threshold, and not greater than a third threshold. If the second condition is not met, the encoding mode of the current frame is the initial encoding mode.

In addition, the encoding mode the residual signal of the current frame can be modified based on the encoding mode indication information of the residual signal of the current frame. If the encoding mode of the residual signal of the current frame is different from the encoding mode of previous frame which is closely adjacent to the current frame, and the encoding mode of previous frame is not modified, the encoding mode of the residual signal of the current frame is used to indicate the encoding mode of the residual signal of the current frame.

END OF CHANGES