3GPP TSG SA WG4 Meeting #130                                                        	S4- 241913

Nov 18-22, 2024, Orlando, US

Source:	Meta USA, Lenovo

Title:	Discussion on range and granularity of bit rate adaptation for XR applications

Document for:	Discussion

Agenda item:	9.3

# 1. Introduction

This paper discusses the LS input from RAN2 (R2-2409273/S4-241803) related to appropriate range and granularity of bit rate adaptation for XR applications.

# 2. Discussion

R2-2409273: RAN2 are discussing MAC-layer signaling enhancements to enable XR applications to adapt bit rates in response to uplink congestion. To proceed with the design, RAN2 would like to request input from SA4 on the appropriate range and granularity of uplink bit rates that need to be supported for the signaling enhancements.

Data streams that can be catered to XR applications can be anything from JPEG formatted images, H.264/265/AV1 video codec streams, volumetric data stream, latent codes, and/or any data streams that are necessary to compose an XR service. The only hard rule is what can the data link/transport support.

Media rate requirements for typical consumer applications can range from 100s of kbps to 100s of Mbps. The throughput rate, based on access network feedback mechanism (e.g., MAC-CE bit rate recommendations), should be defined at least with  finer (e.g., 256kbps to 1Mbps especially in low and medium rate ranges) granularity than what has been specified in current RAN specification in order to be useful by the application.

In UL, mobile XR applications are limited by typical available UL throughput in practical deployments and often cap out their traffic requirements based on its own UL bandwidth estimation algorithm. Today, that UL average bit rates observed are in the high Kbps to low Mbps range. For xR applications, higher average UL throughput is expected (e.g., as for advanced AR/MR applications that may include UL streaming) and the target available range should be at least up to 30 Mbps.

Short analysis on RAN’s MAC layer signalling for bit rate recommendations

MAC-layer signalling today can support the following bit rate recommendations [ref: TS 38.321/table 6.1.2.20-1, TS 38.331]

Table 6.1.3.20-1: Values (kbit/s) for Bit Rate field

| Index | NR Recommended Bit Rate value [kbit/s] | Index | NR Recommended Bit Rate value [kbit/s] |
| --- | --- | --- | --- |
| 0 | Note 1 | 32 | 700 |
| 1 | 0 | 33 | 800 |
| 2 | 9 | 34 | 900 |
| 3 | 11 | 35 | 1000 |
| 4 | 13 | 36 | 1100 |
| 5 | 17 | 37 | 1200 |
| 6 | 21 | 38 | 1300 |
| 7 | 25 | 39 | 1400 |
| 8 | 29 | 40 | 1500 |
| 9 | 32 | 41 | 1750 |
| 10 | 36 | 42 | 2000 |
| 11 | 40 | 43 | 2250 |
| 12 | 48 | 44 | 2500 |
| 13 | 56 | 45 | 2750 |
| 14 | 72 | 46 | 3000 |
| 15 | 88 | 47 | 3500 |
| 16 | 104 | 48 | 4000 |
| 17 | 120 | 49 | 4500 |
| 18 | 140 | 50 | 5000 |
| 19 | 160 | 51 | 5500 |
| 20 | 180 | 52 | 6000 |
| 21 | 200 | 53 | 6500 |
| 22 | 220 | 54 | 7000 |
| 23 | 240 | 55 | 7500 |
| 24 | 260 | 56 | 8000 |
| 25 | 280 | 57 | Reserved |
| 26 | 300 | 58 | Reserved |
| 27 | 350 | 59 | Reserved |
| 28 | 400 | 60 | Reserved |
| 29 | 450 | 61 | Reserved |
| 30 | 500 | 62 | Reserved |
| 31 | 600 | 63 | Reserved |
| Note 1: For bit rate recommendation message this index is used for indicating that no new recommendation on bit rate is given. |  |  |  |

X: Bit rate multiplier. For UEs supporting recommended bit rate multiplier, when bitRateMultiplier is configured for the logical channel indicated by LCID field, X field set to 1 indicates the actual value of bit rate is the value corresponding to the index indicated by the Bit Rate field multiplied by bitRateMultiplier as specified in TS 38.331 [5].

Ref: TS 38.331 bitRateMultiplier-r16 ENUMERATED {x40, x70, x100, x200}

Based on above, under current RAN specifications, it seems that to cater for XR services bit rates a UE needs to support the bit rate multiplier feature. To this end, Table 1 below renders the range of prospective bit rate recommendations based on all the possible multipliers. The application of different multipliers to the base table (i.e., Table 6.1.3.20-1 above) is highlighted by shading as per the attached legend scheme: yellow indicates x40, orange indicates x70, green indicates x100, blue indicates x200. The base table multiplier x1 is left unshaded.

Although it may be apparent that current bit rate multipliers and the resulting extended table provide enough granularity, this generally implies frequent change of multipliers among many adjacent step sizes as highlighted by the table below. In particular, for the case of UL XR applications this fact is captured within first 4 columns covering ranges from 100s kbps up to 30+ Mbps.

Configuring the bitRateMultiplier often for a UE implies in practice a L3 RRC signalling of network configuration for changing the bitRateMultiplier value. Taking such an approach is not a sensible way to achieve with low latency more granular step sizes for XR applications, defeating the purpose of MAC-CE bit rate recommendations in support of avoiding/alleviating UL congestion. Alternatively, applying current multipliers and/or base table, may not lead to a sufficiently granular step size and/or flexible range for XR applications. It is therefore advisable for an XR optimized design to support “out-of-the-box” granular enough bit rate recommendations at MAC layer, i.e., without often reconfiguration needs of bit rate recommendation multipliers.

We would like to recommend RAN to consider following granularity requirements for bit rate recommendations when the available data rate is under 30Mbps.

- <100 kbps steps up to 1 Mbit available rate

- <200 kbps steps up to 2 Mbits available rate

- <400 kbps steps up to 8 Mbits available rate

- <1000 kbps steps up to 16 Mbits available rate - <2000 kbps steps up to 30 Mbits available rate

NOTE: For available bit rates over 30Mbps, the step size is FFS.

Following practical arguments are generally applicable to the above granularity recommendations:

-  At lower bit rates (< 1Mbit), XR traffic is mostly composed of pose/tracking information, audio and/or maybe some low-quality image objects, and the potential for source adaptation is low comparing trade-off penalties one needs to take in QoE. It is thus recommendable to have a much finer granularity at the very low end of bit rates for XR services with 100 kbps as an upper threshold;

- A similar argument is valid for < 2Mbits data rates where already some low-quality video streams may be accommodated by such low bit rates;

- Already with 5+ Mbits one can offer XR services that have good Mean Opinion Score (MOS) QoE metrics, whereas 10+ Mbits very good and excellent QoE are attainable even with coarser step sizes (e.g., 1-2 Mbits) for source bit rate adaptation.

# 3. Conclusion & proposal

Generate a reply LS to RAN2 to indicate that it is recommended to consider MAC-layer signaling enhancements designs optimized for XR taking considering the above UL granularity step size for bit rates up to 30 Mbps.