3GPP TSG-SA4#130 	S4-241947

# Orlando, US, 18-22 November 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.567 | CR |  | rev |  | Current version: | 0.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [SR_IMS] QoE report formats |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, InterDigital Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SR_IMS |  |  |  |  |  | Date: |  |  | 2024-11-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The QoE report format for split rendering metrics is missing in the TS 26.567. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add metrics reporting format in clause 6.3.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete Technical Specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

## 6.3 Metrics Reporting

## 6.3.2 Report format

The QoE report is formatted as an XML document that complies with the XML schema in listing 16.4.1of TS 26.114 [7].

The schema in listing 6.3.2-1 allows the SR-DCMTSI client to report QoE metrics using the metrics reporting mechanism specified in clause 16.4 of TS 26.114 [7].

The filename of this schema is "TS26567_SR_IMSQoEMetrics.xsd".

Listing 6.3.2-1: SR_IMS QoE Metrics XML schema

<?xml version="1.0" encoding="UTF-8"?> <xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema" targetNamespace="urn:3gpp:metadata:2024:RTC:SR_IMSQoEMetrics" xmlns="urn:3gpp:metadata:2024:RTC:SR_IMSQoEMetrics" xmlns:sv="urn:3gpp:metadata:2017:MTSI:schemaVersion" elementFormDefault="qualified"> <xs:element name="QoeReport" type="QoeReportType"/> <xs:complexType name="QoeReportType"> <xs:sequence> <xs:element name="statisticalReport" type="starType" minOccurs="0" maxOccurs="unbounded"/> <xs:any namespace="##other" processContents="skip" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="starType"> <xs:sequence> <xs:element name="mediaLevelQoeMetrics" type="mediaLevelQoeMetricsType" minOccurs="1" maxOccurs="unbounded"/> <xs:element ref="sv:delimiter"/> <xs:element name="latencyQoeMetric" type="QoeMetricType" minOccurs="0" maxOccurs="unbounded"/> <xs:element ref="sv:delimiter"/> </xs:sequence> <xs:attribute name="startTime" type="xs:unsignedLong" use="required"/> <xs:attribute name="stopTime" type="xs:unsignedLong" use="required"/> <xs:attribute name="callId" type="xs:string" use="required"/> <xs:attribute name="clientId" type="xs:string" use="required"/> <xs:attribute name="qoeReferenceId" type="xs:hexBinary" use="optional"/> <xs:attribute name="recordingSessionId" type="xs:hexBinary" use="optional"/> <xs:attribute name="dnn" type="xs:string" use="optional"/> <xs:attribute name="snssai" type="xs:unsignedLong" use=”optional"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="mediaLevelQoeMetricsType"> <xs:sequence> <xs:any namespace="##other" processContents="skip" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="mediaId" type="xs:integer" use="required"/> <xs:attribute name="totalCorruptionDuration" type="unsignedLongVectorType" use="optional"/> <xs:attribute name="numberOfCorruptionEvents" type="unsignedLongVectorType" use="optional"/> <xs:attribute name="corruptionAlternative" type="xs:string" use="optional"/> <xs:attribute name="totalNumberofSuccessivePacketLoss" type="unsignedLongVectorType" use="optional"/> <xs:attribute name="numberOfSuccessiveLossEvents" type="unsignedLongVectorType" use="optional"/> <xs:attribute name="numberOfReceivedPackets" type="unsignedLongVectorType" use="optional"/> <xs:attribute name="framerate" type="doubleVectorType" use="optional"/> <xs:attribute name="totalJitterDuration" type="doubleVectorType" use="optional"/> <xs:attribute name="numberOfJitterEvents" type="unsignedLongVectorType" use="optional"/> <xs:attribute name="totalSyncLossDuration" type="doubleVectorType" use="optional"/> <xs:attribute name="numberOfSyncLossEvents" type="unsignedLongVectorType" use="optional"/> <xs:attribute name="networkRTT" type="unsignedLongVectorType" use="optional"/> <xs:attribute name="internalRTT" type="unsignedLongVectorType" use="optional"/> <xs:attribute name="codecInfo" type="stringVectorType" use="optional"/> <xs:attribute name="codecProfileLevel" type="stringVectorType" use="optional"/> <xs:attribute name="codecImageSize" type="stringVectorType" use="optional"/> <xs:attribute name="averageCodecBitrate" type="doubleVectorType" use="optional"/> <xs:attribute name="callSetupTime" type="xs:unsignedLong" use="optional"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:element name="latencyQoeMetric" type="QoeMetricType"/> <xs:complexType name="QoeMetricType"> <xs:sequence> <xs:choice> <xs:element name="poseToRenderToPhoton" type="PoseToRenderToPhotonType"/> <xs:element name="renderToPhoton" type="RenderToPhotonType"/> <xs:elementname="roundTripInteractionDelay" type="RoundTripInteractionDelayType"/> <xs:element name="userInteractionDelay" type="UserInteractionDelayType"/> <xs:element name="ageOfContent" type="AgeOfContentType"/> <xs:element name="sceneUpdateDelay" type="SceneUpdateDelayType"/> <xs:element name="metadataDelay" type="MetadataDelayType"/> <xs:element name="dataFrameDelay" type="DataFrameDelayType"/> </xs:choice> <xs:element ref="sv:delimiter"/> <xs:any namespace="##other" processContents="skip" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="PoseToRenderToPhotonType"> <xs:attribute name="avgPoseToRenderToPhoton" type="doubleVectorType" use="optional"/> <xs:attribute name="minPoseToRenderToPhoton" type="unsignedIntVectorType" use=" optional"/> <xs:attribute name="maxPoseToRenderToPhoton" type="unsignedIntVectorType" use="optional"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="RenderToPhotonType"> <xs:attribute name="avgPoseToRenderToPhoton" type="doubleVectorType" use="optional"/> <xs:attribute name="minRenderToPhoton" type="unsignedIntVectorType" use="optional"/> <xs:attribute name="maxRenderToPhoton" type="unsignedIntVectorType" use="optional"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="RoundTripInteractionDelayType"> <xs:attribute name="avgRoundTripInteractionDelay" type="doubleVectorType" use="optional"/> <xs:attribute name="numberOfUserActions" type="unsignedIntVectorType" use="optional"/> <xs:attribute name="minRoundTripInteractionDelay" type="unsignedIntVectorType" use="optional"/> <xs:attribute name="minActionIds" type="unsignedIntVectorType" use="optional"/> <xs:attribute name="maxRoundTripInteractionDelay" type="UnsignedIntVectorType" use="optional"/> <xs:attribute name="maxActionIds" type="unsignedIntVectorType" use="optional"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="UserInteractionDelayType"> <xs:attribute name="avgUserInetractionDelay" type="doubleVectorType" use="optional"/> <xs:attribute name="numberOfUserActions" type="unsignedIntVectorType" use="optional"/> <xs:attribute name="minUserInetractionDelay" type="unsignedIntVectorType" use="optional"/> <xs:attribute name="minActionIds" type="unsignedIntVectorType" use="optional"/> <xs:attribute name="maxUserInteractionDelay" type="unsignedIntVectorType" use="optional"/> <xs:attribute name="maxActionIds" type="unsignedIntVectorType" use="optional"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="AgeOfContentType"> <xs:attribute name="avgAgeOfContent" type="doubleVectorType" use="optional"/> <xs:attribute name="mumberOfSceneEvents" type="unsignedIntVectorType" use="optional"/> <xs:attribute name="minageOfContent" type="unsignedIntVectorType" use="optional"/> <xs:attribute name="maxAgeOfContent" type="unsignedIntVectorType" use="optional"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="SceneUpdateDelayType"> <xs:attribute name="avgSceneUpdateDelay" type="doubleVectorType" use="optional"/> <xs:attribute name="numberOfSceneUpdates" type="unsignedIntVectorType" use="optional"/> <xs:attribute name="minsceneUpdateDelay" type="unsignedIntVectorType" use="optional"/> <xs:attribute name="maxsceneUpdateDelay" type="unsignedIntVectorType" use="optional"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="MetadataDelayType"> <xs:attribute name="avgMetadataDelay" type="doubleVectorType" use="optional"/> <xs:attribute name="numberOfMetadataMessages" type="unsignedIntVectorType" use="optional"/> <xs:attribute name="minMetadataDelay" type="unsignedIntVectorType" use="optional"/> <xs:attribute name="maxMetadataDelay" type="unsignedIntVectorType" use="optional"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="DataFrameDelayType"> <xs:attribute name="avgDataFrameDelay" type="doubleVectorType" use="optional"/> <xs:attribute name="numberOfDataFrames" type="unsignedIntVectorType" use="optional"/> <xs:attribute name="minDataFrameDelay" type="unsignedIntVectorType" use="optional"/> <xs:attribute name="maxDataFrameDelay" type="unsignedIntVectorType" use="optional"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:simpleType name="doubleVectorType"> <xs:list itemType="xs:double"/> </xs:simpleType> <xs:simpleType name="stringVectorType"> <xs:list itemType="xs:string"/> </xs:simpleType> <xs:simpleType name="unsignedLongVectorType"> <xs:list itemType="xs:unsignedLong"/> </xs:simpleType> </xs:schema>

_____________________ End of 1st Change________________________________