3GPP TSG-SA WG4 Meeting #130	S4-241960

# Orlando,US - 18th November 2024 - 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | 0019 | rev | <Rev#> | Current version: | <Version#> |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AMD] WT #13: Candidate technologies |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD |  |  |  |  |  | Date: |  |  | 12/11/2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | 19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | No candidate technologies yet in the QUIC-based segment delivery in 5GMS clause. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Based on the discussion on S4aI240165 and revised to S4aI240198 (not disposed and revised to S4-241930), it was mentionned that concrete possible technologies using QUIC help to understand better the analysis of the Work Topic. Based on the discussion on S4aI240165 (whose note are copied below), we propose to list those possible technologies with the important caveats that: - Those are not candidade solutions, as for other Work Topics, but candidate technologies. - The listed candidate technologies are not endorsed by the study but merely recognised to be existing. On point 1), the reason is that the Work Topic was built on the idea of exploring the possible benefits of QUIC (technology based) and the slving a concrete problem. Therefore, there is no solutions rather technologies leveraging QUIC directly or indirectly. On point 2), the Work Topic is clearly not mean to study any those technologies in details nor comparing them. However, we believe it worthwile to capture those technologies seems they were anyway mentioned during the discussions on the Work Topic as assumed application-layer protocols. - Emmanuel presents r01_BBC - Richard: Call flows are good. In the middle call flow there is still an error, wrong sequence 15a, 15 - Imed: You need a protocol on top of QUIC, is it HTTP/3 or is it a different protocol? - Emmanuel: It was MPEG-DASH part 6, but our intention is more the latter. We want to focus on QUIC streams, for parallel delivery. We need to improve and clarify. - Rufael: Follow-up, this is just a DASH diagram, there are no specific issue for QUIC. What is the delta? - Richard: All stuff in blue is the delta. Like init segment - Thomas: The in-concreteness makes it very difficult, because you always need to assume that there is a new delivery protocol. - Emmanuel: We have started the study like this, so we should make sure that we either continue like this or start from functionality. - Richard: M2 and M4 permits H3 already - Saba: Similar comments, we should understand what is on the application layer to make an analysis. - Emmanuel: We try to address your comments. - Emmanuel: We agree on comments. We would not study MOQ, but we need some more offline discussion. - Richard: We should go into details of MOQ potentially, but this is early. Will be interesting what gaps we observe? - Emmanuel: No gaps likely. - Fred: to be noted - Thomas: no need to be noted, nothing wrong. - Rufael: What specific parts are for QUIC? - Richard: merge? - Rufael: would prefer keep it individual - Richard: merging would be easier. - Rufael: do not think it is mergeable |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No progress of the QUIC topic in FS_AMD. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change #1

[x7]	Nguyen, M., Nys, P., Pham, S., Silhavy, D., Arbanowski, S., & Steglich, S., “Toward WebTransport Support in HTTP Adaptive Streaming”, in Tenth International Conference on Communications and Electronics 2024 (ICCE) (pp. 96-101). IEEE. 2024, July.

This added reference is already proposed in S4-241971, but it is repeated here for self-contained review.

Change #2 (new text compared to CR 19)

### 5.24.6	Candidate technologies

#### 5.24.6.1	General

In this clause, the following candidate technologies are instantiations of the following types of 5GMS Clients:

- Media-independent QUIC-aware 5GMS Client

- Media-optimised QUIC-aware 5GMS Client

NOTE:	The QUIC-agnostic 5GMS Client type has no instantiation here since this case is covered in clause 5.4.

The candidate technologies in this clause are merely listed for the purpose of inventorying. The presence of a candidate technology in this list does not bear any sense of endorsement or selection. As a matter of fact, this list is not guaranteed to be exhaustive and other possible candidate technologies leveraging QUIC for segmented content delivery could be missing at the time of writing.

#### 5.24.6.2	MPEG-DASH over HTTP/3 with server push and priority information

##### 5.24.6.2.1	Prerequisites

For this candidate technology, the following specifications are necessary:

- MPEG-DASH part 1

- The HTTP/3 protocol, including the server push functionality [H3]

- The Extensible Prioritization Scheme for HTTP [HTTP-PRIO]

##### 5.24.6.2.2	Description

This candidate technology is an instantiation of the media-independent QUIC-aware 5GMS Client. It enables the delivery of DASH content to a UE using the HTTP/3 protocol on the M4 interface. In addition, both the 5GMS AS and the Media Player support the server push functionality and the extensible prioritisation scheme for HTTP.

The same call flow as in figure 5.24.3.3-1 is applicable for this candidate technology.

#### 5.24.6.3	MPEG-DASH part 6 over WebTransport

##### 5.24.6.3.1	Prerequisites

For this candidate technology, the following specifications are necessary:

- MPEG-DASH part 1

- MPEG-DASH part 6 [DASH-6]

- The WebTransport protocol over HTTP/3 [WT-H3]

##### 5.24.6.3.2	Background

Published in 2017, MPEG-DASH part 6: DASH with Server Push and WebSockets is the part of the MPEG-DASH standard that enables a DASH server to send multiple segments to a DASH client without the need to receive individual segment requests. To this end, the [DASH-6] defines the concept of push directives sent by the DASH client which indicates to the DASH server which segments should be pushed. Two instantiations are defined in [DASH-6]. The first instantiation is based on the HTTP/2 server push functionality and specific HTTP header extensions for the DASH client to send the so-called push directives. The second instantiation is based on the WebSocket protocol and comprises a set of messages for the exchange of MPD, segment and push directives. This constitutes the WebSocket sub-protocol for DASH registered as “2016.serverpush.dash.mpeg.org” on IANA. The WebSocket sub-protocol for DASH uses the binary format for all messages exchanged over the WebSocket connection.

##### 5.24.6.3.2	Description

This candidate technology is an instantiation of the media-optimised QUIC-aware 5GMS Client. It enables the delivery of DASH content to a UE using the WebTransport over HTTP/3 protocol on the M4 interface. In addition, both the 5GMS AS and the Media Player support the WebSocket sub-protocol for DASH defined in MPEG-DASH part 6 define in clause 8.2 of [DASH-6]. However, the communication runs over a WebTransport connection instead of a WebSocket connection.

#### 5.24.6.4	Media-over-QUIC

##### 5.24.6.3.1	Prerequisites

For this candidate technology, the following specifications are necessary:

- MPEG-DASH part 1

- The Media-over-QUIC protocol [MoQ]

##### 5.24.6.3.2	Description

This candidate technology is an instantiation of the media-optimised QUIC-aware 5GMS Client. It enables the delivery of DASH content to a UE using the WebTransport over HTTP/3 protocol or the QUIC protocol on the M4 interface. In addition, both the 5GMS AS and the Media Player support the [MoQ] transport protocol. The media content delivered over the [MoQ] transport protocol is assumed to be DASH segments and it is also assumed that the MPD has been delivered in the initialisation phase such that the Media Player is able to generate SUBSCRIBE messages with the information related to desired Representation or simply called tracks in the context of MoQ.

#### 5.24.6.5	Push-based streaming over WebTransport with server-side throughput estimation

##### 5.24.6.3.1	Prerequisites

For this candidate technology, the following specifications are necessary:

- MPEG-DASH part 1

- The WebTransport protocol over HTTP/3 [WT-H3]

- A specific WebTransport sub-protocol for DASH

##### 5.24.6.3.2	Description

This candidate technology is an instantiation of the media-optimised QUIC-aware 5GMS Client. It enables the delivery of DASH content to a UE using the WebTransport over HTTP/3 protocol on the M4 interface. In addition, both the 5GMS AS and the Media Player support the protocol described in [x7] which in essence works as follows:

- The delivery of the MPD to the DASH client is assumed to have taken place prior to the connection.

- The DASH client connects to the DASH server and establishes a WebTransport connection.

- The DASH server sends default start-up quality for the audio and video Adaptation Set.

- The DASH client starts receiving the segments and starts sending back on the same bidirectional stream the Qlog metrics reports.

- Based on the received Qloq reports, the DASH server continuously estimates the throughput available to the DASH client.

- When the new throughput is estimated, the DASH server may decide to change the pushed Representation or inform the client about the new estimated throughput for the DASH client to switch. (the paper does not explicitly mention which option is used)

END OF CHANGES