3GPP TSG-SA4#130 	S4-241963

# Orlando, US, 18-22 November 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.567 | CR |  | rev |  | Current version: | 0.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [SR_IMS] Updates to procedures of session modification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SR_IMS |  |  |  |  |  | Date: |  |  | 2024-11-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The general procedures for split rendering session modification are mixing P2P and P2A scenarios in the same call flows in TS 26.567. It would be clearer to have a general procedure and separate procedures for different scenarios. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Revise the general procedures in 7.2 Remove P2P related procedures from 7.2 and provide a separate procedure in 7.2.1. Fix bugs of procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete Technical Specification and introducing misleading. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

## 7.2	Procedures for session modification

A UE may trigger split rendering session modification during an established split rendering session e.g., if the current session no longer meets the media rendering requirements. If the existing MF does not meet the new requirements, the session is migrated to a new MF. The session modification procedure in such a case is shown in Figure 7.2-1.

Figure7.2-1: General procedures for session modification

The steps are as follows:

Step 1: IMS session establishment including bootstrap data channel.

Step 2: split rendering session establishment between UE1 and the MF1 (details see clause 7.1)

Step 3: split rendering session is running.

Step 4: During a running split rendering session, when the UE1 discovers that its media capabilities cannot meet the related media rendering requirements, the UE1; can send a request to modify the split rendering session to the IMS AS.

Step 5: The IMS AS sends updated event notifications to DCSF to modify the current split rendering session.

Step 6: The DCSF receives event notification from the IMS AS and  process the session modification request based on the information in the received notification (i.e. associated split rendering related information). The DCSF manages (if applicable) application data channel resources at the MF which meet the new split rendering request; to instruct IMS AS to terminate the media flow of the UE1 to the new MF(s).

Step 7: The IMS AS receives the updated data channel control instructions from the DCSF.

Note: here it is assumed that there are more than one MF in the network. It is also assumed that the serving MF does not satisfy the new request (due to unavailability).

Step 8: The (new) MF2 is discovered (if the existing serving MF1 does not satisfy the UE1 request for session modification).

Editor’s Note: MF discovery for split rendering is FFS.

Step 9: UE1 sends application request to the newly discovered MF2 to request an application list.

Step 10: Accordingly, the MF2 responds with application list including an identifier that if the application is capable to be split rendered. DCSF receives the list and forward to the UE1.

Step 11: The IMS AS sends a split rendering session modification request to the DC AS via DCSF through the established application data channel, the request may include the information of the objects to be rendered in IMS network.

Step 12: The DC AS sends a description of the split rendering output to the IMS AS via DCSF.

Step 13: The IMS AS sends the media resource allocation request to the MF2, to reserve media rendering resource for the UE1.

Step 14: When the resources are allocated successfully, the MF2 returns a successful response to the IMS AS.

Step 15: The IMS AS returns a successful response to the UE1.

Step 16: The modified split rendering session is established between UE1 and MF2 through the data channel.

Note: MF1 should serve the split rendering session until MF2 takes over.

Editor’s note: more details of session modification for P2P and P2A scenarios are provided in clause 7.2.1 and 7.2.2.

7.2.1 Procedures for P2P session modification

The figure 7.2.1-1 indicates the procedures of split rendering session modification for P2P scenarios.

Figure 7.2.1-1: Procedures for P2P split rendering session modification

The steps are as follows:

Steps 1: IMS session establishment between UE1 and UE2, including bootstrap session establishment by UE1 and UE2 with originating MF.

Steps 2 to 4: Split rendering session establishment between UE1 and UE2 anchored by MF1 (details see clause 7.1.1).

Step 5 to 17: The split rendering session is modified; the new session is established between UE1 and MF2 through the data channel.(details see clause 7.2).

Step 18: The new split rendering session is established between UE1 and UE2 anchored by MF2.

_____________________ End of 1st Change________________________________