Source:	Nokia

Title:	Considerations on negotiating IVAS

Document for:	Discussion

Agenda Item:	7.5 – IVAS_Codec_Ph2

# 1. Introduction

Several past and present meeting inputs, e.g., [1], [2], [3], [4], [5], [6], and [7] consider IVAS Levels and enabling immersive audio calls between IVAS UEs that can have significantly different immersive audio capture and reproduction implementations and capabilities. This input discusses some aspects that should be considered in negotiating an immersive voice call.

# 2. Discussion

Implementation complexities and device capabilities both vary over a wide range. In view of the source, an IVAS UE should at minimum be able to encode any IVAS input format according to its Level. An IVAS UE should at minimum be able to decode any IVAS bitstream (of any Level), and to furthermore, decode and render according to its configuration (e.g., binaural) any IVAS bitstream according to its Level.

To enable highest quality of experience, the source considers that it is generally the send side that knows best what and how is captured to generate the immersive audio input for IVAS. This includes the selection of the immersive audio format, resulting in coded format. On the other hand, it is the receive side that knows best what and how the immersive audio will be reproduced with. This includes the selection of output format or the rendering process, which we can also call simply the target format. While the most important aspect may generally be the immersive audio capture implementation and its current configuration for the immersive voice call, for best experience, both aspects should be considered.

This goal can be achieved in a simple way, e.g., with following parameters:

| Parameter | Summary | Notes |
| --- | --- | --- |
| lvl | IVAS level - Indicates UE IVAS level and associated capability - Indicates encoder maximum bitrate - Indicates decoder maximum bitrate with immersive rendering | Shall be present |
| cf | Coded format - Specifies IVAS immersive mode to be used in the session - Bidirectional | Either cf or cf-send shall be present |
| cf-send | Coded format, send direction - Specifies IVAS immersive mode to be used in the send direction based on the (capture) capabilities of the sender | Either cf or cf-send shall be present |
| cf-recv | Coded format, receive direction - Specifies IVAS immersive mode to be used in the receive direction based on (decoding/rendering) preferences or the receiver | Optionally present |
| tf | Target format - Indicates target format for receive side defining any limit regarding level of immersion - For example, indicates use of (head-tracked headphones for) binaural rendering - Values: - MONO, - STEREO, - IMMERSIVE1 (binaural), - IMMERSIVE2 (loudspeakers) | Optionally present |

# 3. Summary

This input provides some IVAS negotiation aspects for discussion and for further consideration to implement SDP offer-answer for IVAS.

Given the wide range of implementations and device capabilities, many aspects on send and receive side affect the immersive experience. While the send side generally knows best how to achieve highest possible quality of experience for the sound scene being captured, knowing the targeted reproduction scenario at the receiver provides the necessary information for both fine tuning of capture and format selection.

# References

[1]	S4-240666, On IVAS Levels, Nokia

[2]	S4-240697, On IVAS complexity levels, Fraunhofer IIS

[3] 	S4-240686, [IVAS] On IVAS Operating Mode Negotiation, Huawei

[4]	S4-240981, [IVAS] Proposal for IVAS service options and operating mode negotiation, Huawei

[5]	S4-241024, Definition of IVAS complexity levels, Fraunhofer IIS

[6]	S4-241071, On IVAS Levels, Nokia

[7]	S4-241940, Proposal on IVAS levels, Dolby Sweden AB, Fraunhofer IIS, Nokia