3GPP TSG S4 Meeting #130	S4-242047

# Orlando, FL, USA 18-22 November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | 0017 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AMD] WT14: pCR to update text on IETF SCONE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD |  |  |  |  |  | Date: |  |  | 2024-11-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | 19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CR0017 r1 was endorsed in SA4#129-e as basis for future work for WT14 on Inband Signaling of QoS for 5G Media Streaming. During MBS SWG AH calls the CR0017 r2 was noted. This pCR updates the existing information in thecurrently endorsed CR0017 r1 on SCONE-PRO BoF sessions. SCONE-PRO BoF sessions further evolved and were approved by IESG into as a new IETF SCONE WG in the Web and Internet Transport with a slightly simplified and clearer charter. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added section on new IETF SCONE WG |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current text on SCONE-PRO does not capture the IETF SCONE WG creation and its charter objectives. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 2	References

[SCONE-PRO]	Secure Communication of Network Properties (sconepro), https://datatracker.ietf.org/group/sconepro/about/.

[SCONE]	Standard Communication with Network Elements (scone), https://datatracker.ietf.org/wg/scone/about/

# * * * * Next change * * * *

#### 5.X.1.3	Standard Communication with Network Elements (SCONE)

SCONE-PRO BoF sessions led to the creation of IETF SCONE WG in Web and Internet Transport with the first WG meeting taking place in November 2024 at IETF 121. SCONE WG charter maintains some of the objectives of the SCONE-PRO BoF sessions in order to support rate-adaptive applications deliver optimal user experience based on available network conditions for a given network UDP 4-tuple. In particular, as per SCONE WG charter, the following objectives are in scope:

- establish a mechanism for network elements capable of rate-limiting a UDP 4-tuple to communicate an upper bound on achievable bitrate, termed "throughput advice", to the sender of packets matching the UDP 4-tuple.

- allow an application through the mechanism to receive notifications containing throughput advice for both upstream and downstream traffic from any network elements capable of dropping or delaying packets on the path of a UDP 4-tuple

- enable the throughput advice as a guideline to enhance user experience given maximum bitrate manageable by a single network element for that user's current connection. The throughput advice is not a strict indicator of network congestion as is intended for adaptive bitrate applications and is not a replacement for congestion control algorithms.

- enable potential dynamic updates to the throughput advice by the network elements

- determine whether it is necessary for an endpoint to explicitly signal its capability of receiving throughput advice, and whether it is necessary for an endpoint to confirm its receipt of throughput advice.

The SCONE WG will focus initially on a solution for QUIC transport with a milestone to submit a standard track protocol to communicate "throughput advice"— from network elements to the endpoint to the IESG for publication within November 2025 timeline.

# * * * * End of changes * * * *