Source:	Bytedance

Title:	[ATIAS ph2] Subjective Test Method for Immersive Audio Capturing with Moving Sound Sources

Document for:	Discussion and Agreement

Agenda Item:	7.8

# 1 Introduction

In previous specs, subjective test methods for immersive audio codec and renderer have been proposed [1], which defined various aspects such as selection of listening subjects, listening environment, test procedures, and lots more. However, there’s no specification that focus on audio fidelity of moving sound sources. This is reasonable considering the fact that most immersive audio renderor these days uses interpolation to mimic a sound field with moving sound sources, which are just cross-faded static sound fields.

However, instead of rendered sound fields, when we try to evaluate a sound field that’s recorded from real world, the quality we get when recording a static sound field doesn’t always translate to the quality when recording a sound field with moving sources, due to possible errors caused by immersive audio recording algorithms, including beamforming and least-square-based optimization methods.

In previous specs, objective test methods were proposed for spatial audio capturing, including tests for user capture and spatial capture. In user capture tests, sound source is a loudspeaker placed at mouth of a dummy (HATS); In spatial capture tests, sound source is the same configured dummy placed around the tested UE.

Currently sound sources are placed at following locations relative to UE [2]:

| UE Type | Self-capture test | Spatial capture test |
| --- | --- | --- |
| Headset UE | Artificial mouth of HATS | 0-degree azimuth and elevation 1m relative to the reference point of the HATS |
| Handheld hands-free UE | At artificial mouth of HATS, 42 cm from the center point of the visual display of the UE | 0-degree azimuth and elevation 1m relative to the reference point of the HATS |
| Table-mounted UE | At artificial mouth of HATS, 40 cm between UE and lip reference plane and a height of 40 cm | 0° azimuth, 26.6° elevation and 89.4 cm relative to the reference point of the HATS |

Table 1. Existing sound source positions in capture tests of UEs

And in tests for directional information, several additional source positions are selected for spatial capture tests, including [2]:

| i | i | i |
| --- | --- | --- |
| 1 | -90 | 0 |
| 2 | -60 | 0 |
| 3 | -30 | 0 |
| 4 | 0 | 0 |
| 5 | 30 | 0 |
| 6 | 60 | 0 |
| 7 | 90 | 0 |

Table 2: Additional source directions

However, all source locations are fixed.

From the information we gathered, it turns that we haven’t got a way to evaluate capturing fidelity when sound field contains moving sound sources, which arguably is the majority case in real life.

# 2 Discussions

In order to evaluate sound field capturing capability with moving sound sources, we propose the following subjective test method:

## 2.1 Test Material

Comply to clause 5.4 in 3GPP TS 26.259 [1].

## 2.2 Test Signal Recording

The method to produce the test candidate, the reference signal, and the anchor signal can be described as follows:

- Record sound field with candidate UE in an environment with following requirements:

  - The recording environment is an anechoic chamber that meets clause 4.0.3 of 3GPP TS 26.260 [2]

  - Have a loudspeaker array that meets all criteria described in clause 4.1.1.4.1 of 3GPP TS 26.260 [2], with an extra requirement that there should be at least one semiarch loudspeaker array can be placed horizontally, with its geometric center placed on the reference point of candidate UEs.

  - Use VBAP [3] to pan a virtual sound source on the loudspeaker arch, with the following trajectory:

    - Start from one end of the arch.

    - On the arch, move towards the other end, with constant angular velocity of 60 degrees per second.

    - Keep the same angular speed, move virtual source back to the starting point.

    - Repeat the above trajectory once.

- Produce reference sound field: A sound object that follows the moving trajectory used to produce the candidate signal.

- Anchors: Same test material, duration, and elevation angle as above, but with a varying and randomized azimuth angle.

## 2.3 Experiment Setup

### 2.3.1 Subject Selection

Comply with clause 5.3 in 3GPP TS 26.259 [1].

### 2.3.2 Listening Environment

Comply with clause 5.6 in 3GPP TS 26.259 [1].

### 2.3.3 Listening System

Comply to clause 5.7 in 3GPP TS 26.259 [1].

### 2.3.4 Listening Level

Comply to clause 5.8 in 3GPP TS 26.259 [1].

### 2.3.5 Questionnaire Design

For each trial, subjects will be told about the ground truth trajectory described above. Subjects will then be asked to listen to the reference, the hidden reference, the anonymous candidate signals, and the hidden anchors, all processed by the same listening material. Subjects are allowed to switch between sound tracks anytime they want.

For each track in each trial, subjects will be asked to rate 2 scores:

- The smoothness of source trajectory comparing to reference, range from 0~5. The definition for scores can be described as follows:

| Score | Definition |
| --- | --- |
| 5 | Source trajectory is very smooth. |
| 4 | Source trajectory is quite smooth, with some occasional glitches |
| 3 | Source trajectory is moderately smooth, with noticeable fluctuations. |
| 2 | Source trajectory is unstable, with frequent jumps and irregular changes of the source position. |
| 1 | Source trajectory is very erratic, with high variability in angular speed and direction. |
| 0 | Source jumps around, no clear trajectory can be perceived. |

Table 3. Score definitions (Trajectory Smoothness)

- The similarity of source trajectory comparing to reference, range from 0~5. The definition for scores can be described as follows:

| Score | Definition |
| --- | --- |
| 5 | Source trajectory is identical to reference. |
| 4 | Source trajectory is very close to reference, with minor deviations that do not affect overall accuracy. |
| 3 | Source trajectory is moderately accurate, with several noticeable deviations from the reference. |
| 2 | Source trajectory shows significant inaccuracies, with many deviations leading to confusion. |
| 1 | Source trajectory is largely inaccurate, with few similarities to the reference trajectory. |
| 0 | Source trajectory is completely different from reference. |

Table 4. Score definitions (Trajectory accuracy)

## 2.4 Data Post Processing

- For scores of each trial, we normalize the range to 0~5 by considering the min and max scores of the trial:

St=St-mintStmaxtSt-mintSt

This eliminates different tendency of people’s rate (for example, one subject may tend to rate all scores higher than 3, and another subject might not have this tendency)

- For each subject, we remove ones that once rates anchor higher than reference.

# 3 Proposal

We kindly ask the SWG to accept our proposed subjective test method as part of immersive audio capture test.

# References

[1] TS 26.259, Subjective test methodologies for the evaluation of immersive audio systems

[2] TS 26.260, Objective test methodologies for the evaluation of immersive audio systems.

[3] Pulkki, V. (1997). Virtual Sound Source Positioning Using Vector Base Amplitude Panning. Journal of The Audio Engineering Society.