3GPP TSG-SA WG4 Meeting #130	S4-242227

# USA, Orlando, 18 – 22 November 2024

Source:	Nokia, Philips, Interdigital, Deutsche Telekom, Fraunhofer HHI, KDDI, Samsung, Sony, Huawei

Title:	Pseudo-CR on Streaming of professionally produced Volumetric Video with single asset containing people – Scenario section 7

Spec:	3GPP TR <26.956 version 0.1.1>

Agenda item:	9.8

Document for:	Decision

1. Introduction

This pCR adds elements for the scenario “Streaming of professionally produced Volumetric Video with single asset containing people ” to the scenario clause 7.X in the TR.

2. Reason for Change

PD 0.0.4 (S4-241714) includes 5 scenarios including scenario “Streaming of professionally produced Volumetric Video with single asset containing people ”, which relies on dense point clouds as representation format. TR 0.1.0 includes already dense point clouds as a representation format and therefore it is proposed to include the corresponding scenario. This document addresses metrics in section 7 of the scenario template.

3. Conclusions

Without the scenario description the motivation of the dense point cloud representation format may not be well understood, which makes the FS_Beyond2D study less comprehensive.

4. Proposal

It is proposed to agree the following changes to 3GPP TS <26.956  version 0.1.1>.

* * * First Change * * * *

<Proposed change in revision marks>

## 7.x	Scenario x: Streaming of professionally produced Volumetric Video with single asset containing people

### 7.x.1	Scenario name

Streaming of professionally produced Volumetric Video with single asset containing people.

### 7.x.7	Performance Metrics and Requirements

- A clear definition on how the performance needs to be evaluated including metrics, etc addressing the main KPIs of the scenario.

- Objective measures such as PSNR, VMAF, etc, may be used.

- Justification on whether objective metrics are sufficient and representative of the subjective performance.

#### 7.x.7.1 Anchors

There is no specification in 3GPP that references a volumetric video anchor codec suitable for the streaming single asset scenario. MPEG V-PCC [R1] is the first codec supporting the dense dynamic point cloud representation format with inter coding and therefore no anchor codec can be selected.

#### 7.x.7.2 Objective tests

Objective tests for dense dynamic point cloud codecs follow the principles as defined in TR 26.955, besides that there is no anchor:

Figure fx Test architecture for objective test

MPEG WG7 specified an objective metric that allows to characterize point cloud codecs. This objective metric is described in in annex B of the Call for Proposals for Point Cloud Compression [R2]. MPEG WG7 used this “point-based” metric to develop codecs for point cloud compression. The “point-based” metric operates in 3D space and provides information on geometry and color distortion.

From 2D video objective testing it is known that a single objective metric is limiting, therefore “PCQM” [R3] is added as a second objective metric in addition to the “point-based” metric.

Both objective metrics, the “point-based” metric and PCQM will be reported for all rate points and for all reference test sequences.

To compute metrics, a patched version of mpeg-pcc-mmetric is provided for internal usage [R4]. The software will be provided and derived from the branch “bugfixes_gcc8.5_eqTFAN” with version v1.1.6. The metric software implements the “point-based” metric and the PCQM metric as described in section 7 of this scenario. A patch to enable computation of the Point2Plane metric is needed on top of this version. The metric software with patch will be provided.

Five rate points that cover a range from low quality to high quality for codecs to be characterized will be selected.

A spreadsheet will be provided to 3GPP SA4 that can be used to visualize the objective results.

Note that objective results obtained with the selected metrics can only be used for comparison with other point cloud codecs. There is no known objective metric that allows to compare different volumetric video codecs (e.g. dense point cloud against mesh) in a fair manner.

#### 7.x.7.3 Subjective tests

Persons or objects in the “Streaming of professionally produced Volumetric Video with single asset containing people” scenario can be viewed from any angle and distances. To cope with this free viewpoint, MPEG WG7 developed a special procedure how to subjectively evaluate such volumetric sequences: The selected reference sequences are rendered with a point cloud renderer by following a pre-defined camera path into a 2D video. The procedure is done with the source reference sequence and with the codec/decoded reference sequence. Both 2D videos can then be evaluated with well-known 2D video evaluation methods.

The software tool that implements rendering of 2D videos from dense dynamic point clouds by following a camera path will be provided to 3GPP SA4 for internal usage.

This representative renderer [R5] used is derived from the mpeg-pcc-renderer where a new rendering mode has been added named “splat blend” mode. This mode draws a camera facing semitransparent splat of radius PointSize centered on the point position. The transparency (or alpha) varies with the distance from the center from fully opaque at the center to nearly transparent at the edge, to provide blending between points and reduce aliasing. In order to correctly use alpha blending, the points are sorted relative to the camera plane. Two blending modes are available, gaussian and linear, and the alpha falloff speed is customizable using the "pointFocus" setting.

This representative renderer supports two methods how to render voxels, one method is like cubes of a fixed size, the second method is splat blend. For objective compression performance tests against the source reference, the first method is recommended. For tests of the dense dynamic point cloud format against other volumetric representation formats such as mesh, the splat blend rendering is recommended as it fills holes between voxels. The representative renderer allows to activate a 3D background model or a neutral background. It also allows to activate a floor so that the test sequences look grounded. The Point Cloud Renderer will be provided to 3GPP SA4 for internal usage.

A characteristic camera path for each volumetric test sequence will be pre-defined to get a length of the output video as close as possible to 10s. A camera path that includes a full person view and a closer view at typical distance to the face is a good example.

The representative renderer produces uncompressed RGB 2D videos that can then be compressed with high bitrate HEVC. Resolution of the videos is 1080p, the frame rate and color space is aligned with the input point cloud.

The produced compressed 2D videos will be made available to SA4 members so that they can judge the quality and make up their mind on a codec to be characterized.

Videos rendered with splat blending can be used to compare sequences dense dynamic point format sequences with sequences in other volumetric video formats such as dynamic mesh.

There is no plan to engage an independent subjective test lab.

#### 7.x.7.4 Correlation between the objective and subjective metrics

The objective metrics described in section 7.x.7.2 and the subjective comparison using the representative renderer in cube mode described in section 7.x.7.3 are used to evaluate coding distortions.

From MPEG there is no information available on the correlation between objective and subjective results and this may be investigated on a best effort basis.

A potential way of obtaining such correlation is described: the 4 licensed test sequences used in the MPEG subjective verification test report for V-PCC [R6] can be obtained from the rights holders, the test sequences can be prepared as used in the verification test and objective metrics described in section 7.x.7.2 can be generated. The objective results can then be compared with the subjective results from the verification test and the correlation can be analysed.

#### 7.x.7.5 Verification and crosscheck

All produced bitstreams, metric results and produced videos shall be crosschecked by at least one other 3GPP SA4 member to ensure that results are correct.

* * * Next Change * * * *

<Proposed change in revision marks>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

[R1] ISO/IEC 23090-5 Visual Volumetric Video-based Coding (V3C) and Video-based Point Cloud Compression (V-PCC) – 2nd edition

[R2] MPEG WG7, Call for Proposals for Point Cloud Compression V2, https://mpeg.chiariglione.org/standards/mpeg-i/point-cloud-compression/call-proposals-point-cloud-compression-v2.html

[R3] PCQM: A Full-Reference Quality Metric for Colored 3D Point Clouds, Gabriel Meynet, Yana Nehmé, Julie Digne, Guillaume Lavoué, https://hal.science/hal-02529668/document

[R4] Patched mpeg-pcc-mmetric, TBD

[R5] Representative renderer, TBD

[R6] MPEG, Subjective verification test report for V-PCC, https://www.mpeg.org/wp-content/uploads/mpeg_meetings/136_OnLine/w20992.zip

* * * Next Change * * * *

<Proposed change in revision marks>

* * * End of Changes * * * *