3GPP TSG-SA WG4 Meeting #131-bis-e			S4-250463

Online, 11 – 17 April 2025

Source:	Vodafone

Title:	[AvCall-MED] Avatar call use case and call flows

Agenda item:	10.7

Document for:	INFORMATION/APPROVAL/DISCUSSION

1. Introduction

Based on Avatar call assistance use case introduced in this change, some enhancements and requirements been suggested for IMS network and potential call flow for identification and mapping to network identifiers.

2. Discussion

A potential use case for avatar calls alongside:

- A potential call flow for the Identification and mapping to network identifiers has been proposed.

- potential enhancements and architectural changes for the IMS network to support Avatar Communication being proposed.

3. Proposal

It is proposed to send LS to SA2/SA3 for the identification and new interface for signaling transaction between XRAS and BAR.

Add use case and high-level architecture of the IMS network to support avatar call to the work item.

updating proposed diagrams in TS 26.264

* * * First Change * * * *

## Avatar Communication USE Case

Avatar Assisted call that can be described as below:

User 2 calls user 1 on phone, (both parties’ mobile devices are supporting IMS DC and its sub features DCMTIS and ARMTSI) User 1 gets call but not in a situation to answer the call so chooses her/his Avatar assistance on answering options on phone.

User 1’s Avatar representer for call’s is loading and communication starts between User 2 and User 1’s Avatar.

Avatar is pre generated and saved in BAR with Avatar ID mapped to his mobile identifiers. user can have more than one avatar stored and mapped to his mobile each have Avatar ID and part of Avatar List.

Avatar list is loaded to selection panel on dialler as per Data channel concept.

Avatar speaks to user 2 as the user 1 is not available to answer personally, so user2 can let Avatar know what’s the reason for call and its urgency, and if User 2 requests for a call back.

User 2 answers the reason for call and may requests for call back in a certain date and time.

Avatar checks User 1’s calendar and if it’s free, confirms the date and time. Adding call back to User 1 in the Calendar.

And sends brief description of call reason and call back request to User 1’s inbox or SMS.

Figure x.x.x-x : avatar call assistance use case call flow

* * * Second Change * * * *

## Mapping to IMS-based Services for Avatar Call

Figure x.x : High level UE and Network Architecture to support network-based avatar communication

The transition between Avatar communication and MMTel session using audio/video is based on the media negotiation between UEs, or between UE and network, described in section in this section mapping to IMS-based Services.

NOTE 1:	The authentication and authorization of avatar will be decided by SA WG3.

When a user needs to use an avatar based on the use case, the Avatar ID list will be downloaded to the UE through bootstrap data channel via DCSF.

DCSF can verify whether UE is allowed to use an avatar representation.

NOTE 2: The UE can have a default avatar and several options to select any other saved avatar from Avatar list on their device screen, depending on the use case.

The DCSF can retrieve the Avatar ID list from DC AS or UDM/UDR repository data as a part of subscription data in UDM/UDR.

Base Avatar Repository (BAR):

- The Base Avatar Repository is used to store and retrieve the avatar models. BAR can be inside the PLMN, e.g. a new network function, or outside the PLMN, e.g. a webserver of the 3rd party provider.

XR Application Server (XR AS):

- The XR Application Server is responsible for service control related to avatar communication, including avatar communication session media control and providing list or shortcuts of avatar on the data channel options on dialler, and so on.

MF/MRF:

- Support media processing, e.g. ASR/TTS/SLR/SSL, under the service control of XR AS.

- Support avatar animation and rendering, under the service control of XR AS.

DCSF:

- Provide avatar ID list in the bootstrap data channel to UE

- communicates to UDM/UDR to retrieve SUPI/GPSI/PEI of the user

BAR (Base Avatar Repository):

- BAR is used to assign Avatar ID per request from UE.

- BAR is used for storing and retrieving the avatar model from 5GC.

NOTE 3: Avatar-id stored in BAR is assigned to each Avatar and user identifier.

Components in the UE:

- IMS Aware Avatar application: It can be embedded in the UE or it can be downloaded from the AR/XR/DC AS. It is used to create the avatar.

IMS stack includes XRMTSI: XRMTSI support within the chipset which have IMS stack, will enable device communication with data channel supported IMS network. SRC and SRS are split rendering components on client and device, and they can be used solo or combined based on use case and device capabilities.

Figure x.x.x-x : IMS Network architecture to support network-based avatar communication

### Reference points

The following reference point is added to support avatar communication service in IMS:

- MC1: Reference point between the DCSF and the MF/MRF.

- DC6: Reference point between XR AS and BAR.

The MC1 reference point is used by DCSF to instruct MF the specific media capabilities will be used in a avatar communication.

The DC6 Reference point is used by BAR and XRAS to communicate Avatar ID

* * * Third Change * * * *

## Identification of Avatar and Mapping to Network Identifiers

In Release 19, avatar communication is supported through the data channel architecture. This can be over IMS Data channel, with mapping to IMS and Non-IMS based services.

The creation and rendering of an Avatar Representative can occur within the UE or the network, either partially or completely, depending on the capabilities of the UE and the network. When using network-based rendering, the DC AS sends the Avatar communication service control instruction to the MF. The MF supports media transcoding, including ASR, TTS, SLR, and SSL.

SA2 (23700-77 NG-RTC phase2) introduced A new Base Avatar Repository (BAR) which stores Avatar representation and related avatar ID to be used in identification and authentication. Avatar IDs would be assigned to registered Avatars by BAR and mapped to SUPI or GPSI, Avatar ID should be stored for Avatar representative (UE) within UDM/UDR and DC AS as part of UE subscription data.

When a user logs in to create an Avatar, authentication occurs using the Application ID and SUPI/GPSI/PEI in the Avatar creation application or client towards the network.

NOTE 4: User application is an AF to the network and authentication would take place as 3rd party authentication to the NEF, using eg. TLS authentication.

Figure 1: Avatar application authentication

NOTE 5: This application may belong to an MNO network, a third party, or an AR/XR supporting device application, which must support authentication and registration with the BAR/UDM/UDR repository.

The Avatar representation can be animated using audio/video or metadata (such as action and facial expression data) by UE or network. The metadata can be generated by UE or the network.

When generated avatar is being submitted to network, during authentication with BAR/UDM/UDR. BAR assigns Avatar ID to the requested avatar and stores to BAR in Avatar profile which contains network identity of subscriber. And updates subscriber info in UDM/UDR) and IMS AS.

Figure 2. identification on Avatar

When an avatar is generated in app it’ll be submitted to Network to be saved in BAR assigned to the UE’s profile.

- The Avatar Registration request is sent from UE and application repository to the IMS AS .

- The IMS AS sends the User Info request to DCSF.  DCSF queries the subscription info from UDM/UDR. And response includes UE’s SUPI/GPSI/PEI and avatar ID list.

- The DCSF, after receiving information from UDR/UDM, sends Avatar Registration request to BAR through XR AS, containing the UE subscription information and Avatar ID list to get new ID from BAR.

NOTE 6: A new interface for signaling (DC6) will be defined for this communication between the XR (DC/AR) AS and BAR.

- BAR checks UE profile for Avatar ID list, generates the new ID and sends it back to DCSF.

- DCSF provides the Avatar ID in response to IMS AS registration request

- The IMS AS updates Avatar ID list in UDR/UDM and XR (DC/AR) AS. The communication towards XR (DC/AR) AS is executed via DC1 and DC4 interfaces. DC4 shall contain GPSI information to identify the UE.

- IMS AS confirms the ID to BAR and sends the new Avatar ID to the UE.

- The UE stores the new avatar in BAR with the corresponding Avatar ID.