3GPP TSG-SA4 Meeting #131-bis-e	S4-250475

# Online, , 11th Apr 2025 - 17th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.522 | CR | 0013 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SDP Signaling to support PDU Set Marking with FEC awareness |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_RTP_Ph2 |  |  |  |  |  | Date: |  |  | 2025-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - One of the objectives of 5G RTP phase to is to provide better support for AL-FEC awareness for PDU Set handling. It is formalated as: “Establish support for PDU Set handling with AL-FEC awareness and specify any necessary (S)RTP header extension and signalling enhancements for PDU Set marking with AL-FEC awareness. 26.510 already supports integrated pdu set handling in dynamic policy provioning API using PduSetHandlingInfo defined in 29.571 5.5.4.11 (PDUSetQoSPara used in UnidirectionalQosSpecification in QosRange which is used in the PolicyTemplate in 26.510 ). However, there is not signaling to the AF to indicate the need for PDU Set integrated handling. ” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add SDP signaling support for PDU set ingegrated info for cases when AL-FEC is used, and not all PDU’s are needed for sucessful reception |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Objectives not met |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.5, 4.2.X (new clause in 4.2) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### ** CHANGE 1 **

### 4.2.5	SDP Signaling

An RTP sender capable of sending RTP HE for PDU Set marking shall use the SDP extmap attribute for RTP HE for PDU Set marking in the media description of the RTP stream(s) carrying the RTP HE for PDU Set marking. An RTP receiver that does not support RTP HE for PDU Set marking can ignore that RTP HE when included. The signaling of the PDU Set marking RTP HE shall follow the SDP signaling design and the syntax and semantics of the "extmap" attribute as outlined in RFC8285.The URN for the PDU Set marking shall be set to "urn:3gpp:pdu-set-marking:rel-18".

The ABNF syntax for the extmap attribute for the signaling of RTP HE for PDU Set marking is defined as follows, extending the ABNF in RFC 8285:

extensionname = "urn:3gpp:pdu-set-marking:rel-18"

extensionattributes = [pdu-set-attribute *2(SP pdu-set-attribute)]

pdu-set-attribute = format / "pdu-set-size" / "num-pdus-in-pdu-set" / “pdu-set-is-error-resillient”

format = "short" / "long"

The extension attributes have the following semantics:

-	format: indicates if the RTP HE for PDU Set marking uses the 1-byte (short) or the 2-byte (long) format. This extension attribute shall not be included more than once.

NOTE:	Regardless if this extension attribute is present or not, the use of long or short format is determined as described by section 4.1.2 of RFC 8285, i.e., based on what format other RTP HEs use in the same RTP session, unless both endpoints announced support for handling mixed format with "a=extmap-allow-mixed" as described by section 6 of RFC 8285.

-	pdu-set-size: if present, this extension attribute indicates that the RTP sender will provide the PDU Set size in bytes in the RTP HE with every RTP packet. This results in an additional 3 bytes of length for the RTP HE. This extension attribute shall not be included more than once.

-	num-pdus-in-pdu-set: if present, this extension attribute indicates that the RTP sender will provide the number of PDUs in the PDU Set (NPDS) in the RTP HE with every RTP packet. This results in an additional 2 bytes of length for the RTP HE. This extension attribute shall not be included more than once

-	pdu-set-is-error-resillient: if this attribute is present, it implies that packets in the marked PDU Sets are error resilient and not all packets are needed for successful reception of the PDU Set. When not set it is assumed that all PDU’s are needed for successful reception.

Below is an example:

a=extmap:7 urn:3gpp:pdu-set-marking:rel-18 short pdu-set-size

### ** CHANGE 2** (new clause) **

### 4.2.X	Awareness for FEC Based Handling

In case the RTP session uses forward error correction for the transmission and PDU sets include both the repair and source packets, it may be that the resulting PDU Sets are tolerant to losses. In this case it can beneficial to signal this explicitly to support PDU Set Integrated Handling. The SDP signaling is defined in clause 4.2.5 using the pdu-set-is-error-resillient attribute.

** END OF CHANGES **