3GPP TSG-S4 Meeting   #131-bis-e	S4-250503

# Electronic Meeting, 11th–17th April 2025	Revision of S4-241753

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.510 | CR | 0008 | rev | 2 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [AMD_PRO-MED] Stage-3 Aspects of Network Slicing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics Co. Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AMD_PRO-MED |  |  |  |  |  | Date: |  |  | 2025-04-05 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | A contribution S4-241753 was prepared and looked at during SA4-129e meeting that proposed to adopt the conclusions and recommendations after FS_MS_NS_Ph2 study. This contribution was endorsed with the decision that this will be adopted into Rel-19 as part of the AMD-PRO-MED WI. Since we now have the Rel-19 WI for stage-3, bringing back the above proposal. In addition, a statement in clause 5.2 is added to introduce the new feature to clarify the impact of these changes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Bullet point 2 of clause 8 of TR 26941 (conclusions and recommendations) proposes the following stage-3 change: “2.The changes to the PolicyTemplate resource data model definition described in clause 6.1.2.1 be implemented in TS 26.510 [42] to support Policy Template provisioning for a plurality of Network Slices and/or Data Networks..”. Updating the PolicyTemplate resource data model as per above agreement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature for network slicing will be incomplete. |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.7, 8.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-241753: Changes to PolicyTemplate data model S4-250503: Extending S4-241753 with an introduction statement about policy provisioning for a plurality of network slices and/or data networks |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.2.7	Dynamic Policy provisioning

#### 5.2.7.1	General

These operations are used by the Media Application Provider to configure Policy Templates for the media delivery sessions of a particular Provisioning Session.

A Policy Template, identified by its policyTemplateId, represents a set of PCF/NEF API parameters which defines the service quality and/or associated charging for the corresponding media delivery session(s). The Policy Template is configured as part of the provisioning procedures with the Media AF using the API specified in clause 8.7 and is subsequently instantiated by a Media Session Handler or Media AS (whichever is acting as Dynamic Policy invoker) using the interactions specified in clause 5.3.3.

When a Policy Template requires media to be delivered in a specific Data Network and/or network slice at reference point M4, the applicationSessionContext array shall be present with at least one of the following properties populated:

-	The dnn property contains the name of the Data Network in which the Media AS is hosted.

-	When Network Slicing is used, the sliceInfo property contains information about the network slice which is serving the UE.

A plurality of Network Slice and/or Data Network tuples maybe provisioned at M1 if Policy Template allows media to be delivered in multiple Data Networks and/or network slices at reference point M4.

When a Policy Template is intended to influence the network QoS of Service Data Flows used for media delivery, the qoSSpecifications array shall be populated with objects of type QosRange (see clause 7.3.3.4). Each member of the array describes the QoS limits of an application service component that a Media Client is permitted request when instantiating the Policy Template:

-	The componentReference property is a string used by the Dynamic Policy invoker to reference this QosRange when instantiating the Policy Template. It shall be unique for all members of the same qoSSpecifications array.

-	The qosReference value, as specified in clause 5.6.2.7 of TS 29.514 [18], is obtained with the Service Level Agreement. See TS 23.502 [3] for detailed usage.

-	The maximumBitRate properties of the downlinkQosSpecification and uplinkQosSpecification objects define the maximal bit rates which are permitted to be requested by a Dynamic Policy invoker on (respectively) downlink and uplink Service Data Flows. These values are defined by configuration of the 5G System and are therefore populated by the Media AF rather than by the Media Application Provider.

-	The maximumAuthorisedBitRate properties of the downlinkQosSpecification and uplinkQosSpecification objects define the maximal bit rates which a Dynamic Policy invoker is authorised to request on (respectively) downlink and uplink Service Data Flows. Higher bit rates are not authorised by the Media Application Provider when the Policy Template is instantiated.

-	The minimumPacketLossRate properties of the downlinkQosSpecification and uplinkQosSpecification objects define the minimal packet loss rates which are permitted to be requested by a Dynamic Policy invoker on (respectively) downlink and uplink Service Data Flows. Lower packet loss rates are not permitted by the Media Application Provider when the Policy Template is instantiated.

-	The pduSetQosLimits properties of the downlinkQosSpecification and uplinkQosSpecification objects define the minimal delay budget and minimal error rates for PDU Sets which are permitted to be requested by a Dynamic Policy invoker on (respectively) downlink and uplink Service Data Flows. Lower delay and error rates are not permitted by the Media Application Provider when the Policy Template is instantiated.

-	The pduSetMarking flag is used to specify whether Media Clients instantiating this Policy Template for uplink media delivery, or Media AS instances for downlink media delivery, are required to apply PDU Set marking to media transport protocol PDUs falling within the scope of a Dynamic Policy Instance based on this Policy Template.

NOTE 1:	PDU Set marking is used by the 5G System to satisfy the QoS requirements of application flows.

When a Policy Template is intended to be used for differential charging, the chargingSpecification property shall be present.

When a Policy Template is intended to be used for Background Data Transfer, the properties of a new Background Data Transfer policy are specified by the Media Application Provider in the bdtSpecification property (of type Bdt‌Policy‌Schedule).

-	The startDate and endDate indicate the time period for which the Background Data Transfer specification is valid. A Background Data Transfer specification may be removed from its parent Policy Template by the Media AF when it expires.

-	The windows property indicates the time windows over which the Background Data Transfer may occur.

-	Each such time window is characterised by a start time (startTime), a duration (duration) and the days of the week on which the time window is scheduled (daysOfWeek).

-	The numberOfUes property indicates the maximum number of UEs permitted to instantiate the Policy Template and make use of Background Data Transfers during a single time window instance.

-	The estimatedDataVolumePerUe that reflects the average data volume that each UE is expected to transfer during a single time window instance.

NOTE 2:	The product of the numberOfUes and estimatedDataVolumePerUe properties represents an estimate of the maximum data volume that may be transferred during any given time window instance.

-	The aggregate‌Uplink‌BitRate‌Limit and aggregate‌DownlinkBitRate‌Limit properties specify limits on the total aggregate bit rate of all currently instantiated Policy Templates to be enforced by the Media AF's admission control function. If omitted, the Media AF may instantiate a Policy Template with a Background Data Transfer specification regardless of additional costs that may be incurred by the Media Application Provider as a result.

HTTP responses for successful and operation-specific failure cases are specified in the following clauses. For all other failure cases, an HTTP response indicating a response code in accordance with clause 7.1.6 shall be returned to the API client. In all failure cases a message body in accordance with clause 7.1.7 shall be included in the response message.

#### 5.2.7.2	Policy Template life-cycle

The state of a Policy Template is exposed by the Media AF in the state property of the Policy Template resource and has one of the values specified in table 5.2.7.2-1.

Table 5.2.7.2-1: Policy Template states

| Policy Template state | Meaning |
| --- | --- |
| PENDING | The Policy Template is awaiting validation by the Media Delivery System, potentially because not all required parameters have yet been provided. This is the default state after Policy Template creation. |
| INVALID | One or more of the Policy Template's properties failed validation by the Media Delivery System. |
| READY | After successful validation by the Media Delivery System the Policy Template moves into this state. |
| SUSPENDED | The Media Delivery System may move a Policy Template into this state under certain conditions defined within the Service Level Agreement. |

Figure 5.2.7.2-1 below is a state diagram showing the life-cycle of a Policy Template resource in the Media AF.

Figure 5.2.7.2-1: Policy Template Resource State Diagram

Policy Templates require Media Delivery System operator verification, and a Policy Template resource that is newly created cannot be used immediately.

1.	Upon creation, a Policy Template resource shall be in the PENDING state. Once all mandatory properties are provided, the Media AF triggers validation.

2.	If the Policy Template is not deemed to be valid by the operator of the Media Delivery System, it shall move to the INVALID state, from where it can be updated to remedy the defect.

3.	Once it has been successfully validated by the Media Delivery System operator, a Policy Template resource shall take the READY state, indicating that it may be applied to media delivery sessions.

4.	If it is subsequently updated by the Media Application Provider, a Policy Template resource shall return to the PENDING state, awaiting revalidation by the operator of the Media Delivery System.

5.	Finally, a Policy Template resource may be SUSPENDED by the Media Delivery System operator, e.g., in case of a violation of the usage terms or for some other reasons, which renders it unusable. The update of any property moves the state from SUSPENDED into PENDING and triggers revalidation.

A Policy Template resource may be destroyed when it is in any of the abovementioned states.

The Media AF shall verify the status of a Policy Template resource prior to allowing a Dynamic Policy Instance to instantiate it. Only a Policy Template resource in the READY state is eligible to be instantiated in this way.

The Media AF shall indicate the current state of a Policy Template in the Policy Template resource in machine-readable form as well as indicating a human-readable reason for this state.

#### 5.2.7.3	Create Policy Template resource operation

This operation is used by the Media Application Provider to create a new Policy Template resource. The HTTP POST method shall be used for this purpose. The request URL shall be a well-known sub-resource of the Provisioning Session resource representing its Policy Templates resource collection, as specified in clause 8.7.2. The HTTP request message body shall be a Policy Template resource representation, as specified in clause 8.7.3.1.

If the Policy Template includes the bdtSpecification property, the Media AF shall attempt to create a series of new Background Data Transfer policies in the PCF using the procedure specified in clause 5.5.2. The individual Background Data Transfer policy associated with each time window shall be created prior to the start of that time window.

If the procedure is successful, the Media AF shall generate a resource identifier to uniquely identify the newly created Policy Template resource. In that case, it shall return a 201 (Created) HTTP response message and the URL of the newly created resource, including its resource identifier, shall be provided as the value of the Location HTTP header field. The response message body shall be a representation of the current state of the Policy Template resource (see clause 8.7.3.1), including any property values set by the Media AF.

If the request is acceptable but the Media AF is unable to provision the resources required by the supplied Policy Template (for example if it fails to create a new Background Data Transfer policy), the create operation shall fail with an HTTP response status code of 500 (Internal Server Error) and an error message body per clause 7.1.7. In this case, the Policy Template resource shall remain in an uncreated state in the Media AF.

This operation may be performed multiple times by a Media Application Provider to provision different Policy Template resources within the scope of a Provisioning Session. Each such resource is assigned a different Policy Template identifier by the Media AF.

The default state of a newly created Policy Template resource is PENDING. If all mandatory property values have been provided, the Policy Template resource is eligible for validation, as specified in clause 5.2.7.2.

#### 5.2.7.4	Retrieve Policy Template resource operation

This operation is used by the Media Application Provider to retrieve the current state of an existing Policy Template resource in the Media AF. The HTTP GET method shall be used for this purpose, citing the resource identifier of the target Policy Template in the request URL.

If the operation is successful, the Media AF shall return a 200 (OK) response that includes a representation of the target Policy Template resource (see clause 8.7.3.1) in the response message body.

#### 5.2.7.5	Update Policy Template resource operation

This operation is invoked by the Media Application Provider to modify the properties of an existing Policy Template resource in the Media AF. All available properties except state may be updated. The HTTP PATCH or HTTP PUT methods shall be used for this purpose. The replacement Policy Template resource representation shall be provided in the body of the HTTP request message.

Any update to the Policy Template resource shall automatically change its state back to PENDING, which makes it temporarily unusable by ongoing media delivery sessions. Accordingly, any attempt to instantiate the Policy Template (see clause 5.4.3) shall fail. If all mandatory property values have been provided by the Media Application Provider, the updated Policy Template is eligible for revalidation.

If the HTTP request is acceptable but the operation results in no change to the resource representation, a 204 (No Content) HTTP response message with an empty body should be returned.

If the operation is otherwise successful, the Media AF shall return a 200 (OK) response message that includes a representation of the target Policy Template resource in the response message body, confirming successful update.

Attempts to modify read-only properties of the target Policy Template resource, such as its state, shall be rejected by the Media AF with a 403 (Forbidden) HTTP response that includes an error message body per clause 7.1.7.

If the request is acceptable but the Media AF is unable to provision the resources required by the supplied Policy Template, the update operation shall fail with an HTTP response status code of 500 (Internal Server Error) and an error message body per clause 7.1.7. In this case, the Policy Template resource in the Media AF shall remain in the state immediately prior to the update operation.

#### 5.2.7.6	Destroy Policy Template resource operation

This operation is used by the Media Application Provider to destroy a Policy Template resource in the Media AF. The HTTP DELETE method shall be used for this purpose, citing the resource identifier of the target Policy Template in the request URL.

If the operation is successful, the Media AF shall return a 204 (No Content) response message with an empty message body.

All currently active media delivery sessions using the destroyed Policy Template shall revert to a default network QoS as a result of destroying the Policy Template resource.

Any subsequent operations citing the resource identifier of a destroyed Policy Template should result in a 410 (Gone) or else a 404 (Not Found) HTTP response message that includes an error message body per clause 7.1.7.

second change

### 8.7.3	Data model

#### 8.7.3.1	PolicyTemplate resource

Table 8.7.3.1-1: Definition of PolicyTemplate resource

| Property |  | Type | Cardinality | Usage | Description |
| --- | --- | --- | --- | --- | --- |
| policyTemplateId |  | ResourceId | 1..1 | C: RO R: RO U: RO | Resource identifier of this Policy Template assigned by the Media AF that is unique within the scope of the Provisioning Session. |
| state |  | string enum | 1..1 | C: RO R: RO U: RO | Current state of this Policy Template (see clause 5.2.7.2) exposed to the 5GMS Application Provider by the Media AF. Only a Policy Template in the READY state may be instantiated as a Dynamic Policy Instance and applied to media streaming sessions. |
| stateReason |  | Problem‌Details | 1..1 | C: RO R: RO U: RO | Additional details about the current state of this Policy Template exposed to the Media Application Provider by the Media AF. The instance sub-property shall be present and shall indicate the URL of this Policy Template resource at reference point M1. The title sub-property shall be present and shall indicate a human-readable representation of the state property specified above, e.g., "Policy Template ready for use" or "Policy Template invalid". The detail sub-property shall be present and shall indicate a human-readable status/error message. All other properties shall be omitted. |
| externalReference |  | string | 1..1 | C: RW R: RW U: RW | Additional identifier for this Policy Template, unique within the scope of its Provisioning Session, that may be cross-referenced with external metadata about a media delivery session. Example: "HD_Premium". |
| application‌Session‌Contexts |  | array(object) | 0..1 | C: RW R: RW U: RW | A set of application session contexts at reference point M4 to which this Policy Template may be applied. Each object in the array shall specify at least one property. If more than one property is specified, instantiation of the Policy Template is restricted to the conjunction of all the object's properties. |
|  | sliceInfo | Snssai | 0..1 | C: RW R: RW U: RW | A Network Slice on which this Policy Template may be instantiated. (See clause 5.4.4.2 of TS 29.571 [33].) |
|  | dnn | Dnn | 0..1 | C: RW R: RW U: RW | A Data Network on which this Policy Template may be instantiated. (See clause 7.3.2.) |
| qoSSpecifications |  | array(Qos‌Range) | 0..1 | C: RW R: RW U: RW | The network Quality of Service policy limits to be applied to the application service component(s) of media delivery sessions that instantiate this Policy Template (see NOTE and clause 7.3.3.4). Each member of the array is identified by a component reference that is unique in this array. If present, the array shall contain at least one object. |
| charging‌Specification |  | Charging‌Specification | 0..1 | C: RW R: RW U: RW | The charging policy to be applied to media delivery sessions that instantiate this Policy Template is instantiated (see NOTE and clause 7.3.3.7). |
| bdtPolicyId |  | BdtReferenceId | 0..1 | C: RW R: RO U: RW | A reference to an existing Background Data Transfer policy in the PCF (see NOTE). Mutually exclusive with bdtSpecification. |
| bdtSpecification |  | Bdt‌Policy‌Schedule | 0..1 | C: RW R: RO U: RW | The Background Data Transfer policy specification to be associated with media delivery sessions that instantiate this Policy Template (see clause 8.7.3.2). Mutually exclusive with bdtPolicyId property. |
| NOTE: Data type BdtReferenceId is specified in TS 29.122 [20]. |  |  |  |  |  |

At least one of the following properties shall be present: qosSpecification, chargingSpecification, bdtPolicyId, bdtSpecification.

#### 8.7.3.2	BdtPolicySchedule type

Table 8.7.3.2-1: Definition of BdtPolicySchedule type

| Property name |  | Type | Cardinality | Description |
| --- | --- | --- | --- | --- |
| startDate |  | Date | 0..1 | The start date of the Background Data Transfer policy. |
| endDate |  | Date | 0..1 | The last date of the Background Data Transfer policy. |
| windows |  | array(object) | 1..1 | The windows when Background Data Transfers are permitted. The array shall contain at least one window specification. |
|  | startTime | TimeOfDay | 0..1 | The starting time of a Background Data Transfer window (see NOTE). |
|  | duration | DurationMin | 0..1 | The duration of the Background Data Transfer window in minutes with the maximum value of 1339 (see NOTE). The duration may result in a window ending on the next calendar day. |
|  | daysInWeek | array(DayOfWeek) | 0..1 | The days of the week that the Background Data Transfer window is in effect. A maximum of seven occurrences can be provided. No two occurrences of array shall have the same value. If omitted, the Background Data Transfer window is applicable on all days of the week. |
|  | numberOfOccurrences | integer | 0..1 | The number of days that the Background Data Transfer window is in effect. Each daysInWeek recurrence is counted as one occurrence. The Background Data Transfer specification ends when either the endDate or the numberofOccurrences is reached, whichever sooner. |
|  | numberOfUes | integer | 0..1 | The maximum number of UEs permitted to use the Background Data Transfer policy in each occurrence of the Background Data Transfer window. Minimum value: 1. |
|  | estimatedDataVolumePerUe | UsageThreshold | 0..1 | An estimate of the data volume an average UE is expected to transfer (in both the downlink and uplink directions) when applying this Background Data Transfer policy in each occurrence of the Background Data Transfer window (see NOTE). |
|  | aggregate‌Downlink‌BitRate‌Limit | BitRate | 0..1 | A limit on the total uplink bit rate of concurrent instances of the parent Policy Template to be enforced by the Media AF. |
|  | aggregate‌Uplink‌BitRate‌Limit | BitRate | 0..1 | A limit on the total downlink bit rate of concurrent instances of the parent Policy Template to be enforced by the Media AF. |
| NOTE: Data types TimeOfDay, DurationMin, DayOfWeek and UsageThreshold are defined in TS 29.122 [20]. |  |  |  |  |

End of changes