3GPP TSG-SA4 Meeting #131-bis-e	S4-250520

Online, 11 – 17 April 2025

Source:	InterDigital Communications

Title:	[SR_IMS] Pseudo-CR on Metadata formats

Spec:	3GPP TS 26.567 v1.0.1

Agenda item:	10.5

Document for:	Agreement

- Introduction

The metadata formats defined in clause 5.4.2 of TS 26.567 has some text in brackets. This contribution proposes to remove the bracketed text on metadata format for Pose and Action.

- Proposal

We propose that the following change to be added to draft TS 26.567.

* * * 1st Change * * * *

## 5.4	Data transport

### 5.4.1	General

An SR-DCMTSI client shall support data channel media and support procedures in clause 6.2.10 of TS 26.114 [7].

An SR-DCMTSI client shall support the data transport requirements specified in clause 7 of TS 26.114 [7].

Application data channels over which meta-data is transported shall support the IMS DC requirements in TS 23.228 [2], and requirements specified in clause 5.4.2.

### 5.4.2	Metadata Formats

#### 5.4.2.1	General

SR-DCMTSI client and Media Function shall support the usage of the IMS data channel for the exchange of split rendering metadata with the MF. The data channel shall declare “3gpp-sr” as the data channel sub-protocol. The message content format depends on the type of the message. The data channel sub-protocol is defined in clause 8.3.3 of TS 26.565 [5].

#### 5.4.2.2	Pose Format

For XR services, the pose information format that is used for IMS-based split rendering shall comply with the format defined in clause 12.2 of TS 26.119 [6]. The pose information shall be carried as part of the data channel messaging mechanism. The metadata data channel message format is as defined in clause A.1.2. The message type shall be “urn:3gpp:split-rendering:v1:pose”.

#### 5.4.2.3	Action Format

The action information format that is used for IMS-based split rendering shall comply with the format defined in clause 12.3 of TS 26.119 [6]. The action information shall be carried as part of the data channel messaging mechanism. The metadata data channel message format is as defined in clause A.1.2. The message type shall be “urn:3gpp:split-rendering:v1:action”.

* * * End of Changes* * * *