3GPP TSG-SA WG4 Meeting #131-bis-e			S4-250580

Online, 11 – 17 April 2025

Source:	Nokia

Title:	On handling of IVAS formats in WebRTC and WebCodec API

Agenda item:	7.8

Document for:	Discussion and Agreement

- Introduction

The 3GPP IVAS codec is a versatile immersive audio codec which supports multiple different coding formats, such as stereo, SBA, MASA, ISM, multichannel, OMASA and OSBA. Some of these coding formats require additional metadata to be inserted to an IVAS encoder together with the audio data. For example, in MASA format, the input to the IVAS encoder comprises MASA audio data and MASA metadata. For ISM format, the input comprises ISM audio data and ISM metadata. For the combined formats, OSBA and OMASA, ISM metadata and MASA metadata (for OMASA) is inserted to the IVAS encoder, in addition to the audio data. Furthermore, many of the IVAS formats require multiple audio channels as input to the encoder.

At the decoder side, IVAS supports an external output (EXT) or passthrough mode, where the decoder output resembles the coded format that is present in the incoming bitstream. For example, an external output for an incoming encoded IVAS MASA bitstream consists of MASA audio and MASA metadata streams.

Metadata input to the audio encoder (and metadata output from the decoder) is a new feature which has not been present in earlier 3GPP audio codecs. Similarly, 3GPP audio codecs with multiple channels of audio did not exist earlier. These were limited to mono, such as 3GPP EVS, for example.

- IVAS with WebRTC and WebCodec API

WebRTC audio architecture processes audio inputs and outputs with relevant class structures [1]. However, these structures do not include a way to handle complementary metadata and there are no other structures that could be used for audio related metadata. More precisely, the WebRTC API does not provide ways to input/output metadata to/from the audio codec. Moreover, the WebRTC implementation does not contain relevant data structures for metadata handling and does not allow moving metadata between processing modules, e.g., from an audio capture (front-end) module to an encoder module. Consequently, the lack of audio metadata structures in WebRTC architecture prevents the use of some coding modes in IVAS without custom modifications to WebRTC by the developers. This applies to both input to the encoder and output from the decoder.

The WebCodec API for the encoder expects only audio data as the input [2]. Thus, metadata input for IVAS formats, such as MASA, ISM, OMASA and OSBA, is not supported. Furthermore, the decoder does not support metadata in the output.

Furthermore, the support for multiple channels of audio input is not complete in WebRTC or WebCodec API implementations. The support in different browser implementations is not consistent and not complete when compared to the available formats in IVAS.

Based on the above discussion, it is identified that the support for metadata as input to the encoder in addition to the audio data does not exist in WebRTC and WebCodec API. Similarly, the support for metadata output from the decoder is missing. Furthermore, support for multichannel audio is limited. These gaps prevent comprehensive support for all IVAS coding formats in WebRTC and WebCodec architecture and API.

- Conclusion

This input document identifies gaps relating to the support of IVAS formats, including support for metadata and multichannel audio, in WebRTC and WebCodec architecture and API. It is proposed to consider this in TR 26.858. For potential future implementations utilizing 3GPP audio codecs in WebRTC, it will be necessary to address these gaps for highest user experience and interoperability.

- References

[1] WebRTC, https://w3c.github.io/webrtc-pc/

[2] WebCodecs API, https://www.w3.org/TR/webcodecs/