3GPP TSG SA4 WG #131e		TDoc S4-250588

online, 11th – 17th April 2025

Title:	On “Inclusion of NTN intended service area in the Service Announcement”

Agenda Item:	x.y

Source:	Ericsson

# 1	Overall description

SA4 is receiving an LS from RAN2 on “Inclusion of NTN intended service area in the Service Announcement” (R2-2501583). The RAN2 LS introduces “Intended Service Areas” (ISA) and “Intended Service Area Ids” (ISA ID):

RAN2 is working on a solution to provide UEs with geographical information of where within an NTN cell an MBS broadcast service is applicable.  This area is called the “intended service area” (ISA) whose geographical definition is provided to the UE via SIB and is associated with an identifier (ISA ID).

Per “Running RRC CR for NR NTN phase 3” CR (R2-2502676), RAN2 is designing a new SIB message, which contains the mapping between an ISA ID and the Intended Service Area. The Intended Service Area can be either a Polygon (list of lat/lon points) or a circle (Centre plus radius).

This design reminds a bit on the eMBMS MBMS Service Area Id (MBMS SAI) concept, where MBMS SAIs were sent within the User Service Announcement and then used by the UE in combination with information from LTE SIB 15 to find the correct carrier.

RAN2 is asking, whether Intended Service Area (i.e. the polygon or the circle) can be provided with the User Service Announcement to UEs.

Observation 1: A Polygon description can a long list of lat/lon points (with a currently unknown value precision). Thus, the size of the ISA field is not known.

Observation 2: The ISA is planned to be transmitted via a new SIB, including the mapping between ISA ID and ISA. The usage of ISA IDs within Service Announcement might be shorter. It is not clear, why RAN2 is mentioning both, the ISA and the ISA ID in the LS and then only ask about ISA.

# Proposal

In principle, it can be ok (despite the overhead) to provide the ISA within the Service Announcement. However, sending just the ISA ID seem to be of lower overhead.

Further, any modification of the ISA shape can be handled only in RAN by updating the ISA in the new SIB, when an ISA ID is provided (instead of the ISA).

Proposal: It is proposed to ask, whether the ISA ID can be provided within the Service Announcement.