3GPP TSG-SA WG4 Meeting #131-bis-e	S4- 250602

Online, 11 – 17 April 2025

Source:	Xiaomi

Title:	Pseudo-CR on 26.265

Spec:	3GPP TS 26.265

Agenda item:	9.7

Document for:	Decision

1. Introduction

This contribution adds video operation points to support monochrome with the purpose of carrying depth and alpha maps in 26.265.

2. Reason for Change

First change introduces defines monochrome operation point to support carrage of depth and alpha maps.

Next change updates the table that lists the operation points to list the introduced monochrome OP.

Next change updates the HDTV video format, to support carriage of monochrome bitstreams.

Next change introduces a FullHDMono-Dec decoding capabilities profile to support decoding of monochrome bitstreams.

Next change introduces a FullHDMono-Enc encoding capabilities profile to support decoding of monochrome bitstreams.

4. Proposal

It is proposed to agree the following changes to 3GPP TS26.265 v1.0.0.

* * * First Change * * * *

### <ALL NEW TEXT>

### 6.3.7	3GPP HEVC Monochrome Operation Point

#### 6.3.7.1	Introduction

The HEVC Monochrome Operation Point permits consistent distribution of monochrome video using HEVC. The remainder of this clause 6.3.7 defines the Bitstream and Receiver requirements for the 3GPP-HEVC-Monochrome receiver.

#### 6.3.7.2	Bitstream Requirements

A 3GPP-HEVC-Monochrome Bitstream shall conform to the following requirements

-	the Bitstream shall be an HEVC-Progressive Bitstream as defined in clause 4.5.3.

Editor’s Note: The HEVC-Format Bitstream is not clearly defined in 4.5.3. Maybe to be added when clarified.

-	the Representation Format included in the Bitstream shall conform to the 3GPP-HDTV Representation format as defined in clause 4.4.3.2.

Editor’s Note: We are proposing extending the 3GPP-HDTV Representation format – alternatively a new format can be created instead.

-	the Bitstream shall be decodable by a decoder with HEVC-FullHDMono-Dec decoding capabilities.

Based on this, the following additional restrictions apply

-	The chroma sub-sampling shall be 4:0:0 and the value of chroma_format_idc shall be set to 0.

-	The aspect_ratio_idc value shall be set to 1, indicating a square pixel format.

[-	In the VUI, the value of colour_primaries shall be set to 128 for alpha map and 129 for depth map, the value of transfer_characteristics shall be set to 1 and the value of matrix_coeffs shall be set to 18.]

NOTE: The colour_primaries and the matrix_coeffs values proposed are currently in the CICP TUC.

-	The value of chroma_sample_loc_info_present shall be set to 0.

The timing information may be present.

-	If the timing information is present, i.e. the value of vui_timing_info_present_flag is set to 1, then the values of vui_num_units_in_tick and vui_time_scale shall be set according to the frame rates allowed for each operation point. The timing information present in the video Bitstream should be consistent with the timing information signalled at the system level.

-	The frame rate shall not change between two RAPs. fixed_frame_rate_flag value, if present, shall be set to 1.

#### 6.3.7.3	Receiver Requirements

Receivers conforming to the Operation Point 3GPP-HEVC-Monochrome shall support decoding and rendering Bitstreams with the restrictions defined in clause 6.3.7.2.

NOTE 1:	Rendering includes adherence to the parameters signalled in the bitstream to characterize the distributed Representation format.

Receivers should ignore the content of all Video Parameter Sets (VPS) NAL units as defined in Recommendation ITU-T H.265 / ISO/IEC 23008-2 [h265].

NOTE 2:	The VPS may be present to address requirements in other Operation Points, but the Bitstream also conforms to this Operation point.

There are no requirements on output timing conformance for H.265/HEVC decoding (Annex C of [6]). The Hypothetical Reference Decoder (HRD) parameters, if present, should be ignored by the Receiver.

* * * Next Change * * * *

## 6.1	Introduction

Video operation points define a restricted subset of representation signals and media capabilities. For each Video Operation Point, requirements for the Bitstream and for the Receiver are defined.

Table 6.1-1 provides an overview of defined video operation points.

Editor’s Note: We are proposing extending the 3GPP-HDTV Representation format – alternatively a new format can be created instead.

Table 6.1-1	Overview of Video Operation Points

| Name | Video Format | Decoding Capabilities | Definition |
| --- | --- | --- | --- |
| 3GPP-AVC-HD | 3GPP-HDTV (see clause 4.4.3.2) | AVC-FullHD-Dec (see clause 5.4) | 6.2.2 |
| 3GPP-HEVC-HD | 3GPP-HDTV (see clause 4.4.3.2) | HEVC-FullHD-Dec (see clause 5.4) | 6.3.2 |
| 3GPP-HEVC-HD-MONO | 3GPP-HDTV (see clause 4.4.3.2) | HEVC-FullHDMono-Dec (see clause 5.4) | 6.3.7 |
| 3GPP-HEVC-HD-HDR | 3GPP-HDR (see clause 4.4.3.3) | HEVC-FullHD-Dec (see clause 5.4) | 6.3.3 |
| 3GPP-HEVC-UHD-HDR | 3GPP-HDR (see clause 4.4.3.3) | HEVC-UHD-Dec (see clause 5.4) | 6.3.4 |
| 3GPP-HEVC-3D | 3GPP-3DTV (see clause 4.4.3.4) | HEVC-UHD-Dec-2 (see clause 5.5) | 6.3.4 |
| 3GPP-MVHEVC-3D | 3GPP-3DTV (see clause 4.4.3.4) | MVHEVC-UHD-2 (see clause 5.5) | 6.3.6 |

* * * Next Change * * * *

An informative summary of the parameters of a 3GPP HDTV format based on the parameters defined in Table 4.4.2-1 is provided in Table 4.4.3.2-1.

Editor’s Note: We are proposing extending the 3GPP-HDTV Representation format – alternatively a new format can be created instead.

Table 4.4.3.2-1	Video Signal Parameters for 3GPP HDTV format

| Parameter | Restrictions |
| --- | --- |
| Picture aspect ratio | 16:9 |
| Spatial Resolution width x height | 1920 × 1080 NOTE 1: Down-sampled resolutions may be created for distribution, for example in case of adaptive streaming. NOTE 2: To accommodate the block coding structure of a given specification, quite often the encoded signal may be padded. In such cases, normative cropping is typically applied to remove spatial samples that are not intended to be presented. |
| Scan Type | The source scan type of the pictures as defined in clause 7.3 of Rec. ITU-T H.273 is progressive. |
| Chroma format indicator | The chroma format indicator is 4:2:0 or 4:0:0. |
| Bit depth | The permitted values are 8 or 10 bit. The bit depth is the same for all samples. |
| Colour primaries | Only the value 1, 128, or 129, as defined in clause 8.2 of Rec. ITU-T H.273, are permitted. NOTE: value 1 means that the pictures contain colour information. value 128 means that the pictures contain alpha map. value 129 means that the pictures contain depth map. |
| Transfer Characteristics | Only the value 1, as defined in clause 8.2 of Rec. ITU-T H.27,3 is permitted. |
| Matrix Coefficients | Only the value 1 or 18, as defined in clause 8.2 of Rec. ITU-T H.273, are permitted. |
| Frame rates | The permitted values are 60, 60/1.001, 50, 30, 30/1.001, 25, 24, 24/1.001 fps. |
| Frame packing | No frame packing is applied. |
| Projection | No projection is used. |
| Sample aspect ratio | The pixel aspect ratio is 1 (square pixel), i.e. only the value 1 as defined in clause 7.3 of Rec. ITU-T H.273 is permitted. |
| Chroma sample location type | The location of the chroma samples relative to the luma samples for progressive frames as defined in Rec. ITU-T H.273, clause 8.7, is set to 0 (chroma samples are horizontally co-sited with and vertically centered between the first luma sample at the top-left corner and the first two luma samples at the top-left corner, respectively). |
| Range | The restricted video range is used. |

* * * Next Change * * * *

### 5.3.2	HEVC Decoding Capabilities

The following decoding capabilities are defined:

-	HEVC-HD-Dec: the capability to decode bitstreams conforming to both, HEVC/ITU-T H.265 Main Profile, Main Tier, Level 3.1 [h265] bitstreams with progressive constraints as defined in clause 4.5.3.

-	HEVC-FullHD-Dec: the capability to decode bitstreams conforming to HEVC/ITU-T H.265 Main 10 Profile, Main Tier, Level 4.1 [h265] bitstreams with progressive constraints as defined in clause 4.5.3.

-	HEVC-FullHDMono-Dec: the capability to decode bitstreams conforming to HEVC/ITU-T H.265 Main 10 Profile, Main Tier, Level 4.1 [h265] bitstreams with progressive constraints as defined in clause 4.5.3.

-	HEVC-UHD-Dec: the capability to decode bitstreams conforming to HEVC/ITU-T H.265 Main 10 Profile, Main Tier, Level 5.1 [h265] bitstreams with progressive constraints as defined in clause 4.5.3.

* * * Next Change * * * *

## 5.4	Single-Instance Encoding Capabilities

The following encoding capabilities are defined:

-	AVC-FullHD-Enc: the capability to encode a video signal to a bitstream that is decodable by a decoder that is AVC-FullHD-Dec capable as defined in clause 5.3 with the following additional constraints:

-	up to 245,760 macroblocks per second;

-	up to a frame size of 8,192 macroblocks;

-	up to 240 frames per second;

-	the chroma format being 4:2:0; and

-	the bit depth being 8 bit;

NOTE 1: 	The 3GPP HDTV format if restricted to 8 bit as defined in clause 4.4.3.2 may be encoded with an AVC-FullHD-Enc capable encoder.

-	HEVC-HD-Enc: the capability to encode a video signal with

-	up to 33,177,600 luma samples per second;

-	up to a luma picture size of 983,040 samples;

-	up to 120 frames per second;

-	the chroma format being 4:2:0; and

-	the bit depth being 8 bit;

to a bitstream that is decodable by a decoder that is HEVC-HD-Dec capable as defined in clause 5.3.

NOTE 2: 	A restricted version of the 3GPP HDTV format as defined in clause 4.4.3.2 may be encoded with an HEVC-HD-Enc capable encoder.

-	HEVC-FullHD-Enc: the capability to encode a video signal to a bitstream that is decodable by a decoder that is HEVC-FullHD-Dec capable as defined in clause 5.3 with the following additional constraints:

-	up to 133,693,440 luma samples per second;

-	up to a luma picture size of 2,228,224 samples;

-	up to 240 frames per second;

-	the chroma format being 4:2:0; and

-	the bit depth being either 8 or 10 bit;

-	HEVC-FullHDMono-Enc: the capability to encode a video signal to a bitstream that is decodable by a decoder that is HEVC-FullHDMono-Dec capable as defined in clause 5.3 with the following additional constraints:

-	up to 133,693,440 luma samples per second;

-	up to a luma picture size of 2,228,224 samples;

-	up to 240 frames per second;

-	the chroma format being 4:0:0; and

-	the bit depth being either 8 or 10 bit;

* * * End of Changes * * * *