3GPP TSG-SA WG4 Meeting #131-bis-e	S4-250621

Online, 11 – 17 April 2025

3GPP TSG-CT WG4 Meeting #128	C4-251475

# Wuhan, P.R. China; 07th – 11th April 2025

# Title:	Reply LS on request for IMS Data Channel related clarifications

# Response to:	LS on request for IMS Data Channel related clarifications (C4-251013/S3i240707) from SA3-LI

# Release:	Rel-18

# Work Item:	NG_RTC

Source:	CT4

To:	SA3-LI

Cc:	SA2, SA4, CT1

Contact Person:

Name:	Rong Wang

Tel. Number:

E-mail Address:	wangrongyjy@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	C4-251470

1. Overall Description:

CT4 thanks SA3-LI for the LS regarding the request for IMS Data Channel related clarifications.

Based on the reply LS from other WGs, CT4 would like to confirm that CT4 has the same view as SA2, SA4 and CT1 on Q1~Q4, Q5, Q5-1, Q5-3 to Q5-4. For Q5-2, CT4 prepared the following response and the corresponding CRs for TS29.175 to clarify how the calling and called identities are populated by IMS AS.

5-2:	If it is the former, what would be purpose including the Called Identity in the Session Event Notification, specially, when that user is not using the IMS Data Channel?

CT4 answers:

- For the IMS AS serving the originating user, the Session Event Notification that includes the Called Identity (redirecting party identity) is sent upon receiving the initial INVITE request. But communication forwarding and the diverted-to user identity is known by the IMS AS via the responses (e.g., 181 or 200 OK) to the initial INVITE request while the 200 OK shall trigger the Session Event Notification with SESSION_ESTABLISHMENT_SUCCESS event type. CT4 thinks it is necessary to update the called identity to the diverted-to user identity with this Session Event Notification (please check the attached agreed CR).

- For the IMS AS serving the diverting user, the Session Event Notification that includes the Called Identity (redirecting party identity) is sent upon receiving the initial INVITE request. When the communication forwarding happens, the IMS AS may send Session Event Notification with other session related EventType (e.g., SESSION_ESTABLISHMENT_FAILURE) without the sessionInfo and indicate to DCSF that the original session (session between originating user identity and the redirecting party identity) is no longer valid. In such case, CT4 believes it is not necessary to update the called identity with the Session Event Notification.

2. Actions:

To SA3-LI group.

ACTION: 	CT4 kindly asks SA3-LI to take the above answers into account.

3. Date of Next CT4 Meetings:

CT4 Meeting calendar can be found at:

https://www.3gpp.org/dynareport?code=Meetings-C4.htm