3GPP TSG-SA4 Meeting #131-bis-e	S4-250637

# Online, , 11th Apr 2025 - 17th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.114 | CR | 0584 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of a=3gpp-req-app parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Japan K.K. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_MEDIA_MTSI_ext, TEI19 |  |  |  |  |  | Date: |  |  | 2025-04-14 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The "a=3gpp-req-app" SDP attribute has an optional "app-dc-info" parameter, but it is not described what it means when it is not present, or what it means when it is present but corresponding "a=dcmap" line(s) are not present. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification of the semantics for presence and non-presence of the optional "app-dc-info" parameter to the "a=3gpp-req-app" SDP attribute, and its relation to corresponding "a=dcmap" lines. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Risk for misinterpretation that could cause interoperability problems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.13, 6.2.13.1, 6.2.13.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-250637 Initial revision (Rel-18 Cat F in S4-250636) |  |  |  |  |  |  |  |  |

**** First change ****

### 6.2.13	The a=3gpp-req-app SDP attribute

#### 6.2.13.1	General

When a DCMTSI client initiates the addition of application data channel(s) to a call with a peer DCMTSI client for different applications, the SDP offer/answer shall identify the applications and which data channels are to be created for them by signalling the identification for that application via the media level "a=3gpp-req-app" attribute added to the media lines describing application data channels for the application as discussed in section 6.2.10.1. The application identification can be included in the data channel application.

Both DCMTSI clients negotiating application data channels for a call between them shall identify the application requesting application data channel(s) to be established via the "req-app-id" parameter of an "a=3gpp-req-app" media-level SDP attribute that conveys a value set by the application launched on a UE, and configured on that application when it is made available for retreival via a bootstrap data channel. The "a=3gpp-req-app" attribute may include an "app-dc-info" parameter to explicitly describe which data channel stream ID(s) belong to the application identified by the "req-app-id" parameter. The "a=3gpp-req-app" attribute may also include an "adc-stream-id-endpoint" variant of the "app-dc-info" parameter to allow the UEs to identify the endpoints for the application data channels used for communication to a network server or to the remote UE. The combination of "req-app-id" and "app-dc-info" parameters allows the communicating UEs to bind the offers and answers for each data channel being negotiated for the identified application.

#### 6.2.13.2	3gpp-req-app ABNF syntax and semantics

3gpp-req-app-value = req-app-id [";" app-dc-info] *(";" 3gpp-req-app-opt)

req-app-id = quoted-string

app-dc-info = adc-info *[ SP adc-info]

adc-info = adc-stream-id / adc-stream-id-endpoint

adc-stream-id = 4*5DIGIT

adc-stream-id-endpoint = adc-stream-id "-" endpoint-type

endpoint-type = "UE" / "Server"

3gpp-req-app-opt = token ; must be syntactically different from app-dc-info, to avoid misinterpretation

quoted-string = DQUOTE *(quoted-char / escaped-char) DQUOTE

quoted-char = SP / quoted-visible

quoted-visible = %x21 / %x23-24 / %x26-7E ; VCHAR without " or %

escaped-char = "%" HEXDIG HEXDIG

DQUOTE = <from RFC 5234>

token = <from RFC 4566>

The "req-app-id" parameter identifies the application in a way that is sufficiently unique to avoid ambiguity in the context where it is used.

The optional "app-dc-info" parameter indicates the application data channels used by the application identified by "req-app-id" parameter and the type of the remote endpoint of every application data channel. By including "adc-stream-id-endpoint" parameter, the SDP sender indicates to the network if the corresponding data channel stream is intended to be established towards a server or the remote UE. The application is responsible for distinguishing which data channel stream is towards a server or remote UE.

A data channel stream ID that is present in an "app-dc-info" parameter but has no corresponding "a=dcmap" attribute in the same SDP media description, shall be interpreted as closed. That is, only the "a=dcmap" SDP attribute (in alignment with [172]) has any impact on the data channel status, regardless of the "app-dc-info" parameter value.

When the optional "app-dc-info" parameter is not included in the "a=3gpp-req-app" SDP attribute, the "req-app-id" parameter applies to all application data channels in the SDP media description containing the "a=3gpp-req-app" SDP attribute that are not described by the "app-dc-info" parameter of other "a=3gpp-req-app" SDP attributes for the same SDP media description, if any.

**** End of changes ****