3GPPSA4#67	Tdoc S4-120031

30 Jan - 3 Feb, 2012, Edinburgh, Great Brittan

Agenda item: 		Multicast-Broadcast-Streaming (MBS) SWG

Source: 		Broadcom

Title: 			Proposed FEC Overhead Evaluation Procedure

Document for		Proposal and Approval

# 1. Introduction

One objective of current work item EMM is to improve the reliability of streaming and download services delivery over MBMS [1]. This requires comparison of the proposed codes. This document recommends how evaluate code performance and overhead.

# 2. Overhead Evaluation Procedure for BEC

Data to be transmitted is partitioned into K symbols.  These K symbols are used to generate N total symbols to be transmitted, where N>=K.  Typically, N is set by a network operator based on the maximum amount of overhead that can be tolerated.  The N symbols are transmitted through an erasure channel with erasure probability P_e.  The erasure channel is IID and it operations on the data symbol by symbol.  This erasure channel is illustrated in Figure 1 and Figure 2.  Successful decoding requires at least K symbols to be received, but in some cases additional received symbols may be necessary.   Denote the number of symbols received in excess of K to be O.  The decoding failure probability distribution is a function of O and is given as P(O)=Pr{decoding with O overhead symbols or less fails}.

Figure 1: Illustration of the erasure channel.  Data is passed through the channel with probability   1-Pe, and erased with probability Pe.

Figure 2: Data is passed through the IID erasure channel, with erasure probability of Pe.  Data is delivered to the decoder in the order in which it was transmitted.

The transmit overhead describes the number of symbols that need to be transmitted for successful decoding.  Define the number of transmitted symbols as TX, where by necessity 0<=TX<=N.  The decoding failure probability distribution is a function of TX and is given as P(TX)=Pr{decoding with TX transmit symbols or less fails}.

- Fix K, the number of encoded symbols

- Fix N, the maximum number of symbols (systematic or repair) to be transmitted

- Use an Erasure Channel with probability of error Pe for each symbol.  [IID Pe=5%]

- Set O= -1 and TX= -1

- Set Number_of_received_symbols=0

- While (Number_of_received_symbols<K)

  - If (TX+1>N)

    - Note the case as “undecodable”

    - Goto 4

  - TX=TX+1

  - Transmit a symbol through the BEC.  If the symbol is delivered by the Erasure Channel

    - Number_of_received_symbols=Number_of_received_symbols+1

- Attempt to Decode with the received symbols

- If decoding is not successful

  - If(TX+1>N)

    - Note O and that the case was “undecodable”

    - Goto 4

  - TX=TX+1

  - Transmit a symbol through the Erasure Channel.  If the symbol is delivered by the Erasure Channel

    - O=O+1

  - Goto 7

- If decoding is successful

  - Note TX and O

  - Goto 4

- Compare code performance in range of 100% to 1% probability of failure

  - Compare P(O)

In this way, code performance is compared in a realistic manner in which there is a maximum number of possible transmit symbols.  Additionally, the decoder receives data in the order in which it is transmitted.

2.1 Test Cases

The following test cases are determined for the purpose of evaluating the code performance. These test cases evaluate the code at 15% maximum transmit overhead.

Table 1 Test Cases for Code Performance

| Number | K | N | Channel |
| --- | --- | --- | --- |
| CP1 | 32 | 37 | IID Pe=5% |
| CP2 | 256 | 295 | IID Pe=5% |
| CP3 | 1024 | 1178 | IID Pe=5% |
| CP4 | 8192 | 9421 | IID Pe=5% |

### 2.2 Performance Metrics

For each of the above test cases the following performance metrics shall be reported.

- The probability that decoding is not successful with O = 0 symbols Pf(O=0)

- The probability that decoding is not successful with O <= 1 symbols Pf(O=1)

- The probability that decoding is not successful with O <= 2 symbols Pf(O=2)

- The probability that decoding is not successful with O <= 3 symbols Pf(O=3)

- The probability that decoding is not successful with O <= 4 symbols Pf(O=4)

- The probability that decoding is not successful with O <= 5 symbols Pf(O=5)

- The probability that decoding is not successful with O <= 6 symbols Pf(O=6)

- The probability that decoding is not successful Pf(Undecodable)

Table 2 Reporting format for Code Performance

| Case | Pf(O=0) | Pf(O=1) | Pf(O=2) | Pf(O=3) | Pf(O=4) | Pf(O=5) | Pf(O=6) | Pf(Undecodable) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CP1 |  |  |  |  |  |  |  |  |
| CP2 |  |  |  |  |  |  |  |  |
| CP3 |  |  |  |  |  |  |  |  |
| CP4 |  |  |  |  |  |  |  |  |
| CP5 |  |  |  |  |  |  |  |  |
| CP6 |  |  |  |  |  |  |  |  |
| CP7 |  |  |  |  |  |  |  |  |
| CP8 |  |  |  |  |  |  |  |  |

# 3. Conclusion

This document proposes how to compare FEC performance using a BEC.

References

[1] S4-110786, “Work Item Description (Enhancements to Multimedia: PSS, MMS, and MBMS Enhancements and Performance Improvements)”