3GPP TSG-SA4 #67 Meeting	Tdoc S4-120089

30th Jan. – 3rd Feb., 2012, Edinburgh, GB

Agenda item: 		13.3.1

Source: 		ETRI

Title: 	Graceful Degradation - Forward Error Correction Code (GD-FEC) in MBMS and Evaluation Criteria

Document for		Decision

1. Introduction

There are increasing demands for video services delivered not only through wired networks but also through wireless networks in which the terminals are constantly moving, resulting in the network conditions varying as the time and location of the terminals move and change. The variation of the network conditions may also result in varying packet error rates, throughputs, and packet delays that may easily distort the media data delivery and may result in video users having bad experiences. Moreover, the terminals in broadcast services cannot be adapted individually when they enter difficult reception conditions; thus, they can experience abrupt service severances with consecutive or burst packet errors. In order to prevent these errors, a minimum level of acceptable quality should be guaranteed by providing graceful degradation that can be realized using the view scalability in the video or the differentiation of the transmission robustness in different sections of the video streams, which is referred to as unequal error protection (UEP; e.g. base layer or I frame in a GoP must be protected for minimum level of service; see Figure 1 in [1]). The graceful degradation - forward error correction code (GD-FEC) proposed in the contribution is only to recover the I-frames; when all frames in consecutive GoPs are lost, the receiver can protect display disconnections, thus enhancing the service quality and user experience by recovering each I-frame in each GoP using a AL-FEC (e.g., RaptorQ )

The redundancy for the minimum level of service in GD-FEC can be below 1%. Moreover, the amount of redundancy can be controlled. One drawback of using GD-FEC is its delay; however, this method is effective for stored video broadcasting and real time broadcasting that allows a few seconds delay.

2. Relationship between the GD-FEC and AL-FEC

Although the GD-FEC and AL-FEC increase the delivery reliability of media data, they should not performed in the media equipment being used for general purposes because their parities are needless redundancies. Instead, the GD-FEC and AL-FEC should be performed in the transmission equipment that receives the media equipment outputs such as the audio, video, dynamic and interactive multimedia scenes (DIMS), text, etc. streams (see Figure 1).

Figure 1. Media equipment, transmission equipment, and network: the GD-FEC and AL-FEC should be included in the transmission equipment. The user equipment (UE) has both media and transmission functions in the one device. BM-SC is the broadcast and multicast – service center.

However, unlike the location of the AL-FEC, that of the GD-FEC should be the first, or at least an early, step in the transmission function because it should be able to isolate the I and P frames from the media equipment output signals before they become interleaved, enciphered, or complicatedly packetized signals for the AL-FEC (see Figure 2). Instead, as Figure 2 shows, the AL-FEC block or layer located next to the transmission blocks, such as the real-time transport protocol (RTP) or secure RTP (SRTP), must receive their output signals that are already interleaved, enciphered, or complicatedly packetized by RTP or SRTP. Therefore, the input signals of the AL-FEC must be blindly grouped or symbolized from the input signal data whose contents may not be discriminated.

Figure 2. GD-FEC and AL-FEC mechanisms for the streaming delivery method interaction diagram [3]: the GD-FEC location is at the first step (or at least an earlier processing block) in the transmission equipment.

3. Graceful Degradation - Forward Error Correction Code (GD-FEC)

Figure 3 shows an example timing diagram of a GD-FEC encoding process in an MBMS system, and Figure 4 shows the GD-FEC encoding and frames comprising a GoP in an MBMS system. In the figures, the macro frame (Mfr) is the group of frames or part of a frame that contributes to generating the parity frames (Pfr) of the GD-FEC. That is, L consecutive Mfrs can be the input of the GD-FEC and the divided parity frame (DPfr) is the Pfr divided with a sufficient size to be inserted in an appropriate position in the GoP (e.g. the final GoP position); its generating mechanism will be described in detail in the next section.

Figure 3. Timing diagram of an example GD-FEC encoding process in an MBMS system.

4. GD-FEC Encoding and Numerical Values

Let’s define G as the group of GoPs that contribute to generating the parities from one GD-FEC encoding; then, the number of GoPs in G becomes L as shown in Figures 3 and 4. In order to obtain the Mfr length, the GD-FEC process compares and chooses the longest I frame in each L GoP (e.g. frame 5 in Figure 4) and designates it as the Mfr length. In order to create the Mfr, the GD-FEC process selects the I frame first; if the length is still less than the Mfr length, more bits must be attached from the other frames and the priority order of the frames is I, P, and B as shown in Figure 4.

Figure 4. GD-FEC encoding and the frames that comprise a GoP in an MBMS system.

After determining the Mfrs, the GD-FEC process formulates the input source symbols by connecting the Mfrs consecutively and performing the GD-FEC encoding and generating parities. With these parities, the GD-FEC process creates DPfrs using the following processes.

{DPfr 4,8,12,16,20} <~ F{T, Mfr i, i=1,5,9,13,17}//(D-T)

{DPfr 24,28,…    } <~ F{T, Mfr I, i=21,25,29,…}//(D-T)

…,

where F denotes a RaptorQ-like coding function, T denotes the number of GoPs or Mfrs that could be restored after several consecutive GoP losses, and D denotes the number of source GoPs as the input of the RaptorQ-like coding and is equal to L in the given example. Furthermore, the operator ‘<~’ is defined as a generating function for the DPfr that can be inserted into an appropriate position in each GoP in G and the operator ‘//’ is an operation for the DPfr generated by dividing the RaptorQ-like encoding output data with (D-T). The DPfr length can be determined using Corollary 2 as defined below. Without loss of generosity, the amount of parity bits that are more than those of the I-frame need to be restored; this concept is illustrated in Theorem 1 and Corollaries 1 and 2 as described below.

Theorem 1: Suppose that a channel capacity is B bits per channel use. Then, we can obtain a transmit rate of R=B k/(k+m)≤ C=B where k and m are the message length and parity length, respectively. Further suppose that α fraction of the message is deleted in the bearer delivery process. Then, the remaining message becomes a length of 1-αk and m>αk.

Proof: The deleted message can be understood as erased bits over a BEC. Then, the inequality R=Bk1-αk+m<C=B1-αkk+m=B1-αk+mk+m<B or inequality R'=k1-αk+m<C'=1-αkk+m=1-αk+mk+m<1 must be satisfied. Suppose m=αk. Then, R'=1, which becomes larger than C'. Therefore, this inequality is only satisfied if m>αk.

Corollary 1: According to Theorem 1, the amount of parities is larger than loss bits which are α portion of loss in k bit messages. If a good AL-FEC (e.g. RaptorQ Code) is used, the amount of parities can approach the loss bits. Thus, for a small positive value ϵ, the amount of parity m can be expressed by m=αk+ϵ.

Corollary 2: The Dfr length is given by (1) as follows:

length(DPfr) = [T.length(Mfr) + ϵ] 1D-T .                         (1)

According to Theorem 1 and Corollary 1, there must be greater amounts of parity bits to be restored than information bits, which could be the I frame bits that should be recovered when all frames in consecutive GoPs are lost. As a result, when the amount of loss is αk, the possible parity length required to restore should be m=αk+ϵ; in the same manner, to restore the T.Mfr, [T.length(Mfr) + ϵ] is required, and when T amount of GoP in G is lost, the required information can be restored using a (D-T) amount of GoP and using RaptorQ-like decoding. Therefore, the final required length of Dfr is given by (1).

In order to calculate the amount of increased redundancy for the GD-FEC using scalable video coding (SVC), the following SVC data can be prepared. Undoubtedly, the following numeric can be adapted to suit various needs, but it can be assumed this as an example of a practical case. The test stream consists of four layers: a 320 x 240 ‘Base Layer’ (L0), a 640 x 480 ‘Enhanced Layer’ Q0 (L1), a 640 x 480 ‘Enhanced Layer’ Q1 (L2), and a 640 x 480 ‘Enhanced Layer’ Q2 (L3). The bitrates were 476 kbps for L0, 301 kbps for L1, 2600 kbps for L2, and 5559 kbps for L3; thus, the total rate was 8936 kbps. Given this numeric, the portion of the base layer in the total stream is below five percent (i.e. 476/8936 = 5%). The portion of I frame in the base layer is about 25% and so total portion of I frame is about 1% and this percent can also be adjusted for the capacity of the last position in a GoP in MBMS.

5. Evaluation Criteria

According to Theorem 1, we must have greater amounts of parity bits than the amounts of bits to be restored. Furthermore, we also need the bits of the I-frame that we want to recover when the all frames in consecutive GoPs are lost. Therefore, it is not adequate for the GD-FEC to adopt the minimum required parity proposed for ordinary AL-FEC performance measures because GD-FEC is not focusing on ϵ (see Corollaries 1 and 2).

The proposed loss model is a burst loss model that can provide the results of losing whole frames in consecutive GoPs using the Gilbert Elliot (GE) model [2]. This model was proposed and the liaison for its evaluation was sent to RAN1 and RAN2 at the Jeju meeting.

For the GD-FEC performance metric, we propose the following.

- The loss rate should be a weighted coded frame loss rate because the GD-FECs can be applied after ordinary AL-FEC decoding (e.g. RaptorQ) and focusing on only I-frames.

- To evaluate clear performance, when the I-frame is lost, all frames in that GoP are assumed to be lost because without I frame in a GoP pictures are easily smeared for incorrect prediction.

4. Conclusion

We hope the EMM-EFEC SWG accepts the proposed evaluation and selection criteria for the GD-FEC in MBMS.

References

- ETRI, “Double XOR Operational AL-FEC for Graceful Degradation in MBMS,” 3GPP TSG-SA4 #66 Tdoc S4-110918, Jeju Island, South Korea, November 7-11, 2011.

- Fraunhofer HHI, Samsung, ETRI, “MMT D1-FEC: Proposal evaluation model for AL-FEC schemes,” ISO/IEC JTC1/SC29/WG11, MPEG 98th meeting, Doc. Number M22840, November/December 2011.

- 3GPP, TS 26.346 v10.1.0, Sep. 2011.