3GPP TSG-SA4 Meeting #67 	S4-(11) S4-120149

# Edinburgh, UK , 28-29 January 2012

| CR-Form-v9.9 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST (DRAFT) |  |  |  |  |  |  |  |  |
|  | 26.131 | CR |  | rev | - | Current version: | 10.4.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment of sidetone delay requirements specification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Deutsche Telekom, Vodafone D2 GmbH, Orange SA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ext_ATS |  |  |  |  |  |  | Date: |  | 17/01/2012 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-11 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | Currently there is no sidetone delay requirements/recommendations for narrow band while for wide band this exists. A requirement for narrowband is added. The recommendation for wideband is aligned to the requirment introduced for narrowband. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Inclusion of new requirements for narrow band side tone delay. Alignment of the recommendation for wideband to be a requirement as well. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Terminal may have a disturbing sidetone delays for narrow band and wideband calls. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 5.5.2 and section 6.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Modified Clause

-------------------------------------START OF MODIFIED CLAUSE---------------------------------

## 5.5	Sidetone characteristics (handset and headset UE)

### 5.5.1	Sidetone loss

The talker sidetone masking rating (STMR) shall be ≥ 15 dB and should be ≤ 23 dB for the nominal setting of the volume control. For all other positions of the volume control, the STMR shall be ≥ 10 dB.

Compliance shall be checked by the relevant test described in 3GPP TS 26.132. The bandwidth for the sidetone path provided by the UE may in some terminals not be restricted to the narrowband range. In case the sidetone path operates in a mode other than narrowband (to be declared by the manufacturer), compliance shall be checked using the test described for “Wideband telephony transmission performance”.

NOTE 1:	Where a user-controlled receiving volume control is provided, it is recommended that the sidetone loss is independent of the volume control setting.

NOTE 2:	In general, it is recommended to provide a terminal sidetone path for handset and headset UEs.

NOTE 3:	In case the human air-conducted sidetone paths are obstructed (one example being some binaural insert type headset UEs), it is important to provide a terminal sidetone path.

### 5.5.2	Sidetone delay

The maximum sidetone delay shall  be ≤ 5 ms, measured in an echo-free setup.

NOTE:	The measured result is only applicable where the level of the electrical sidetone is sufficiently high to be measured. While the STMR value may indicate the presence of sidetone it should be ensured that this is not primarily due to the acoustical or mechanical sidetone path when interpreting sidetone delay results.

Compliance shall be checked by the relevant test described in TS 26.132.

-------------------------------------END OF MODIFIED CLAUSE---------------------------------

-------------------------------------START OF MODIFIED CLAUSE---------------------------------

## 6.5	Sidetone characteristics (handset and headset UE)

### 6.5.1	Sidetone loss

The talker sidetone masking rating (STMR) shall be ≥ 15 dB and should be ≤ 23 dB for the nominal setting of the volume control. For all other positions of the volume control, the STMR shall be ≥ 10 dB.

Compliance shall be checked by the relevant test described in TS 26.132.

NOTE 1:	Where a user-controlled receiving volume control is provided, it is recommended that the sidetone loss is independent of the volume control setting.

NOTE 2:	In general, it is recommended to provide a terminal sidetone path for handset and headset UEs.

NOTE 3:	In case the human air-conducted sidetone paths are obstructed (one example being some binaural insert type headset UEs), it is important to provide a terminal sidetone path.

### 6.5.2	Sidetone delay

The maximum sidetone delayshall  be ≤ 5 ms, measured in an echo-free setup.

NOTE:	The measured result is only applicable where the level of the electrical sidetone is sufficiently high to be measured. While the STMR value may indicate the presence of sidetone it should be ensured that this is not primarily due to the acoustical or mechanical sidetone path when interpreting sidetone delay results.

Compliance shall be checked by the relevant test described in TS 26.132.

-------------------------------------END OF MODIFIED CLAUSE---------------------------------