3GPP TSG-SA4 Meeting #67 	S4-(11) S4-120150

# Edinburgh, UK , 28-29 January 2012

| CR-Form-v9.9 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST (DRAFT) |  |  |  |  |  |  |  |  |
|  | 26.132 | CR |  | rev | - | Current version: | 10.3.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment of narrow band sidetone delay tests methods to those of wide band |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Deutsche Telekom AG, Vodafone D2 GmbH, Orange SA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ext_ATS |  |  |  |  |  |  | Date: |  | 17/01/2012 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-11 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | Currently there is no sidetone delay tests method for narrow band while for wide band this exists. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Inclusion of test method for narrow band side tone delay |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Terminal may have a disturbing sidetone delay for narrow band calls. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 7.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Modified Clause

-------------------------------------START OF MODIFIED CLAUSE---------------------------------

## 7.5	Sidetone characteristics

### 7.5.1	Connections with handset UE

The test signal to be used for normative measurements shall be the artificial voice according to ITU-T Recommendation P.50 [10]. Alternatively, a speech-like test signal as described in ITU-T Recommendation P.501 may be used. The type of test signal used shall be stated in the test report. The spectrum of the acoustic signal produced by the artificial mouth is calibrated under free-field conditions at the MRP. The test signal level shall be –4,7 dBPa measured at the MRP. The test signal level is averaged over the complete test signal sequence.

#### 7.5.1.1 void

#### 7.5.1.2 Connections with handset UE – HATS method

The handset UE is setup as described in clause 5. The application force shall be 13 N on the Type 3.3 artificial ear.

Where a user-operated volume control is provided, the measurements shall be carried out at the nominal setting of the volume control. In addition, the measurement is repeated at the maximum volume control setting.

Measurements shall be made at 1/12-octave intervals as given by the R.40 series of preferred numbers in ISO 3 for frequencies from 100 Hz to 8 kHz inclusive. For the calculation, the averaged measured level at each frequency band (ITU-T Recommendation P.79 [16], table 4, bands 4 to 17) is referred to the averaged test signal level measured in each frequency band.

The sidetone path loss (LmeST), as expressed in dB, and the Sidetone Masking Rating (STMR), expressed in dB, shall be calculated from formula 5-1 of ITU-T Recommendation P.79 [16], using m = 0.225 and the weighting factors in table B.2 (unsealed condition) of ITU-T Recommendation P.79 [16]. No leakage correction (LE) shall be applied. DRP-ERP correction is used.

### 7.5.2	Headset UE

The test signal to be used for normative measurements shall be the artificial voice according to ITU-T Recommendation P. 50 [10]. Alternatively, a speech-like test signal as described in ITU-T Recommendation P.501 may be used. The type of test signal used shall be stated in the test report. The spectrum of the acoustic signal produced by the artificial mouth is calibrated under free-field conditions at the MRP. The test signal level shall be –4,7 dBPa measured at the MRP. The test signal level is averaged over the complete test signal sequence.

Measurements shall be made at 1/12-octave intervals as given by the R.10 series of preferred numbers in ISO 3 for frequencies from 100 Hz to 8 kHz inclusive. For the calculation, the averaged measured level at each frequency band (ITU-T Recommendation P.79 [16], table 4, bands 4 to 17) is referred to the averaged test signal level measured in each frequency band.

The sidetone path loss (LmeST), as expressed in dB, shall be calculated from each band of the 14 frequencies given in table 1 of ITU-T Recommendation P.79 [16], bands 4 to 17. The STMR (in dB) shall be calculated from formula B-4 of ITU-T Recommendation P.79 [16], using m = 0.225 and the weighting factors in table B.2 (unsealed condition) of ITU-T Recommendation P.79 [16]. No leakage correction (LE) shall be applied. DRP-ERP correction is used.

### 7.5.3	Hands-free UE (all categories)

No requirement other than echo control.

### 7.5.4	Sidetone delay for handset or headset

The handset or headset terminal is setup as described in clause 5.

The test signal is a CS-signal complying with ITU-T Recommendation P.501 using a PN-sequence with a length, T, of 4 096 points (for a 48 kHz sample rate test system). The duration of the complete test signal is as specified in ITU-T Recommendation P.501. The level of the signal shall be -4,7 dBPa at the MRP.

The cross-correlation function xy() between the input signal Sx(t) generated by the test system in send direction and the output signal Sy(t) measured at the artificial ear is calculated in the time domain:

The measurement window, T, shall be identical to the test signal period, T, with the measurement window synchronized to the PN-sequence of the test signal.

The sidetone delay is calculated from the envelope E() of the cross-correlation function xy(). The first maximum of the envelope function occurs in correspondence with the direct sound produced by the artificial mouth; the second one occurs with a possible delayed sidetone signal. The difference between the two maxima corresponds to the sidetone delay. The envelope E() is calculated by the Hilbert transformation H {xy()} of the cross-correlation:

It is assumed that the measured sidetone delay is less than T/2.

-------------------------------------END OF MODIFIED CLAUSE---------------------------------