Source:	Huawei, HiSilicon,China Telecom

Title:	GAP analysis of the use cases for Operator control of DASH

Document for:	for discussion and agreement

Agenda Item:	7

# Introduction

In TR26.938 [1], several use cases about operator control of DASH are presented. In this paper, the GAP analysis between current EPC architecture and DASH use cases are provided.

# Analysis

## Use case 7.14.2.1 Analysis

In use case 7.14.2.1 of TR26.938 [1], it is mentioned that the operator is aware of cell congestion and communicates with the UE to decrease the bitrate of the video.  SA2 has started working on the user plane congestion awareness, so we assume that the DASH proxy is aware of congestion of cell in this paper. The question will become when the congestion of cell occurs, how does the DASH proxy mitigate the congestion?

As mentioned in use case 7.10, the following three cases can happen:

-	The operator is able to read the MPD and knows how client will use the MPD in terms of issuing requests for specific Adaptation Sets, Representations, and so on;

-	The operator is able to read the MPD but does not know how client will use the MPD;

-	The operator does not have access to the MPD.

If the DASH proxy is able to read the MPD and knows how client will use the MPD, and the DASH proxy is aware of cell congestion, then the DASH proxy can tailor the MPD per current cell congestion info and operator policy, only the allowed adaption set/ representation info will be remained in the MPD, then the DASH proxy can push the tailor MPD down to the UE. The allowed adaption set/representation info refers to a lower bitrate of the video. When the UE requests segment within the tailored representation, the DASH proxy serves the segment per UE request. If the UE requests segment outside of tailored representation, the DASH proxy may return a segment close to the requested segment or error message.

If the DASH proxy is able to read the MPD but does not know how client will use the MPD, and the DASH proxy is aware of cell congestion, then the DASH proxy can behave the same way as previous case.

If the DASH proxy does not have access to the MPD and the DASH proxy is aware of cell congestion, and the segment request from the UE comes to the DASH proxy, the DASH proxy may return a different version of adaption set or presentation in order to downgrade the video quality and relief the cell congestion.

If the cell is heavily congested, the DASH proxy can reject the HTTP request from the UE or delay the HTTP request for a while with the HTTP status code 503(service unavailable).

As discussed above, the DASH proxy should have following functionality:

- Be aware of cell congestion

- Retrieve the MPD from the DASH server

- Tailor MPD info per cell congestion info and operator policy

- Update the tailored MPD info with the UE

- Cache adaption/representation/segment locally

- Select appropriate segment for the HTTP request from the UE

- Admission control of HTTP request, including rejection the new coming HTTP request or delay of the new HTTP request for a certain time

If the DASH server locates inside of operator domain, the DASH server can have the similar functionality as DASH proxy.

## Use case 7.14.2.2 Analysis

In the use case 7.14.2.2 ‘Operator control using mobile network subscription’, the premium user can still enjoy the better video quality in cell congestion case. As described in the previous case, the DASH proxy can’t differentiate premium user from ordinary user for streaming service without the subscription info. In current EPC architecture, the subscriber info is stored at the HSS. SPR also has subscriber info, and PCRF can retrieve the subscriber info via Sp interface from the SPR. Considering the DASH proxy acts as an application server, so the DASH proxy can interact with the PCRF via Rx interface and know whether a user is a premium user or not, then the DASH proxy can deliver the segment requested by a premium user’s and provide low quality segment to the ordinary user’s HTTP request. Moreover, the DASH proxy can interact with PCRF to decrease the ordinary user’s default EPS bearer bandwidth during cell congestion period. The PCRF can adjust ordinary user’s default EPS bearer bandwidth back to normal when the cell congestion is released.

In TS23.203, Application Function (AF) has been defined to communicate with PCRF for the PCC rule selection in order to support application QoS requirement. In TS26.247 Annex I, it is mentioned that the HTTP streaming server or HTTP proxy can act as Application Function. Based on the existing working result, The DASH proxy can appear as an AF and provide information to the PCRF for the purpose of operator control of DASH service.

As discussed above, the DASH proxy should support following functionality:

- Interact with the PCRF for the verification of premium user for the DASH service

- Interact with the PCRF to update the EPS default bearer bandwidth for the DASH service

## Use case 7.14.2.3 Analysis

In the use case 7.14.2.3 “Operator management of DASH service over the radio network”, the operator needs to limit the aggregate bandwidth of video service over the air in order to give some space to other service or subscriber. It also assumes that “The network is able to evaluate, where possible, the appropriate bitrate to be used by DASH users over a 3GPP radio, based on radio conditions and user subscriptions.”  Beside of awareness of cell load info, this use case implies that the aggregate bandwidth of video service within a cell is limited or below a threshold, the RAN node should be able to throttle the aggregate bandwidth of video service. If all DASH traffic is carried over dedicated EPS bearer associated with MBR/GBR attribute, the PCRF may know the sum of aggregate bandwidth of the video within the cell, then the PCRF can perform admission control or QoS update procedure to allow other service or subscriber to be served by the cell. If DASH traffic is carried over EPS default bearer associated with APN-AMBR attribute, the DASH traffic mixes with other non-DASH traffic, how to calculate the sum of DASH traffic bandwidth among UEs within the cell will be a problem? The following issues should be investigated:

Identify the video traffic in the radio network

Determine the aggregate traffic of video service within the cell

Setting of the video traffic threshold in the cell

As discussed above, this use case requires the DASH proxy to interact with PCRF to throttle the DASH traffic over the air. The current EPC architecture can address partial use case here, and the PCC and EPC architecture part need to be enhanced to support the use case.

## Use case 7.15 Analysis

In the use case 7.15, the usage of DASH proxy cache is presented. It requires the DASH proxy has the caching function. However, how does the DASH proxy perform caching or which segment/representation is cached is totally implementation specific, it should be outside of 3GPP scope. When the DASH proxy doesn’t have the requested segment/representation, the DASH proxy is far from the original server and similar segment/representation content is cached locally, in order to improve the user’s DASH service experience, the DASH proxy can provide alternative segment/presentation close to the requested segment/presentation to the UE.

As discussed above, this use case requires the DASH proxy to support local caching and rate adaption of segment/representation.

# Proposed DASH proxy Deployment Model

Proposed the deployment model of AF(DASH proxy) inside of operator network is depicted below.

Figure 1: DASH proxy Deployment Model

The AF(DASH proxy) acts as an application server and connects to the PCRF via Rx interface. DASH server can locate inside of operator domain or outside of operator domain.  The DASH server-a is inside of operator domain and connects to the PCRF via Rx interface. The DASH server-b is outside of operator domain. IP interface is used between the DASH proxy and DASH server. The P-GW connects to DASH proxy and DASH server via SGi interface respectively.

When the operator control of DASH service is supported at the AF (DASH proxy or DASH server) , some possible impact on Rx interface are listed below:

- MPD update based the information exchange on Rx interface

- Representation/Segment selection based on the information exchange on Rx interface

- Admission control of DASH traffic based on the information exchange on Rx interface

The deployment of AF(DASH proxy/cache) doesn’t impact the behaviour of DASH client in the UE, the AF(DASH proxy/cache) will be transparent to the DASH client in the UE.

If the DASH traffic is carried over BE type EPS bearer, video traffic identification in the RAN and the throttle of video traffic in the RAN should be investigated.

# Proposal

# It is proposed to discuss and agree the text in clause 2 and incorporated them into sec7.14 and 7.15 respectively.

# It is proposed to discuss and agree the text in clause 3 and incorporated them into a new sub section in 6.4.

# Reference

[1] TR26.938 “Improved Support for Dynamic Adaptive Streaming over HTTP in 3GPP”

[2] TS23.203 “Policy and charging control architecture”

[3] TR26.938 “Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)”