Source:	HEAD acoustics GmbH

Title:	VoLTE Clock Drift Measurements using different signals and procedures

Document for:	Discussion

Agenda Item:	9.5

# Introduction

This contribution provides an overview of different types of clock drift measurements using different types of signals and different types of improved slope estimation.

# Measurements

## Configuration setup

The setup shown in Fig. 1

Figure 1: Test setup of VoLTE acoustic testing.

The connections are as follows:

- ACQUA measurement system to

- MFE VI.1 via USB to

- MFE VIII.1 via AES to

- CMW 500 to

- DUT to HMS with HHP to MFE VI.1

Since all measurements were made in receiving the de-jitter jitter buffer setting of the MFE VIII.1 does not influence any measurement, any effects of the de-jitter buffer observed are due to the jitter buffer handling of the DUT..

The evaluation was performed with 3+2 VoLTE DUTs in receiving direction.

The delay vs. time curves are calculated as described in the draft CR to 26.132 in Annex D.

## P.501 test signals with step size 32 s

The British English P.501 sentences (8 sentences each centered in 4 seconds grid) are repeated 30 times leading to a sound file of 960 s = 16 min. To calculate the delay vs. time curve, the whole measurement is used with a step size of 32 s leading to 30 delay values.

The estimated clock drift for DUT A is ∆delay∆time⋅106= 136.9ms-141.4ms938.2s-21.8s⋅106 = -4.9 ppm, meaning that the clock of the DUT is a little bit faster than the clock of the MFE VIII.1.

The estimated clock drift for DUT B is ∆delay∆time⋅106= 156.7ms-159.4ms+145.4ms-150.5ms306.2s-21.8s+938.2s-337.8s⋅106 = -8.8 ppm, calculated in the two “jump-free” segments.

The estimated clock drift for DUT C is ∆delay∆time⋅106= 187.8ms-210.4ms938.2s-21.8s⋅106 = -24.8 ppm.

To simulate a DUT with higher clock drift, the clock of the MFE VIII.1 is put out of tune by -50ppm and used with DUT A. This is called DUT A+ in the following.

The estimated clock drift for DUT A+ is ∆delay∆time⋅106= 128.6ms-139.3ms+118.6ms-145.1ms+128.0ms-137.1ms243.0s-53.4s+748.6s-306.2s+938.2s-780.2s⋅106 = -58.6 ppm.

To simulate another DUT with even higher clock drift, the clock of the MFE VIII.1 is put out of tune by -100ppm and used with DUT B. This is called DUT B+ in the following.

In this measurement a delay jumps of about 20 ms occurs about every 3 minutes, i.e., every 5 to 6 steps. A clean trend however, can only be seen in the first and the last two segments. In the two segments between, the neighboring time steps are severely affected by the delay jumps leaving to few time steps for a clean trend.

The estimated clock drift for DUT B+, based on the last two segments, is ∆delay∆time⋅106= 148.4ms-162.4ms+147.0ms-160.8ms717.0s-590.6s+906.6s-780.2s⋅106 = -110.2 ppm. The clock drift based on the first segment is ∆delay∆time⋅106= 134.6ms-146.2ms148.2s-21.8s⋅106 = -91.6 ppm and thus strongly deviates from the last two segments.

## P.501 test signals with step size 4 s

For this evaluation, the first 160 s = 2 min 40 s of the same measurement as before are used to calculate the delay vs. time curve, this time with a step size of 4 s, i.e., one step per sentence, leading to 40 delay values.

The estimated clock drift for DUT A is ∆delay∆time⋅106= 140.4ms-141.4ms159.8s-2.7s⋅106 = -6.2 ppm.

The estimated clock drift for DUT B is ∆delay∆time⋅106= 159.6ms-158.1ms159.8s-2.7s⋅106 = -9.4 ppm.

The estimated clock drift for DUT C is ∆delay∆time⋅106= 206.8ms-210.5ms159.8s-2.7s⋅106 = -24.0 ppm.

The estimated clock drift for DUT A+ is ∆delay∆time⋅106= 131.3ms-138.8ms159.8s-18.8s⋅106 = -53.2 ppm.

The estimated clock drift for DUT B+ is ∆delay∆time⋅106= 130.1ms-145.6ms159.8s-14.8s⋅106 = -107.3 ppm.

## P.501 female 1 sentence 2 test signal with step size 4 s

For this evaluation, the second sentence of British English female 1 speaker of the P.501 sentences (centered in 4 seconds grid) is repeated 40 times leading to a sound file of 160 s length and a step size of 4 s is chosen. This specific sentence was chosen since it had the best autocorrelation properties of the eight British English sentences of P.501.

The estimated clock drift for DUT A is ∆delay∆time⋅106= 127.1ms-128.0ms161.3s-2.7s⋅106 = -5.4 ppm.

The estimated clock drift for DUT B is ∆delay∆time⋅106= 136.6ms-138.2ms161.3s-2.7s⋅106 = -9.9 ppm.

The estimated clock drift for DUT C is ∆delay∆time⋅106= 225.1ms-229.2ms161.3s-2.7s⋅106 = -25.9 ppm.

The estimated clock drift for DUT A+ is ∆delay∆time⋅106= 117.0ms-125.8ms161.3s-2.7s⋅106 = -55.3 ppm.

The estimated clock drift for DUT B+ is ∆delay∆time⋅106= 140.6ms-143.6ms+138.2ms-151.6ms30.5s-2.7s+161.3s-38.4s⋅106 = -109.1 ppm.x

## MSMP test signals with step size 4 s

As a further test signal, the MSMP signal from P.501 of length 150 s is used.

For the following evaluation, a step size of 4 s is chosen.

The estimated clock drift for DUT A is ∆delay∆time⋅106= 125.4ms-126.1ms150.7s-2.7s⋅106 = -4.8 ppm.

The estimated clock drift for DUT B is ∆delay∆time⋅106= 155.1ms-155.4ms+144.6ms-145.6ms34.7s-2.7s+150.7s-38.7s⋅106 = 
-9.3 ppm.

The estimated clock drift for DUT C is ∆delay∆time⋅106= 157.8ms-161.3ms150.7s-2.7s⋅106 = -24.1 ppm.

The estimated clock drift for DUT A+ is ∆delay∆time⋅106= 125.2ms-133.3ms150.7s-2.7s⋅106 = -54.9 ppm.

The estimated clock drift for DUT B+ is ∆delay∆time⋅106= 122.2ms-131.4ms+142.6ms-147.9ms86.7s-2.7s+150.7s-102.7s⋅106 = 
-110.3 ppm.

## CSS test signals with step size ⅓ s

As an alternative test signal, a CSS burst is repeated to fill a sound file of 160 s length.

Note, that the step size must be smaller than the repetition period of the CSS burst to avoid random delay jumps by the length of the CSS burst.

For the following evaluation, a step size of ⅓ s is chosen leading to 479 delay values.

The estimated clock drift for DUT A is ∆delay∆time⋅106= 127.4ms-128.2ms159.8s-0.5s⋅106 = -5.0 ppm.

The estimated clock drift for DUT B is ∆delay∆time⋅106= 483.6ms-485.1ms159.5s-0.5s⋅106 = -9.4 ppm.

The estimated clock drift for DUT C is ∆delay∆time⋅106= 149.5ms-153.4ms156.8s-0.2s⋅106 = -24.7 ppm.

The estimated clock drift for DUT A+ is ∆delay∆time⋅106= 468.3ms-476.8ms159.5s-2.8s⋅106 = -54.8 ppm.

The estimated clock drift for DUT B+ is ∆delay∆time⋅106 = -109.3 ppm, calculated in the 4 clean segments.

# Comparison of Test Signals with Compensated Clock

## P.501 test sentences with step size 4 s

Below the delay vs. time curve of the clock drift compensated DUT A is exemplary shown with the P.501 test sentences and step size 4 s. If the clock drift were not compensated, the variation of this curve were superposed with a linear increase/decrease, possibly including steps due to the jitter buffer over-/underrun. The exact shape of the variation can, however, not be predicted as it depends among others on the acoustic path of the measurement.

The second graph shows the difference between adjacent delay values of the first graph in ms. It can be seen that the difference between two time steps is regularly at about 0.5 ms and in maximum more than 0.8 s.

To illustrate the negative impact of this variation of the delay vs. time curve, let us assume that one of the time steps with a difference of 0.5 ms cannot be considered during clock drift estimation, e.g. due to a jump at that time step or a short disturbance during measurement. In this case, the overall slope of the delay vs. time curve is decreased by 0.5 ms/160 s = 3.1 ppm as depicted in the following graph.

Even though using methods of linear regression for slope estimation might decrease this effect, it cannot be fully compensated. Accordingly, minor changes in the measurement situation, like the position of the jump, minor distortions, a different acoustic path, and so on, have a severe impact on the accuracy.

It should be noted that this is a systematic property of the speech test signal and not an artefact of the calculation technique.

## P.501 test sentences with step size 32 s

With step size 32 s, the variation is averaged out as shown below.

The downside is, that very long measurement times of about 16 minutes are necessary to acquire enough measurement points and that the maximum measureable clock drift is limited by the too frequent occurrence of delay jumps of 10 ms or 20 ms, which are caused by jitter buffer over-/underruns.

## P.501 female 1 sentence 2 test signal with step size 4 s

Below the delay vs. time curve of the clock drift compensated DUT A is exemplary shown with the second sentence of British English female 1 speaker of the P.501 sentences and step size 4 s. The variation of the curve is greatly reduced besides a small distortion in this example.

## MSMP test signals with step size 4 s

Below the delay vs. time curve of the clock drift compensated DUT A is exemplary shown with the MSMP test signal and step size 4 s. The variation of the curve is greatly reduced.

## CSS test signals with step size ⅓ s

Below the delay vs. time curve of the clock drift compensated DUT A is exemplary shown with the CSS test signal and step size ⅓ s. The variation of the curve is also very low.

# Improved slope estimation algorithm

In this section, two improved algorithms to analyze the delay vs. time graph to estimate the clock drift in parts per million (PPM) are presented.

The main problem of clock drift estimation are the delay jumps which are caused by packet insertions or losses due to jitter buffer over-/underruns. Therefore, the general idea is cut out the delay jumps and stitch together the remaining parts of the curve without gap. In the exemplary curve above, the first step of the orange part is stitched on the last step of the green part, the first step of the yellow part is stitched on the last step of the orange part, and so on. This way, a “continuous” delay vs. time curve is generated whose slope is calculated.

A delay jump caused by a jitter buffer over-/underrun results in a high delay difference between two time steps of a few milliseconds. Therefore, delay jumps can easily be detected after differentiation of the measured delay vs. time curve.

Silent signal segments, e.g. at the start or the end, should also be cutted out.

In detail, the cutting and stitching algorithm consists of the following steps:

- Apply a median filter of length 3 to the delay vs. time series Y(k) yielding the time series Y(k). This smoothes away small spikes without soften the delay jumps.

- Calculate the delay difference per time step ΔYk=Yk+1-Yk.

- Exclude all time steps whose absolute delay differences is larger than a threshold ΔYmax.

  - With larger step sizes, the normal clock drift results in higher differences ΔYk and therefore requires a larger threshold ΔYmax. On the one hand, a minimum threshold of 1 ms is given by the “quantization” of the delay vs. time curve. On the other hand, the threshold should not be too large to reliably detect packet losses and insertions:
ΔYmax= minmaxmaximum detectable clock drift in PPM106⋅step size, 1 ms, 4 ms
where the maximum detectable clock drift is chosen to 500 PPM.

  - Since the delay jump might also effect the surrounding time steps, the steps before and after the jump are also excluded.

- Generate the “continuous” delay vs. time series Y(k) as the cumulative sum of all remaining, i.e., not excluded delay differences ΔYk. Note, that the length of the “continuous” series Y(k) is smaller than the length of the series Yk.

- The clock drift is estimated based on the stitched, “continuous” delay vs. time series Yk using the methods of linear regression. In detail, it is calculated from the regression coefficient β, which minimizes the squared error:
D= βstep size⋅106 PPM.

# Discussion

Using the P.501 test signals, the benefit of a step size of 32 s is a very smooth delay vs. time curve which eases automatic clock drift estimation for low clock drifts. On the downside, each measurement cycle takes 16 minutes. Furthermore, the maximum measureable clock drift is limited by the too frequent occurrence of delay jumps of 10 ms or 20 ms, which are caused by jitter buffer over-/underruns.

With step size 4 s and P.501 test sentences, a measurement cycle takes less than 3 minutes but has a high drift variance.

The selected single sentence of the P.501 test set (British English female 1 sentence 2) has a low drift variance and yields good results in less than 3 minutes.

The MSMP signal also has a low drift variance, which is comparable to the single P.501 test sentence and also yields good results in less than 3 minutes.

A fast measurement in less than 3 minutes with very high precision can, however, be accomplished using a composite source signal. Using the CSS test signal, large segments with clean slopes, i.e., very low drift variance, can be found in every measurement. In theory, the CSS test signal of 160 s length can under ideal circumstances yield a clock drift precision of less than 0.15ppm (which is given by resolution of the cross correlation due delay calculation).

The variation of the delay vs. time curve is a direct consequence of the used test signal and has great influence on the accuracy of the clock drift estimation. To achieve a fast measurement which can also capture high clock drifts as well as a reliable estimation with high accuracy, a test signal should be used which has a low variation at step sizes of 4 s or less. Among the examined signals, the selected single P.501 sentence, the MSMP signal, and repeated CSS burst satisfy this requirement.

## Hand read clock drift

The following table summarizes the clock drifts which are extracted by hand from the measurements above

|  | P.501 – 32 s | P.501 – 4 s | P.501 BE F1-2 – 4 s | MSMP – 4 s | CSS – ⅓ s |
| --- | --- | --- | --- | --- | --- |
| DUT A | -4.9 ppm | -6.2 ppm | -5.4 ppm | -4.8 ppm | -5.0 ppm |
| DUT B | -8.8 ppm | -9.4 ppm | -9.9 ppm | -9.3 ppm | -9.4 ppm |
| DUT C | -24.8 ppm | -24.0 ppm | -25.9 ppm | -24.1 ppm | -24.7 ppm |
| DUT A+ | -58.6 ppm | -53.2 ppm | -55.3 ppm | -54.9 ppm | -54.8 ppm |
| DUT B+ | -110.2 ppm | -107.3 ppm | -109.1 ppm | -110.3 ppm | -109.3 ppm |

## Proposed method

|  | P.501 – 32 s | P.501 – 4 s | P.501 BE F1-2 – 4 s | MSMP – 4 s | CSS – ⅓ s |
| --- | --- | --- | --- | --- | --- |
| DUT A | -4.9 ppm | -4.3 ppm | -5.2 ppm | -4.9 ppm | -4.9 ppm |
| DUT B | -8.3 ppm | -9.4 ppm | -10.1 ppm | -9.6 ppm | -9.4 ppm |
| DUT C | -25.2 ppm | -24.6 ppm | -25.9 ppm | -24.3 ppm | -24.5 ppm |
| DUT A+ | -60.7 ppm | -57.4 ppm | -54.7 ppm | -55.2 ppm | -55.2 ppm |
| DUT B+ | -111.4 ppm | -110.4 ppm | -107.1 ppm | -108.7 ppm | -109.4 ppm |

## Old method

|  | P.501 – 32 s | P.501 – 4 s | P.501 BE F1-2 – 4 s | MSMP – 4 s | CSS – ⅓ s |
| --- | --- | --- | --- | --- | --- |
| DUT A | -4.9 ppm | -17.1 ppm | -5.3 ppm | -5.0 ppm | 0.0 ppm |
| DUT B | -9.5 ppm | -18.5 ppm | -10.1 ppm | -10.7 ppm | 0.0 ppm |
| DUT C | -25.0 ppm | -23.8 ppm | -25.6 ppm | -23.4 ppm | -20.5 ppm |
| DUT A+ | -55.8 ppm | -55.8 ppm | -56.3 ppm | -56.4 ppm | -62.4 ppm |
| DUT B+ | -56.9 ppm | -112.0 ppm | -109.1 ppm | -99.7 ppm | -118.2 ppm |

In general, the proposed improved slope estimation algorithm is capable of estimating the clock drift well with a precision

- of 0.4 ppm for the CSS signal with ⅓ s step size for all DUTs,

- of 0.3 ppm for the MSMP signal with 4 s step size for all DUTs but DUT A+,

- of 0.6 ppm for the second sentence of British English female 1 speaker of the P.501 sentences with 4 s step size for all DUTs but DUT A+,

- of 4.2 ppm for the P.501 sentences with 4 s step size for all DUTs but DUT A+, and

- of 2.1 ppm for the P.501 sentences with 32 s step size for all DUTs. If the clock drift is too high, the estimation might fail because the distance between the delay jumps becomes too small.

In general, the precision decreases with increasing the clock drift, especially for DUT B+. In order to the improve accuracy of such measurements a two step approach should be used. In the first step a coarse drift compensation is performed and the clock of the reference client is adjusted to the ppm derived in the first step. A fine compensation is performed in the second step by applying the same procedure. The final clock drift in ppm is the sum of both clock drifts.

# References

- ITU-T Recommendation P.501:Test Signals for Use in Telephonometry, 01/2012

- 3GPP Tdoc S4-140715: Draft CR to 26.132 on LTE UE delay test methods”, TSG SA4-79 Meeting

# Annex: Matlab Implementation of Proposed Method

function Drift = EstimatePPM(DelayVsTime, StepSize, DiffMaxPPM=500)

% Difference threshold from which on a difference is assumed to be a packet

% loss or insertion.  With larger step sizes, the normal clock drift

% results in higher differences between steps and therefore requires a

% larger threshold.

DelayDiffThreshold = min( max( DiffMaxPPM*1e-6*StepSize, 1e-3 ), 4e-3 );

% Calculate median filter of length 3 to smooth away small spikes without

% soften the steps.

DelayMedian = DelayVsTime(:);

for i = 2:numel(DelayVsTime)-1

DelayMedian(i) = median(DelayVsTime(i-1:i+1));

end

% Calculate delay difference per time step.

DelayDiff = diff( DelayMedian );

% Exclude all indices with high delay differences as well as some

% transition range before and after.

Mask = true(size(DelayDiff));

for i = find( abs(DelayDiff) > DelayDiffThreshold )

Mask(i-1:i+1) = false;

end

% Generate "continuous" delay vs. time series

Delay = [ 0; cumsum( DelayDiff(Mask) ) ];

Time = ( 0:numel(Delay)-1)';

% Estimate clock drift in PPM as slope of linear regression line

Drift = cov( Time, Delay, 1 ) / cov( Time, Time, 1 ) / StepSize * 1e6;

# Proposed Substitution for Annex D of 26.132

Annex D [normative]:
Clock drift measurement

This Annex describes a method to measure the clock drift between the reference client and the device under test.

As speech test signal, the second sentence of the first female speaker (female1.wav) of the English test sentences according to ITU-T P.501 is used, pre-filtered according to the application (NB or WB) and normalized to an active speech level of -16dBm0. The sequence is centered within a 4.0s time window and repeated 40 times, resulting in a test file of 160.0s length.

The clock drift is determined as follows:

- The variable delay per time step is calculated as the time shift of the maximum of the cross correlation between measured and source signal for each time step.

- The step size of the calculation of the variable delay is 4.0s for the above test signal, which corresponds to one single sentence of the source signal.

- This analysis provides the output signal Y(tn) consisting of one single delay value for each time step, i.e., for each sentence of the above source signal.

- A median filter of length 3 is applied to the delay vs. time series Y(tn) yielding the time series Ŷ(tn). This smoothes small spikes without soften the delay jumps.

- Delay jumps due to jitter buffer over- or underrun are detected as the time steps where the absolute delay difference abs(Ŷ(tn+1) - Ŷ(tn)) is larger than a threshold ΔYmax, which is appropriate to detect a maximum clock drift of 500 PPM.

- A “continuous” delay vs. time series is generated where the delay jumps are eliminated, e.g., by cutting out the delay jumps and “stitching” together the jump-free segments.

- The clock drift is estimated as the regression coefficient β of the “continuous” delay vs. time series in parts per million (PPM):
D= βstep size⋅106 PPM.

In case of clock drifts higher than 50 ppm the accuracy of the cross-correlation calculation decreases. In such event it is recommended to take a two-step approach. In the first step a coarse drift compensation is performed and the clock of the reference client is adjusted to the ppm derived in the first step. A fine compensation is performed in the second step by applying the same procedure. The final clock drift in ppm is the sum of both clock drifts.