3GPP TSG-SA4 Meeting #81	S4-141283

# Santa Cruz, Tenerife, Spain, November 3 – 7, 2014

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 26.445 | CR | - | rev | - | Current version: | 12.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Further typographical corrections to TS 26.445 (Release 12) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM, Fraunhofer IIS, Huawei Technologies Co. Ltd, Nokia Corporation, NTT, NTT DOCOMO, INC., ORANGE, Panasonic Corporation, Qualcomm Incorporated, Samsung Electronics Co., Ltd., VoiceAge and ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | EVS_Codec |  |  |  |  |  |  | Date: |  | 2014-10-28 |
| Category: | D |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Minor errors identified in the algorithmic description text |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction of typos and insertion of additional clarification text |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification TS 26.445 describing the EVS codec algorithm will be incorrect and/or erroneous/misleading |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.8, 7.1, 7.1.1, 7.1.2, 7.1.3, 7.1.4, 7.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** Start of first change ***

### 5.7.8	Algebraic codebook search

For interoperability reasons, the algebraic codebook structure and pulse indexing is the same as subclauses 5.8.1 and 5.8.2 of [9]. The algebraic codebook search procedure is the same as described in subclause 5.8.3 of [9] except the pulse-sign pre-selection described in the last paragraph of Section 5.2.3.1.5.9 (The search criterion at lower bitrates), which is also used at the lowest bit-rate of AMR-WB-interoperable modes.

*** Start of second change ***

## 7.1	Bit allocation for the default option

The allocation of the bits for various operating modes in the EVS encoder is shown in detail for each bitrate in the following tables. Note that the most significant bit (MSB) of each codec parameter is always sent first. In the tables below, the abbreviation CT is used to denote the coder type and the abbreviation BW is used to denote the bandwidth.

### 7.1.1	Bit allocation at VBR 5.9, 7.2 – 9.6 kbps

The EVS codec encodes NB and WB content at 7.2 and 8.0 kbps with CELP core or HQ-MDCT core. No extension layer is used at these bitrates. The EVS codec encodes NB and WB content at 9.6 kbps with CELP core or TCX core. To encode WB signals at 9.6 kbps, the CELP core uses TBE extension layer and the TCX core uses IGF extension layer. Similarly to encode SWB signals at 9.6 kbps, the CELP core uses TBE extension layer and the TCX core uses IGF extension layer.

VBR mode uses 4 different active frame types with different bit rates to achieve the average bit rate of 5.9 kbps. The 4 different frame rates are 2.8 kbps PPP frame, 2.8 kbps NELP frame, and 7.2 kbps and 8 kbps CELP frames. The CT bits are allocated as 1 bit to differentiate active 2.8 kbps (PPP or NELP) frames from any other 2.8 kbps frames (such as SID frame with payload header) and the remaining 2 bits are used to represent NB PPP, WB PPP, NB NELP and WB NELP frames.

Table 179: Bit allocation at 7.2 – 9.6 kbps and 2.8 kbps PPP/NELP

| Description | 2.8 PPP | 2.8 NELP | 7.2 | 8.0 | 9.6 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| core | CELP | CELP | CELP HQ-MDCT | CELP HQ-MDCT | CELP |  | TCX |
| ext. layer | NO | NO | NO | NO | SWB TBE | WB TBE | IGF |
| Number of bits per frame | 56 | 56 | 144 | 160 | 192 |  |  |
| BW |  |  | 4 |  | 2 |  |  |
| CT | 3 | 3 |  |  | 3 |  |  |
| core bits | 53 | 53 | 140 | 156 | 171 | 181 | 187 |
| WB/SWB ext. layer bits |  |  |  |  | 16 | 6 |  |

Note that the BW and CT parameters are combined together to form a single index at 7.2 and 8.0 kbps. This index conveys the information whether CELP core or HQ-MDCT core is used. At 9.6 kbps, the information about using the CELP core or the TCX core is encoded as a part of the CT parameter.

### 7.1.2	Bit allocation at 13.2 kbps

The EVS codec encodes NB, WB and SWB content at 13.2 kbps with CELP core, HQ-MDCT core, or TCX core. For WB signals, the CELP core uses TBE or FD extension layer. For SWB signals, the CELP core uses TBE or FD extension layer, and the TCX core uses IGF extension layer.

Table 180: Bit allocation at 13.2 kbps

| Description | 13.2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| core | CELP | HQ-MDCT | TCX | CELP |  |  |  | TCX |
| ext. layer | NO | NO | NO | WB TBE | WB FD | SWB TBE SWB FD |  | IGF |
| Number of bits per frame | 264 |  |  |  |  |  |  |  |
| BW, CT, RF | 5 |  |  |  |  |  |  |  |
| TCX/HQ-MDCT core flag |  | 1 | 1 |  |  |  |  | 1 |
| TCX CT |  |  | 2 |  |  |  |  | 2 |
| TD/FD ext. layer flag |  |  |  | 1 | 1 |  | 1 |  |
| core bits | 259 | 258 | 256 | 238 | 252 |  | 227 | 256 |
| WB/SWB ext. layer bits |  |  |  | 20 | 6 |  | 31 |  |

Note that the BW, CT, and RF parameters are combined together to form a single index. This index also conveys the information whether LP-based core or MDCT-based core (TCX or HQ-MDCT) is used. The decision between the HQ-MDCT core and the TCX core is encoded with one extra bit called MDCT core flag. At this bitrate, the TCX coder type is encoded with 2 extra bits (TCX CT).

### 7.1.3	Bit allocation at 16.4 and 24.4 kbps

The EVS codec encodes NB, WB, SWB and FB content at 16.4 and 24.4 kbps with CELP core, HQ-MDCT core or TCX core. For SWB and FB signals, the CELP core uses TBE extension layer and the TCX core uses IGF extension layer.

Table 181: Bit allocation at 16.4 kbps

| Description | 16.4 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| core | CELP | TCX | HQ-MDCT | CELP | TCX | CELP |
| ext. layer | NO | NO | NO | SWB TBE | IGF | FB TBE |
| Number of bits per frame | 328 |  |  |  |  |  |
| BW | 2 |  |  |  |  |  |
| Reserved flag | 1 |  |  |  |  |  |
| CT | 3 | 4 | 2 | 3 | 4 | 3 |
| core bits | 322 | 321 | 323 | 286 | 321 | 287 |
| SWB ext. layer bits |  |  |  | 33 |  | 31 |
| FB ext. layer bits |  |  |  |  |  | 4 |
| Padding bits |  |  |  | 3 |  |  |

Table 182: Bit allocation at 24.4 kbps

| Description | 24.4 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| core | CELP | TCX | HQ-MDCT | CELP | TCX | CELP |
| ext. layer | NO | NO | NO | SWB TBE | IGF | FB TBE |
| Number of bits per frame | 488 |  |  |  |  |  |
| BW | 2 |  |  |  |  |  |
| Reserved flag | 1 |  |  |  |  |  |
| CELP/MDCT core flag | 1 |  |  |  |  |  |
| TCX/HQ-MDCT core flag |  | 1 | 1 |  | 1 |  |
| CELP->HQ core switching flag |  |  | 1-2 |  |  |  |
| CT | 2 | 2 |  | 2 | 2 | 2 |
| core bits | 482 | 481 | 481-2 | 422 | 481 | 423 |
| SWB ext. layer bits |  |  |  | 57 |  | 55 |
| FB ext. layer bits |  |  |  |  |  | 4 |
| Padding bits |  |  |  | 3 |  |  |

The information about using the CELP core or the MDCT-based core (HQ-MDCT or TCX) is transmitted as a 1-bit CELP/MDCT core flag. In the case of MDCT-based core, the next bit decides whether HQ-MDCT core or TCX core is used. In the case of TCX, the remaining 2 bits are used to represent the TCX coder type (TCX CT). In the case of HQ-MDCT core, the next one or two bits signal whether the previous frame was encoded with the CELP core or not. The second bit is used to signal its internal sampling rate (12.8 or 16 kHz) only when the previous frame was encoded with the CELP core.

### 7.1.4	Bit allocation at 32 kbps

The EVS codec encodes WB, SWB and FB content at 32 kbps with CELP core, HQ-MDCT core, or TCX core. For SWB and FB signals, the CELP core uses TBE or FD extension layer and the TCX core uses IGF extension layer.

Table 183: Bit allocation at 32 kbps

| Description |  | 32 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| core |  | CELP |  | HQ-MDCT |  | TCX |  | CELP |  | TCX |  | CELP |  |
| ext. layer |  | NO |  | NO |  | NO |  | SWB TBE/FD |  | IGF |  | FB TBE/FD |  |
| Number of bits per frame |  | 640 |  |  |  |  |  |  |  |  |  |  |  |
| CELP/MDCT core flag |  | 1 |  |  |  |  |  |  |  |  |  |  |  |
| CELP->HQ core switching flag |  |  |  | 1-2 |  |  |  |  |  |  |  |  |  |
| TCX/HQ-MDCT core flag |  |  |  | 1 |  | 1 |  |  |  | 1 |  |  |  |
| BW |  | 4 |  | 2 |  | 2 |  | 4 |  | 2 |  | 4 |  |
| CT |  |  |  |  |  | 2 |  |  |  | 2 |  |  |  |
| TBE/FD ext. layer flag |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| core bits |  | 634 |  | 632-3 |  | 632 |  | 602 |  | 633 |  | 576 |  |
| SWB ext. layer bits |  |  |  |  |  |  |  | 55/31 |  |  |  | 55/31 |  |
| FB ext. layer bits |  |  |  |  |  |  |  |  |  |  |  | 4 |  |

The information about using the CELP core or the MDCT-based core (HQ-MDCT or TCX) is transmitted as a 1-bit CELP/MDCT core flag. If CELP core is selected, the BW and CT parameters are combined together to form a single index. In the case of MDCT-based core, the next bit decides whether HQ-MDCT core is used or the TCX core is used. In the case of TCX, the remaining 2 bits are used to represent the TCX coder type (TCX CT). In the case of HQ-MDCT core, the next one or two bits signal whether the previous frame was encoded with the CELP core or not. The second bit is used to signal its internal sampling rate (12.8 or 16 kHz) only when the previous frame was encoded with the CELP core. Finally, 1 bit is used to distinguish between TBE and FD extension layer in the case of CELP core.

### 7.1.5	Bit allocation at 48, 64, 96 and 128 kbps

The EVS codec encodes WB, SWB and FB content at 48 kbps with TCX core only. For SWB and FB signals, the TCX core uses IGF extension layer. At 64 kbps, the EVS codec encodes WB, SWB and FB content with CELP core or HQ-MDCT core. For SWB and FB signals, the CELP core uses FD extension layer.

Table 184: Bit allocation at 48, 64, 96 and 128 kbps

| Description | 48 |  | 64 |  |  |  | 96 |  | 128 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| core | TCX | TCX | CELP | HQ-MDCT |  | CELP | TCX | TCX | TCX | TCX |
| ext. layer | NO | IGF | NO | NO |  | SWB FD FB FD | NO | IGF | NO | IGF |
| Number of bits per frame | 960 |  | 1280 |  |  |  | 1920 |  | 2560 |  |
| CELP/MDCT core flag |  |  | 1 |  |  |  |  |  |  |  |
| CELP->HQ core switching flag |  |  |  |  | 1-2 |  |  |  |  |  |
| TCX/HQ-MDCT core flag |  |  |  |  | 1 |  |  |  |  |  |
| BW | 2 |  | 4 |  | 2 | 4 | 2 |  | 2 |  |
| CT |  |  |  |  |  |  |  |  |  |  |
| Reserved flag | 1 |  |  |  |  |  | 1 |  | 1 |  |
| TCX CT | 3 |  |  |  |  |  | 3 |  | 3 |  |
| core bits | 954 | 954 | 1275 |  | 1274-5 | 954 | 1914 | 1914 | 2554 | 2554 |
| ext. layer bits |  |  |  |  |  | 326 |  |  |  |  |

At 64 kbps, the information about using the CELP core or the HQ-MDCT core is transmitted as a 1-bit CELP/MDCT core flag. If CELP core is selected, the BW and CT parameters are combined together to form a single index. In the case of HQ-MDCT core, the next one or two bits signal whether the previous frame was encoded with the CELP core or not. The second bit is used to signal its internal sampling rate (12.8 or 16 kHz) only when the previous frame was encoded with the CELP core.

*** End of changes ***