3GPP TSG-SA WG4 Meeting #90 	S4-160873

# Ljubljana, Slovenia, 3-9 September 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.931 | CR | CRNum | rev | - | Current version: | 0.0.9 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Additional information on use of P.862 and P.863 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Knowles Inc., Samsung Electronics Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_SEATS |  |  |  |  |  |  | Date: |  | 2016-06-27 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Extends analysis of behaviour of subjective and predictive measures in background noise with noise suppression to WB and SWB |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates to 5.4.1, and 5.4.2; new clauses 5.4.3, 5.4.4; former 5.4.3 now updated to 5.4.5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete understanding of ability of metrics to describe and predict the subjective ratings. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1, 5.4.2., 5.4.3, 5.4.4, 5.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | N | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### ----------start of modified clause--------

### 5.4.1 Comparison of P.862 to subjective results for noise suppression

ITU-T Recommendation P.862, Perceptual Evaluation of Speech Quality (PESQ) [1], was published in 2001 to predict subjective scores as obtained in ITU-T Recommendation P.800 Absolute Category Rating (ACR) tests of Overall Speech Quality [2]. P.862 [1] is a very useful tool for assessing the speech quality of devices in many situations; however it has significant limitations for assessing speech quality when noise suppression is used, primarily because the model was developed and trained before modern UE noise suppression evolved. Since almost all modern UE’s contain some form of noise suppression algorithm, P.862 [1] should not be used for testing UE’s in background noise as it produces misleading results as demonstrated in the following sections.

The inadequacy of P.862 [1] for devices incorporating noise suppression is well documented. ITU-T Recommendation P.862.3, Application Guide for Objective Quality Measurement Based on Recommendations P.862, P.862.1, and P.862.2 [3] provides unambiguous guidance regarding usage with noise suppression as can be seen in the following excerpt [p3-4]:

Figure 1:  Excerpt from ITU-T P.862.3 [3]

ITU-T Recommendation P.835 is used for subjective evaluation of systems with noise suppression [4].  P.835 [4] evaluations use three rating scales: SIG assessing the amount of speech distortion; BAK assessing the degree of intrusiveness of any background noise; OVRL providing an overall speech quality assessment.  ETSI TS 103 106 [5] was designed specifically to predict the three ratings from a P.835 [4] test and was explicitly trained on noise reduction.  In contrast, both P.862 [1] and P.863 [6] were designed to predict results from P.800[2] listening tests, which only have one rating scale, overall speech quality, MOS-LQS.  In fact, P.835 [4] was developed partly in reaction to a high degree of uncertainty observed in P.800 [2] tests on conditions with noise reduction.  Studies directly comparing P.800 [2] MOS-LQS scores to P.835 [4] OVRL scores are predominantly proprietary, but experience has shown that there is generally good correlation between these two metrics.

An objective quality predictor is normally trained on a wide variety of conditions, while a single subjective experiment includes a more limited set of conditions which potentially changes the experimental context. Therefore results from a specific subjective experiment may deviate from objective predictions, hence the results below should be considered as case studies, not as exhaustive and definitive results.  However, these case studies can be considered as indicative of the nature of issues that could arise when predictors are applied outside their scope.

### 5.4.2 Experiment 1 – NB P.835 versus P.862.1

### 5.4.2.1 Setup

In Experiment 1, a recording of American English speech consisting of four sentences from each of two male and two female native talkers, as used in ETSI TS 103 106 [5], was reproduced through an equalized HATS artificial at 92.3 dB SPL active speech level at the MRP of HATS.  The background noise generation method described in ETSI ES 202 396-1 [7] was used to reproduce eight noises described in Table 2d of Clause 7.12 of 3GPP TS 26.132 [8].  Six handsets were used, denoted as A, B, C, D, E, and F in the following plots.  Each device was mounted on the HATS in standard position and recordings made of clean speech, and speech mixed with the eight noises described above, at the output of a UMTS base-station simulator with AMR speech encoding at 12.2 kbit/s.

ITU-T Recommendation P.835 [4] is the recommended methodology for assessing the listening quality of systems incorporating noise suppression. A group of 32 naïve listeners, all native speakers of American English, using the P.835 [4] methodology, rated all sentences in all conditions in a partially balanced randomized blocks design, resulting in 128 votes per condition.  Results were used as training data for ETSI TS 103 106 [5] with further details in Clause 7.2.1 of that document.

For computing metrics, all recordings were taken at 48 kHz sample rate.  For P.862.1 [9], the source file was filtered to NB and the scores were computed separately on sentence pairs then averaged to obtain per-condition values.  For TS 103 106 [5], the NB operational mode was selected and scores were computed on individual sentences and then averaged to obtain per-condition values.

#### 5.4.2.2 Results

The results of this experiment are shown below, for four noise types:

MOS

MOS

MOS

MOS

Figure 2:  Results of Experiment 1: comparing scores from P.835 [4] and P.862.1 [9] in 4 noise types

It can be seen from these graphs that P.862.1 [9] significantly underestimates the performance of the various handsets when compared to the results obtained from the P.835 [4] test with human listeners. In addition the crosing of the lines shows that the results obtained using P.862.1 [9]  do not preserve the rank order that is obtained with human listeners. For example handset D is the top or almost top performing handset for all noise types when human listeners are used, with a score of fair to good, however when tested with P.862.1 [9], it is the 4th  or 5th ranking handset in 3 of the noise types with a score of only poor.

Figure 3 shows a scatter plot of the subjective P.835 [4] OVRL ratings against objective scores.  The red-colored symbols are for P.862.1 [9] while the blue-colored symbols are for the GMOS output from the P.835 [4] predictor of TS 103 106 [5].  As noted above, the range of the P.862.1 [9] predictions is compressed relative to the range of subjective ratings.  For example, subjective ratings in the range of 3.5, or mid-way between 3 “Fair” and 4 “Good” are predicted by P.862.1 [9]  in the range of 2 “Poor”.   In contrast, for GMOS, the predicted scores span the full range of the subjective ratings, and fall very close to the diagonal line of slope 1.

Figure 3:  Results of Experiment 1 - correlation of P.835 [4] with P.862.1 [9] and TS 103 106 [5] (NB).  Absolute maximum error for P.862.1 [9] is 1.630, absolute maximum error for TS 103 106 [5] is 0.880.  Spearman rank-order correlation, accounting for 95% confidence interval, for P.862.1 [9] is 0.914 and for TS 103 106[5] is 0.984.

### 5.4.3 Experiment 2 – Problems with tuning for P.862.1

### 5.4.3.1 Setup

In Experiment 2, a recording of American English speech consisting of four sentences from each of two male and two female native talkers, as used in ETSI TS 103 106 [5], was reproduced through an equalized artificial mouth. The active speech level was -1.7 dBPa at the MRP of HATS.  The background noise generation method described in ETSI ES 202 396-1 [7] was used to reproduce eight noises described in Table 2d of Clause 7.12 of 3GPP TS 26.132 [8].  In this case, a single handset was used, but with noise suppression parameters adjusted in two ways.  The tuning labeled “Tuned for P.862.1” was defined so as to provide high values of ITU-T Recommendation P.862.1 [9].  The tuning labeled “Alternative Tuning” was defined in general accordance with the requirements of the marketplace, based on network operator requirements.  The device was mounted on the HATS in standard position, and recordings made of clean speech, and speech mixed with the eight noises described above, at the output of a UMTS base-station simulator with AMR speech encoding at 12.2 kbit/s.

A group of 32 naïve listeners, all native speakers of American English, using the ITU-T Recommendation P.835 [4] methodology, rated all sentences in all conditions in a partially balanced randomized blocks design, resulting in 128 votes per condition.

### 5.4.3.2 Results

Figure 4:  Results of Experiment 2 – P.835 [4] Overall Score

Figure 5:  Results of Experiment 2 – P.835 [4] Signal Score

Figure 6:  Results of Experiment 2 – P.835 [4] Background Score

The results for the overall performance in the P.835 [4] listening test of figure 4 show that tuning an algorithm to get the best P.862.1 [9] score does not produce an optimum result. The version of the algorithm tuned for P.862.1 [9] significantly underperforms in 6 of the 9 noise types. The graphs in figures 5 and 6 break down the performance in terms of the signal and background noise scores. These show that while tuning an algorithm to maximize the P.862.1 [9] score slightly improves the perceived quality of the speech signal, figure 5, it significantly degrades the perception of the background noise signal, figure 6, leading to the degraded overall score as shown figure 4.

### ---------end of modified clause--------

### ---------start of new clause--------

### 5.4.3 Experiment 3: WB P.835 v P.862.2, P.863 and TS 103 106

#### 5.4.3.1 Setup

The method used was very similar to the one described for narrow-band in 5.4.2.1. However, the UMTS base-station simulator was set to use speech encoding with AMR-WB at 12.65 kbit/s.

For computing metrics, all recordings were taken at 48 kHz sample rate.  For P.862.2 [10] the source file was filtered to WB and the scores were computed separately on sentence pairs then averaged to obtain per-condition values.  For P.863 [6], version 2.4 was used; the source was filtered to SWB and the scores were computed using the SWB mode, separately on sentence pairs then averaged to obtain per-condition values.  For TS 103 106 [5], scores were computed on individual sentences and then averaged to obtain per-condition values.

#### 5.4.3.2 WB Correlation Results

Figure 7 shows a scatter plot of the subjective P.835 OVRL [4] ratings against objective scores.  The red-colored (+) symbols are for P.862.2 (MOS-LQOw) [10], while the blue-colored (x) symbols are for the GMOS output from the P.835 [4] predictor of [5] and the green-colored (o) symbols are for P.863 Version 2.4 [6].

As for narrowband, the range of the P.862.2 [10] predictions is compressed relative to the range of the subjective ratings.  Subjective ratings of about 3.5, or midway between “Fair” and “Good” are scored by P.862.2 [10] at about 1.7, or below “Poor”, and the RMSE is 1.659.

For P.863 [6] (MOS-LQOs), there is some compression and offset of the predictions relative to the subjective ratings, but the compression is not as severe as for P.862.2 [10].  Subjective ratings of about 3.5 are predicted as about 2.0 by P.863, and the RMSE is 1.108.

The TS 103 106-WB [5]scores generally span the same range as the subjective ratings, and fall close to the reference line, without the compression observed in the P.862.2 [10] predictions.  The RMSE is 0.196; however for some cases, particularly at lower scores, the error can again be large. These larger errors are not unexpected, since during the training phase of TS 103 106 [5] there was relatively little data available with low scores.

Figure 7:	Correlation of P.835 Overall with P.862.2 [10] (MOS-LQOw), TS 103 106 [5] (WB) GMOSw and P.863 [6] (MOS-LQOs).  Maximum error of P.862.2 [10] is 2.192, of P.863 [6] is 1.459, and of TS 103 106 [5] is 0.424. Spearman rank-order correlation, accounting for 95% confidence interval, for P.862.2 [10] is 0.890, for P.863 [6] is 0.897, and for TS 103 106 [5] is 0.975.

#### 5.4.3.3 WB Rank Order Results

The graphs in figure 8 compare the scores of the various measures for each of the six phones (A-F), in each noise type.   Figure 9 then shows the number of absolute rank order errors for each metric in each noise type when compared to the results from the listening test. These results do not take account of overlapping confidence intervals; hence the errors may be exaggerated, especially for the clean condition where the listening tests results were very similar.

For P.862.2 [10] the rank order is not preserved, again there are frequent shifts of one and two positions, as well as shifts of three positions in cafeteria and car noise. For P.863 [6] rank order errors are observed, but not as many as for P.862.2 [10]. There are still frequent single rank switches, but fewer two-rank switches, although there are larger errors observed in the clean condition where the confidence intervals overlap. TS 103 106 [5] has the best performance; however single rank switches are still common and occasional two and three-rank switches also occur in clean, cafeteria and pub noise.

Figure 8:	Scores per device and noise type for P.835 Overall [4], P.862. 2[10] (MOS-LQOw), TS 103 106 [5] (WB) GMOSw and P.863 [6] (WB) (MOS-LQOw)

Figure 9:	Rank Order Errors for each Objective Measure in WB

### 5.4.4 Experiment 4 SWB P.835 v P.862.2, P.863 and TS 103 106

#### 5.4.4.1 Setup

The method used was similar to the one described in 5.4.2.1. However, the following differences should be noted. As commercial super-wideband terminals are not generally available, a mock- up of a handset was used.  The mock-up was the size and shape of a typical mobile handset, and was equipped with several microphones, as in some current commercially available wideband terminals.  The same speech and background noise generation as for NB and WB was used, but recordings were made from the microphones on the mock-up.  The signals were processed with offline processing to produce the noise-reduced signals.  No speech encoding was used.

For computing metrics, all recordings were taken at 48 kHz sample rate.  For P.862.2 [10], the source file was filtered to WB and the scores were computed separately on sentence pairs then averaged to obtain per-condition values.  For P.863 [6] version 2.4 was used; the source was filtered to SWB and the scores were computed separately on sentence pairs then averaged to obtain per-condition values.  For TS 103 106[5], the source file was used as SWB and scores were computed on individual sentences using the WB mode of the tool, and then averaged to obtain per-condition values.

#### 5.4.4.2 SWB Correlation Results

Figure 10 shows a scatter plot of the subjective P.835 [4] OVRL ratings against objective scores.  The red-colored (+) symbols are for P.862.2 (MOS-LQOw) [10], while the blue-colored (x) symbols are for the GMOS output from the P.835 predictor of [5] and the green-colored (o) symbols are for P.863 (ITU-T P.863) V2.4 [6].

As expected the P.862.2 [10] results show a poor correlation with the SWB listening scores with an RMSE of 1.458. SWB P.863 [6] scores are more consistent and have an RMSE of 0.815, however they show a similar compression of the range of scores as was observed for NB and WB.  Also as expected, the TS 103 106-WB [5] scores are less well correlated in SWB than in WB with an RMSE of 0.345.

Figure 10:	Correlation of P.835 Overall with P.862.2 [10](MOS-LQOw), TS 103 106 [5](WB) GMOSw and P.863 [6] (SWB) (MOS-LQOs).  Maximum absolute error for P.862.2 [10] is 2.463, for P.863 [6] is 1.326, and for TS 103 106 [5] is 0.842. Spearman rank-order correlation, accounting for 95% confidence interval, for P.862.2 [10] is 0.755, for P.863 [6] is 0.855, and for TS 103 106 [5] is 0.956.

#### 5.4.4.3 SWB Rank Order Results

The graphs in figure 11 compare the scores of the various measures for the mock-up phones with various levels of noise suppression (0-15), in each noise type.   Figure 12 then shows the number of absolute rank order errors for each metric in each noise type when compared to the results from the listening test.

None of the objective measures preserve the rank order of the listening tests for SWB. All three measures have frequent shifts of up to three ranks and occasional larger shifts. P.863 [6] also commonly shifts by four positions.

It can also be seen from the graphs in figure 11 that if these scores were used to try and tune the noise suppression parameter in this algorithm, different results would be obtained depending on which objective measure was used, and in addition none of the measures would reliably select the same optimization as that determined by the human listeners.

Figure 11:	Scores per device and noise type for P.835 [4] Overall, P.862.2 [10] (MOS-LQOw), TS 103 106 [5] (WB) GMOSw and P.863 [6] (SWB) (MOS-LQOs)

Figure 12:	Rank Order Errors for each Objective Measure in SWB

### ---------end of new clause--------

### ---------start of modified clause--------

### 5.4.5 Conclusions

The use of P.862.1 [9] and P.862.2 [10] for assessing the performance of handsets in noise should be avoided. Almost all modern UEs now provide noise suppression as part of their default operation and it is shown that P.862.1 [9] and P.862.2 [10] produces misleading results when used in conjunction with noise suppression algorithms. Using P.862.1 [9] and P.862.2 [10] to compare UE’s is unreliable since a comparison of the results with actual listening tests shows a different rank order between the two tests, i.e. the best P.862.1 [9] and P.862.2 [10] score does not always produce the best overall score from a listening test. The P.862.1 [9] and P.862.2 [10] scores (MOS_LQO) also substantially under predict the P.835 [4] OVRL scores obtained from the listening test. In addition, in Annex B of ETSI EG 202 396-3 [11], results for P.835 [4] listening tests are compared to predictions from P.862.1 [9] and P.862.2 [10] and are shown to not correlate well with the subjective results.

Finally, the use of P.862.1 [9] and P.862.2 [10] for comparing handsets may steer manufacturers to tune their algorithms to maximize the P.862.1 [9] and P.862.2 [10] score. For the reasons exposed, such tuning may actually degrade the speech quality as perceived by human listeners.

For WB it is not too surprising that the objective tools that are intended to predict the perceived quality of telephone speech with noise reduction (e.g. TS 103 106 [5]) perform that task better than tools which were not initially designed to do so.  However there is still room for improvement, as even TS 103 106 [5] does not preserve the rank order, which can make comparative evaluations unreliable.

For SWB none of the three predictors performed particularly well. Again this is to be expected since P.862.2 [10] and TS 103 106 [5] were not designed to be used on SWB speech, and P.863 [6] was not designed for use with modern terminal noise suppression algorithms.

Further work is needed to develop more effective objective measures especially for wider bandwidths. It is particularly important to ensure that maximum error and rank order are taken into account as well as just RMSE, which would enable more reliable comparative evaluations of solutions across a range of operation scenarios including different background noises and noise suppression technologies.

# References for section 5.4 of TR 26.931

[1] Recommendation ITU-T P.862, Perceptual evaluation of speech quality (PESQ): An objective method for end-to-end speech quality assessment of narrow-band telephone networks and speech codecs, 02/2001.

[2] Recommendation ITU-T P.800, Methods for subjective determination of transmission quality, 08/1996.

[3] Recommendation ITU-T P.862.3, Application guide for objective quality measurement based on Recommendations P.862, P.862.1 and P.862.2, 11/2007.

[4] Recommendation ITU-T P.835, Subjective test methodology for evaluating speech communication systems that include noise suppression algorithm, 11/2003.

[5] ETSI TS 103 106 v1.3.1 (2014-04), Speech quality performance in the presence of background noise: Background noise transmission for mobile terminals – objective test methods.

[6] Recommendation ITU-T P.863 (2014 -09), Perceptual objective listening quality assessment.

[7] ETSI ES 202 396-1 v1.4.1 (2014-01), Speech quality performance in the presence of background noise; Part 1: Background noise simulation technique and background noise database.

[8] 3GPP TS 26.132, V12.5.0 (2015-03), Speech and video telephony terminal acoustic test specification.

[9] Recommendation ITU-T P.862.1, Mapping function for transforming P.862 raw result scores to MOS-LQO, 11/2003.

[10] Recommendation ITU-T P.862.2, Wideband extensions to Recommendation P.862 for the assessment of wideband telephone networks and speech codecs, 11/2007.

[11] ETSI EG 202 396-3 v1.5.1 (2015-05), Speech quality performance in the presence of background noise; Part 3: Background noise transmission – Objective test methods.

### ---------end of modified clause--------