3GPP TSG-SA4 Meeting #90	 S4-160937

# Ljubljana, Slovenia, 5. - 9. September 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.445 | CR | 0023 | rev | - | Current version: | 13.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Give "NO_REQ" and "none" a clear definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | EVS |  |  |  |  |  |  | Date: |  | 2016-08-30 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The EVS-CMR value 0x7Fh = 127d, "NO_REQ" in Table A.3 has no clear definition. The same is true for the code point "none" (0x7h) in Table A.2 for the "Codec Mode Request (CMR) in Compact format". In RFC 4867, "RTP payload format ... for AMR", a similar code point for the AMR-CMR was defined "0xFh" = 15d, cite "CMR value 15 indicates that no mode request is present", which lead to substantial interworking problems, because different interpretations arose. Meanwhile an errata exist to this RFC 4867, stating "... if a terminal has no preference in which mode within the negotiated mode set to receive, it SHOULD set CMR=15 in all its outbound payloads". This errata corresponds to Interpretation 2 below. Interpretation 1 was: "CMR=15 means that no new Codec Mode Request value is present, so the most recent previous CMR value is still valid". With other words: CMR=15 shall be ignored. Most likely, this was the original intention. Interpretation 2 was: "CMR=15 means that there is no Codec Mode Restriction present, i.e. all modes of the mode-set can be used". It is unclear what benefit that interpretations would have, as it is redundant to CMR="highest mode of the selected mode-set". After a long debate, SA4 agreed in TS 26.114 and IETF agreed in an errata to RFC 4867, that interpretation 2 shall be the basis for AMR and AMR-WB and CMR=15 shall be interpreted as identical to the highest mode of the selected mode-set. The same problem arises for EVS, if TS 26.445 does not precisely define the meaning of NO_REQ in Table A.3 and the meaning of "none" in Table A.2. Note, that TS 26.454 specifies that CS networks and CS UEs shall not use this code-point NO_REQ (code-point "none" is anyway not used in CS). Note, that it is possible to have within one call-path different EVS Configurations, "Config 1" on one side of a MGW and "Config 2" on the other side, both TrFO-compatible to each other. The MGW maps CMRs passing through into the configuration of the outgoing termination. |  |  |  |  |  |  |  |  |
| Summary of change: |  | CMR code-point NO_REQ is specified as equivalent to no CMR-value being sent, i.e. no Codec Mode Request value is present in the received packet, i.e. equivalent to packets without the CMR byte: The receiver of "NO_REQ" shall ignore it. Note 1: This definition is according to Interpretation 1 and therefore not the same as agreed for AMR and AMR-WB. Interworking is, however, guaranteed by MGWs in the path, translating between these different interpretations. Note 2: Other specifications, e.g. TS 26.114 for MTSI or TS 26.454 for EVSoCS add rules, when not to send NO_REQ. Such rules are out of scope of TS 26.445, the generic, application-agnostic description for EVS. Similarly, the code-point "none" in Table A2 is specified with the same meaning and treatment as "NO_REQ": The receiver of "none" shall ignore it. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Substantial interworking problems must be expected, as the experience with CMR=15 for AMR and AMR-WB have shown. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2.1.2.1, A.2.2.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Begin of first change

#### A.2.1.2.1	Representation of Codec Mode Request (CMR) in Compact format for EVS AMR-WB IO mode

The 3-bit CMR field carries the codec mode request information to signal to the media-sender the requested AMR-WB [37] or EVS AMR-WB IO codec mode to be applied for encoding. The signallings of AMR-WB and EVS AMR-WB IO with the 3-bit CMR field are defined as shown in Table A.2. The CMR field in Compact format for EVS AMR-WB IO mode comprises a 3-bit element [c(0), c(1), c(2)] for signalling codec mode requests for the following EVS AMR-WB IO or AMR-WB codec modes.

Table A.2: 3-bit signalling element and EVS AMR-WB IO/AMR-WB CMR

| C(0) C(1) C(2) | Requested Mode |
| --- | --- |
| 0 0 0 | 6.6 |
| 0 0 1 | 8.85 |
| 0 1 0 | 12.65 |
| 0 1 1 | 15.85 |
| 1 0 0 | 18.25 |
| 1 0 1 | 23.05 |
| 1 1 0 | 23.85 |
| 1 1 1 | none |

Due to the 3-bit limitation, there is not enough signalling space for all EVS AMR-WB IO codec modes. Consequently, CMRs in Compact format for EVS AMR-WB IO are limited to include the most frequently used set of EVS AMR-WB IO /AMR-WB modes as shown in Table A.2. CMRs for EVS AMR-WB IO / AMR-WB modes 14.25 and 19.85 are not supported in Compact format for EVS AMR-WB IO. In case a request needs to be transmitted for either mode, it should be re-mapped to the next lower rate mode (12.65 and 18.25, respectively). Alternatively the Header-Full format may be used to transmit CMRs to 14.25 and 19.85 modes.

In case of restrictions in the allowed codec modes to an Active Codec Set (ACS) defined in TS 26.103 [39] or codec mode set (e.g. Config-WB-Code=0 used in TS 26.114 [13]), the mode shall be re-mapped to the next lower rate mode in this ACS. Codec mode requests for EVS primary modes shall be made using the Header-Full format.

CMR code-point "none" is specified as equivalent to no CMR-value being sent. The receiver of "none" shall ignore it.

NOTE:	The meaning of "none" and "NO_REQ" (see A.2.2.1.1 below) for EVS is not equivalent to code-point "CMR=15" for AMR and AMR-WB, as specified according to TS 26.114 and RFC 4867 with its errata. MGWs in the path, repacking between the RTP format according to RFC 4867 and the RTP format according to the present document, translate between these code-points.

Begin of second change

#### A.2.2.1.1	CMR byte

Codec Mode Request (CMR) byte structure is shown in Figure A.4. This CMR byte shall be present for EVS AMR-WB IO speech and SID frames in Header-Full format. For EVS Primary mode, the CMR byte is only used when a CMR needs to be transmitted or if required by session negotiation. The request indicated in the CMR byte shall comply with the media type parameters (e.g. allowed bit-rates or audio bandwidths) that are negotiated in the session.

The bit-rate in the CMR byte of EVS Primary mode indicates the highest bit-rate that the receiver wants to receive. The sender should use the bit-rate requested in the CMR and shall not use a higher bit-rate. The sender may use a lower bit-rate than the requested bit-rate within the set of negotiated bit-rates.

If a single audio bandwidth is negotiated for EVS Primary mode, the CMR shall indicate this bandwidth unless the mode is switched between EVS Primary and EVS AMR-WB IO.

If a range of audio bandwidths is negotiated for EVS Primary mode, the bandwidth in the CMR byte of EVS Primary mode indicates the highest bandwidth that the receiver wants to receive. The sender should use the bandwidth requested in the CMR. The sender may use a lower bandwidth within the set of negotiated bandwidths.

When a CMR is received, requesting a bit-rate and/or audio bandwidth that does not comply with the negotiated media parameters, it shall be ignored.

The request in the received CMR is valid until a new request is received.

Figure A.4. CMR byte

H (1 bit): Header Type identification bit. For the CMR byte this bit is always set to 1.

T (3 bits): These bits indicate the Type of Request in order to distinguish EVS AMR-WB IO and EVS Primary bandwidths.

D (4 bits): These bits indicate the codec mode request.

The possible values and corresponding CMRs are defined in Table A.3.

Table A.3: Structure of CMR

| Code |  | Definition |  | Code |  | Definition |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T | D |  |  | T | D |  |  |
| 000 | 0000 | NB | 5.9 (VBR) | 010 | 0000 | WB | 5.9 (VBR) |
|  | 0001 | NB | 7.2 |  | 0001 | WB | 7.2 |
|  | 0010 | NB | 8.0 |  | 0010 | WB | 8 |
|  | 0011 | NB | 9.6 |  | 0011 | WB | 9.6 |
|  | 0100 | NB | 13.2 |  | 0100 | WB | 13.2 |
|  | 0101 | NB | 16.4 |  | 0101 | WB | 16.4 |
|  | 0110 | NB | 24.4 |  | 0110 | WB | 24.4 |
|  | 0111 |  | Not used |  | 0111 | WB | 32 |
|  | 1000 |  | Not used |  | 1000 | WB | 48 |
|  | 1001 |  | Not used |  | 1001 | WB | 64 |
|  | 1010 |  | Not used |  | 1010 | WB | 96 |
|  | 1011 |  | Not used |  | 1011 | WB | 128 |
|  | 1100 |  | Not used |  | 1100 |  | Not used |
|  | 1101 |  | Not used |  | 1101 |  | Not used |
|  | 1110 |  | Not used |  | 1110 |  | Not used |
|  | 1111 |  | Not used |  | 1111 |  | Not used |
| 001 | 0000 | IO | 6.6 | 011 | 0000 |  | Not used |
|  | 0001 | IO | 8.85 |  | 0001 |  | Not used |
|  | 0010 | IO | 12.65 |  | 0010 |  | Not used |
|  | 0011 | IO | 14.25 |  | 0011 | SWB | 9.6 |
|  | 0100 | IO | 15.85 |  | 0100 | SWB | 13.2 |
|  | 0101 | IO | 18.25 |  | 0101 | SWB | 16.4 |
|  | 0110 | IO | 19.85 |  | 0110 | SWB | 24.4 |
|  | 0111 | IO | 23.05 |  | 0111 | SWB | 32 |
|  | 1000 | IO | 23.85 |  | 1000 | SWB | 48 |
|  | 1001 |  | Not used |  | 1001 | SWB | 64 |
|  | 1010 |  | Not used |  | 1010 | SWB | 96 |
|  | 1011 |  | Not used |  | 1011 | SWB | 128 |
|  | 1100 |  | Not used |  | 1100 |  | Not used |
|  | 1101 |  | Not used |  | 1101 |  | Not used |
|  | 1110 |  | Not used |  | 1110 |  | Not used |
|  | 1111 |  | Not used |  | 1111 |  | Not used |

Table A.3: Structure of CMR (continued)

| Code |  | Definition |  | Code |  | Definition |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T | D |  |  | T | D |  |  |
| 100 | 0000 |  | Not used | 110 | 0000 | SWB | 13.2 CA-L-O2 |
|  | 0001 |  | Not used |  | 0001 | SWB | 13.2 CA-L-O3 |
|  | 0010 |  | Not used |  | 0010 | SWB | 13.2 CA-L-O5 |
|  | 0011 |  | Not used |  | 0011 | SWB | 13.2 CA-L-O7 |
|  | 0100 |  | Not used |  | 0100 | SWB | 13.2 CA-H-O2 |
|  | 0101 | FB | 16.4 |  | 0101 | SWB | 13.2 CA-H-O3 |
|  | 0110 | FB | 24.4 |  | 0110 | SWB | 13.2 CA-H-O5 |
|  | 0111 | FB | 32 |  | 0111 | SWB | 13.2 CA-H-O7 |
|  | 1000 | FB | 48 |  | 1000 |  | Not used |
|  | 1001 | FB | 64 |  | 1001 |  | Not used |
|  | 1010 | FB | 96 |  | 1010 |  | Not used |
|  | 1011 | FB | 128 |  | 1011 |  | Not used |
|  | 1100 |  | Not used |  | 1100 |  | Not used |
|  | 1101 |  | Not used |  | 1101 |  | Not used |
|  | 1110 |  | Not used |  | 1110 |  | Not used |
|  | 1111 |  | Not used |  | 1111 |  | Not used |
| 101 | 0000 | WB | 13.2 CA-L-O2 | 111 | 0000 | Reserved |  |
|  | 0001 | WB | 13.2 CA-L-O3 |  | 0001 | Reserved |  |
|  | 0010 | WB | 13.2 CA-L-O5 |  | 0010 | Reserved |  |
|  | 0011 | WB | 13.2 CA-L-O7 |  | 0011 | Reserved |  |
|  | 0100 | WB | 13.2 CA-H-O2 |  | 0100 | Reserved |  |
|  | 0101 | WB | 13.2 CA-H-O3 |  | 0101 | Reserved |  |
|  | 0110 | WB | 13.2 CA-H-O5 |  | 0110 | Reserved |  |
|  | 0111 | WB | 13.2 CA-H-O7 |  | 0111 | Reserved |  |
|  | 1000 |  | Not used |  | 1000 | Reserved |  |
|  | 1001 |  | Not used |  | 1001 | Reserved |  |
|  | 1010 |  | Not used |  | 1010 | Reserved |  |
|  | 1011 |  | Not used |  | 1011 | Reserved |  |
|  | 1100 |  | Not used |  | 1100 | Reserved |  |
|  | 1101 |  | Not used |  | 1101 | Reserved |  |
|  | 1110 |  | Not used |  | 1110 | Reserved |  |
|  | 1111 |  | Not used |  | 1111 | NO_REQ |  |

CMR code-point "NO_REQ" is specified as equivalent to no CMR-value being sent. The receiver of "NO_REQ" shall ignore it.

NOTE:	The meaning of "NO_REQ" and “none” (see A.2.1.2.1 above) for EVS is not equivalent to code-point "CMR=15" for AMR and AMR-WB, as specified according to TS 26.114 and RFC 4867with its errata. MGWs in the path, repacking between the RTP format according to RFC 4867 and the RTP format according to the present document, translate between these code-points.

End of changes