3GPP TSG-S4 Meeting #92 	S4-170294

# Busan, Korea, 24-28 April 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.346 | CR | 0576 | rev |  | Current version: | 14.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on Transport Delivery Method |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics Co., Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AE_enTV-S4 |  |  |  |  |  |  | Date: |  | 2017-04-18 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As part of the enhanced TV work item, the delivery to TV services will require the support for a transparent delivery mode that forwards the TV service streams as is, for consumption at the receiver by a 3rd party application. Currently, there is no support for this transparent delivery that integrates well with the xMB interface and MBMS API. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces a new delivery method, the Transport delivery method that will enable the transport of TV streams |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1, 4.2, 6, 8B, 8.3.1.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First Change

## 4.1	MBMS functional layers

Three distinct functional layers are defined for the delivery of MBMS-based service. They are Bearers, Delivery method and User service. Figure 1 depicts these layers with examples of bearer types, delivery methods and applications.

User Service/
Application

Delivery Method

Bearer

PSS

MMS

GCS

ATSC

Other

Streaming

Download

Group Communication

Transport

Unicast Bearer

MBMS Bearer

Bearers:	Bearers provide the mechanism by which IP data is transported. MBMS bearers as defined in 3GPP TS 23.246 [4] and 3GPP TS 22.146 [3] are used to transport multicast and broadcast traffic in an efficient one-to-many manner and are the foundation of MBMS-based services. MBMS bearers may be used jointly with unicast PDP contexts in offering complete service capabilities.

Delivery Method:	When delivering MBMS content to a receiving application one or more delivery methods are used. The delivery layer provides functionality such as security and key distribution, reliability control by means of forward-error-correction techniques and associated delivery procedures such as file-repair, delivery verification. Four delivery methods are defined, namely download, streaming, transport, and group communication. Delivery methods may be added beyond the current release. Delivery methods may use MBMS bearers and may make use of point-to-point bearers through a set of MBMS associated procedures.

User service:	The MBMS User service enables applications. Different applications impose different requirements when delivering content to MBMS subscribers and may use different MBMS delivery methods. As an example a messaging application such as MMS would use the download delivery method while a streaming application such as PSS would use the streaming delivery method, and a group communications application such as MCPTT would use the group communication delivery method.

Figure 1: Functional Layers for MBMS User Service

Second Change

## 4.2	MBMS User Service Entities

Figure 2 shows the MBMS user service entities and their inter-relations. Relation cardinality is depicted as well.

Figure 2: Entities and Relations

An MBMS user service is an entity that is used in presenting a complete service offering to the end-user and allowing him to activate or deactivate the service. It is typically associated with short descriptive material presented to the end-user, which would potentially be used by the user to decide whether and when to activate the offered service.

A single service entity can contain multiple distinct multimedia objects, streams, or object flows, which may need to be provided over various MBMS download session, MBMS streaming session, MBS transport session, or MBMS group communication delivery method. A download session or a streaming session is associated with either a unicast bearer or one or more MBMS bearers and a set of delivery method parameters specifying how content is to be received on the mobile side. A delivery method for the carriage of a group communication service through the MBMS system is associated with one or more MBMS bearers. A transport session is associated with exactly one MBMS bearer. The MBMS User Service Session may be mapped either on MBMS Bearer Services or on unicast bearer services.

A set of one or more MBMS bearers can be used for delivering data as part of an MBMS download or streaming session. As an example, the audio and visual parts can be carried on separate MBMS bearers. However, it is recommended to transfer MBMS download and/or streaming sessions, which belong to the same MBMS user service on the same MBMS bearer service.

An MBMS bearer service (identified by TMGI) may be used to transport data for one or more MBMS download, streaming, MBMS transport, or GC sessions (3GPP TS 22.246 [3], clause 5).

Third Change

# 6	Introduction on Delivery Methods

Four delivery methods are defined in the present document - the download delivery method, the streaming delivery method, the transport delivery method and the group communication delivery method. MBMS delivery methods make use of MBMS bearers for content delivery but may also use the associated procedures defined in clause 9.

Use of MBMS bearers by the download delivery method is described in clause 7. The File Repair Procedure and the Reception Reporting Procedure (described in clause 9) may be used by the download delivery method.

Use of MBMS bearers by the streaming delivery method is described in clause 8.

Use of MBMS bearers by the group communication delivery method is described in clause 8A.

Use of MBMS bearers by the transport delivery method is described in clause 8B.

Fourth Change

# 8B	Transport Delivery Method

## 8B.1	Introduction

The MBMS transport delivery method delivers a UDP/IP packet flow to the UE. A single MBMS bearer is used for the multicasting of the UDP/IP packet flow. The BM-SC acts as a pass through by receiving UDP/IP packets and forwarding them to the destination multicast IP address and port number. Both IPv4 and IPv6 may be used by the transport delivery method.

The following figure depicts the reference model for packet ingestion in the BM-SC.

Figure 8B-1 Reference Model for BM-SC Transport Delivery

The transport protocol on top of UDP/IP is transparent to the MBMS user service and is not defined in this specification.

A Transport Framing mechanism as defined in section 8B.2 shall be used. It allows for the addressing of UDP datagrams, the detection of missing packets, and for maintining the end-to-end MBMS system delay. \

## 8B.2	Transport Framing

The Transport framing protocol stack is depicted in the following figure:

MBMS transport

UDP

UDP

Multicast IP

Transport Framing

Transport Framing

Application

Application

Buffer Model

QoS Measurement

Sender

Receiver

Figure 8B-2. Architecture and Protocol Stack for Transport Framing

The BM-SC shall encapsulate the user plane UDP datagram payload in an IP/UDP multicast packet where the payload is preceded by the Transport Framing header.

Service Payload

…

NTP Short Format Timestamp

Packet Sequence Number

UDP header

IP Multicast header

Transport

Framing

Header

Figure 8B-3 Transport Framing Header

The packet sequence number is a 32 bit integer that starts with a random offset for the first packet in the transport session and is incremented by 1 for each following packet. More precisely, the sequence number space is shared among all UDP/IP packets that share the same source and destination IP addresses irrespective of the source and destination port numbers.

The NTP short format timestamp is a timestamp that indicates the UTC timestamp at the time of transmission of this packet by the BM-SC. The format is defined in RFC 5905.

The UDP checksum shall be recalculated after the transport framing is performed.

## 8B.3 	Transport Buffer Model

The BM-SC shall use the transport buffer model as defined in this section to ensure conformance of the transport stream to the requested QoS parameters and the device requirements. The following figure depicts the transport buffer model:

Transport Buffer

Pkt (#n,t)

Pkt

Pkt

Pkt

Figure 8B-4. Transport Buffer

The transport buffer operation is as follows:

- The transport buffer (TB) is initially empty

- Define max_buffer_size and min_buffer_delay (< max allowed delay) as the QoS parameters provided by the content provider

- For incoming packet with timestamp ts, size s, and sequence number n

  - if size(TB)+s<max_buffer_size, insert packet into transport buffer

  - otherwise, discard packet n

- for each packet pkt in buffer TB,

  - if timestamp(pkt)+min_buffer_delay>=now, remove and forward packet pkt

  - Goto step 3

## 8B.4	Session Description for Transport Delivery Method

In the Transport delivery method, the BM-SC shall act as the source for the multicast traffic. The SDP for the transport delivery method shall be created by the BM-SC and may be shared as a fragment of the service announcement or sent to the content provider, if the latter selects to perform service announcement by itself.

### 8B.4.1 SDP Parameters

The SDP shall contain at least the following SDP parameters:

- The destination IP address and port number

- The protocol ID for each media session

- The TMGI of the MBMS Bearer

- The maximum delay tolerance attribute

- The requested QoE reporting as defined in section 8.3.2.1 as a media level attribute

#### 8B.4.1.1	Destination IP address and port number for channels

The IP destination address shall be defined using the "connection data" field ("c=") of [14]. The destination port number shall be defined according to the <port> sub-field of the media announcement field ("m=") of [14].

In case multiple media sessions are present, all of them shall share the same destination multicast IP address. The “c=” parameter shall be a session level attribute.

#### 8B.4.1.2	Protocol Identifier

The protocol identifier shall be provided for each session in the <proto> field as part of the “m=” line. The actual protocol over UDP or over UDP/transport framing is transparent to the MBMS service and the UE should ensure that it supports the protocol before starting the consumption of the MBMS service.

The UE shall use the protocol identifier to detect the usage of the transport framing and remove the transport framing header prior to forwarding it to the application.

#### 8B.4.1.3	MBMS Bearer Information

The SDP shall contain exactly one MBMS bearer mode attribute (defined in section 7.3.2.7) at session level. It may contain an alternative TMGI attribute as defined in section 7.3.2.12.

#### 8B.4.1.4	Maximum Allowed Delay

The “max-allowed-delay” is a media level SDP attribute that informs the receiver about the maximum allowed delay from the time of transmission of the UDP datagram at the BM-SC to the time of passing it to the receiving application by the MBMS client on the UE. This time sets an upper boundary on end-to-end delay in the MBMS system, including any operations at the MBMS client. This value is indicated in milliseconds and is a session level parameter that applies to all media sessions.

#### 8B.4.1.5 	Minimum Buffer Size

The “minimum-buffer-size” provides the required minimum size of the transport buffer in bytes. The UE shall provide a buffer of at least this size for receiving and processing the packets from the transport session before forwarding to the application. This parameter can be a session level or media level parameter and applies to all packets that share the same source IP address and destination IP address.

Fifth Change

#### 8.4.1.11	UDP Datagram Loss

The metric “UDP_Datagram_Loss” indicates the number of consecutive UDP datagram losses that are detected through a gap in transport framing packet sequence number. This metric is only applicable to the Transport Delivery method.