TSG SA4#93 meeting	Tdoc S4-170373

24-28 April, Busan, Republic of Korea	Agenda Item: 5.3

3GPP TSG CT4 Meeting #77	C4-172170

Spokane, US; 03rd – 07th April 2017

Title:	Reply LS on Clarification on Rel-14 MMCMH_Enh features

Response to:	LS C4-172092 / S4-170220 on Clarification on Rel-14 MMCMH_Enh features from SA4

Release:	Release-14

Work Item:	Enhancements to Multi-stream Multiparty Conferencing Media Handling

Source:	3GPP CT WG4

To:	3GPP SA WG4

Cc:

Contact Person:

#### Name:	Nevenka Biondic

E-mail Address:	Nevenka dot Biondic at ericsson dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

CT4 thanks SA4 for their incoming LS (S4-170220) on "Clarification on Rel-14 MMCMH_Enh features" and for

sharing with CT4 the MMCMH related CRs to TS 26.114 and TR 26.980.

CT4 has reviewed the CRs attached to LS (S4-170220) and would like to get clarifications on the following text from S4-170202 (CR 26.114-0404), Clarification of Thumbnail Handling in MMCMH:

"A thumbnail "m="-line is generally not dedicated to a certain conference participant and the number of "m="-lines for thumbnails therefore need not match the number of conference participants. If RTP stream selective forwarding (see clause S.6) is used for thumbnails by the MSMTSI MRF, though the forwarded participant may change at any point in time, a single thumbnail "m="-line can map to an RTP stream from any one of the participants. The number of thumbnail "m="-lines actually used between an individual MSMTSI client in terminal and an MSMTSI MRF is thus decided by the minimum number of thumbnail "m="-lines in any of their SDPs, irrespective of the MSMTSI client in terminal or the MSMTSI MRF being the entity supporting the fewest thumbnail "m="-lines."

- CT4 agrees with SA4 that the number of thumbnail "m="-lines does not need to match the number of conference participants.

- CT4 assumes that the number of thumbnail "m="-lines supported by an individual MSMTSI client in terminal should not limit the number of thumbnail "m="-lines used between an MSMTSI MRF and other MSMTSI client in terminals participating in a conference.
However, this is not clear from the above text.

CT4 kindly asks SA4 to confirm if CT4 assumption is correct and possible to consider further clarification of requirements related to the handling of thumbnail videos.

2. Actions:

To SA4 group.

ACTION:	CT4 kindly asks SA4 to answer the above CT4's question and to consider whether related updates in SA4 TS 26.114 is required.

3. Date of Next CT and CT4 Meetings:

CT4#78	15th – 19th May 2017		Zhangjiajie, P.R. China

CT#76	06th – 07th June 2017		West Palm Beach, US

CT4#79	21st – 25th August 2017		Krakow, Poland