3GPP TSG SA4#96	S4-171233

November 13 – 17, 2017, Albuquerque, New Mexico, USA

Agenda item: 	11.6

Source: 	Fraunhofer HHI, Nokia Corporation

Title: 	VRStream: Client Reference Architecture

Document for	Approval

# Introduction

During SA4#94 the New Work Item “3GPP Virtual Reality Profiles for Streaming Media (VRStream)” in S4-170751 was agreed and afterwards approved in by SA plenary #77 in SP-170612.

One of the objectives of this work item is as follows:

- Document content generation and content consumption guidelines, taking into account a reference client architecture that provides: (a) the signalling and processing steps for download delivery, (b) PSS delivery, (c) interfaces between the VR service platform, the VR application (e.g. sensors), and the VR rendering system (displays, GPU, loudspeakers).

This document primarily addresses Section 4.3 (Client Reference Architecture) of TS 26.118 and aims to define a DASH streaming client model and a download client model.

************************************  START OF CHANGES ***************************************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 26.918: "Virtual Reality (VR) media services over 3GPP".

[3GP]	3GPP TS 26.244: "Transparent end-to-end packet switched streaming service (PSS); 3GPP file format (3GP)".

[3GP-DASH]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[DASH]	Information technology – Dynamic adaptive streaming over HTTP (DASH) – Part 1: Media presentation description and segment formats, including ISO/IEC 23009-1:2014/Amd.4:2016 (ISO/IEC 23009-1)

[ISOBMFF]	Information technology – Coding of audio-visual objects – Part 12: ISO Base Media File Format (ISO/IEC 14496-12)

[NAL]	Information technology – Coding of audio-visual objects – Part 15: Carriage of network abstraction layer (NAL) unit structured video in the ISO base media file format (ISO/IEC 14496-15:2017)

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

3DOF	3 Degrees of freedom

6DOF	6 Degrees of freedom

DASH	Dynamic Adaptive Streaming over HTTP

ERP	Equirectangular projection

FOV	Field of view

HMD	Head Mounted Display

HOA	High Order Ambisonics

HRTF	Head-related transfer function

ISO BMFF	ISO Base Media File Format

MPD	Media Presentation Description

OMAF	Omnidirectional MediA Format

SRQR	Spherical Region-wise Quality Ranking

SSR	Spherical Spatial Relationship

VR	Virtual Reality

# 4	Architectures and Interfaces for Virtual Reality

## 4.1	Definitions and Reference Systems

<Creates the basics on VR, coordinate systems, rendering aspects, media components, spatial alignment>

## 4.2	End-to-end Architecture

<end-to-end architecture – aligned with Fig 4.24 in TR26.918>

## 4.3	Client Reference Architecture

<Streaming client, file format client, application, decoder, rendering, interfaces/APIs>

This section provides an overview of the DASH streaming client model as well as Download client model and describes their components and interfaces.

DASH Streaming client model

Figure 4.X shows a high-level structure of OMAF-DASH client model with interfaces for streaming. The client model consists of 5 submodules: 3GP-DASH access engine, VR application, media engine, renderer and sensor data.

The 3GP-DASH access engine downloads all OMAF media streams and fetches the Media Presentation Description (MPD). The 3GP-DASH access engine is connected to a VR application module, which based on VR-related metadata as provided in the MPD instructs the 3GP-DASH access engine on the adaptation sets selected for requests of media segments. The selected adaptation sets can be based on descriptors defined in OMAF, such as the spherical spatial relationship (SSR) descriptor indicating the coverage information of the omnidirectional video or in case of viewport-dependent streaming spherical region-wise quality ranking (SRQR) descriptor indicating quality rankings of sphere regions of the omnidirectional video. A player is recommended to parse the SRQR descriptors and select the Adaptation Sets and Representations that match the user's viewing orientation. When Preselections are applied, the player is expected to parse the SRQR descriptor of the Main Adaptation Set and when @dependencyId is applied the player is expected to parse the SRQR descriptor of the dependent Representation.

NOTE:  Adaptation Sets and Representations that match the user's viewing orientation could be derived from the SRQR descriptor when the quality ranking value on the region covering the viewport is greater than 0 and less than that for other regions or the resolution of the region covering the viewport is suitable for the display.

The output of 3GP-DASH access engine consists of a single file in media container formats according to ISO Base Media File Format [ISOBMFF] and specifically the 3GP file format [3GP]. Additionally, timing information is provided that maps the timing of the media to timeline of the Media Presentation [3GP-DASH].

The single file (OMAF conforming media stream) is input to the 3GP-compliant file parser, as a result of an adaptive streaming process and from a concatenation of DASH Segments/Subsegments. For the viewport-dependent streaming case, @dependencyId or Preselection may be used and, in that case, the DASH Segment/Subsegment concatenation may consist of DASH Segments/Subsegments of several Representations as specified in [3GP-DASH] for dependent and complementary representations.

NOTE:  Preselections are specified in [DASH]. Changes to TS 26.247 [3GP-DASH] are necessary to enable Preselection functionality in 3GPP.

The OMAF conforming media stream is processed by the 3GP-compliant file parser. Rendering metadata that is present in the media stream and replicated at file format level is extracted and forwarded to the Rendering function, i.e. texture to sphere mapping. The function generates the described 3D/spherical signal. The sensor viewpoint information is then used to generate the actually rendered view. The VR application controls the rendering depending on the sensor data and HMD capabilities. OMAF describes the metadata to translate from the decoder output texture information to a 3D/spherical video.

The elementary media stream is decoded by the media decoder that outputs decoded pictures and, potentially, metadata into Renderer. In some environments, elementary stream metadata may not be output and be accessible. The same metadata is also available as file format metadata, being alternatively available and used for such environments. Based on this information, a spherical content can be generated dynamically for each eye using the information about the desired viewport.

Figure 4.X. OMAF-DASH Streaming client model.

Note 1: Changes to TS 26.244 [3GP] are necessary to enable the OMAF functionality in 3GPP.

Note 2:  OMAF refers to "media engine" as "file decoder". Both terms are equivalent but "media engine" is kept as this is generally used for DASH streaming.

Download client model

Figure 4.Y shows a high-level structure of OMAF-Download client model with interfaces. The client model consists of 5 submodules: 3GP-Download engine, VR application, media engine, renderer and sensor data.

In the case of viewport-dependent file download (shown in Figure 4.Y with dashed arrows and boxes), the main difference to the OMAF-DASH Streaming client model in Figure 4.X is that no subsegment concatenation is carried out since the download file contains all required tracks. Tracks can be distinguished by file format level metadata indicating regions of the omnidirectional video at different qualities (e.g. different resolutions). Also, the VR application has an interface to the Track Selection function (1) indicating the current user viewport to select which track from the file (2) to play back (3) based on the sensor data and the described metadata. Such a viewport-dependent case can be achieved using extractor tracks selected dynamically based on the current user viewport that extracts the current samples from referenced tracks of corresponding quality/resolution. The entire 3GP file is passed to the 3GP file parser (2) where only the selected extractor track is parsed and a single corresponding HEVC Main 10 level 5.1 elementary stream (4) is passed to the Media Decoder.

NOTE: The ISO BMFF Extractor is specified in [NAL] and contains all aggregation rules for reconstruction of a single HEVC Main 10 level 5.1 elementary from multiple Motion Constrained Tile Set (MCTS) tracks. Changes to TS 26.244 [3GP] are necessary to enable the extractor functionality in 3GPP.

In the case of viewport-independent delivery, the entire 3GP file is parsed and a single corresponding HEVC Main 10 level 5.1 elementary stream (4) is passed to the Media Decoder.

The elementary media stream is decoded by the media decoder that outputs decoded pictures and, potentially, metadata into Renderer. In some environments, elementary stream metadata may not be output and be accessible. The same metadata is also available as file format metadata, being alternatively available and used for such environments. Based on this information, a spherical content can be generated dynamically for each eye using the information about the desired viewport.

Figure 4.Y. OMAF-Download Client model. Dashed arrows and box indicate necessary elements for viewport-dependent delivery.

Note 1: Changes to TS 26.244 [3GP] are necessary to enable the OMAF functionality in 3GPP.

Note 2:  OMAF refers to "media engine" as "file decoder". Both terms are equivalent but "media engine" is kept as this is generally used for DASH streaming.

************************************  END OF CHANGES ***************************************