3GPP TSG-SA5 (Telecom Management)	S5eHNB0021

3GPPSA5-Virtual Meeting on HeNB

- Source:	Verizon

- Title:	System Parameters for HeNB

- Document for:	Discussion and Approval

Agenda Item:

# 1	Decision/action requested

The group is asked to discuss the proposed high level system parameter, and introduce the agreed part into the draft TS 32.592 for HeNB

# 2	References

[1] 3GPP TS 32.582

[2] 3GPP TS 32.592

[3] TR-196

# 3	Rationale

The Home ENodeB is expected to be a complex entity when compared to a typical home router. The Home eNodeB being an entity in a subscriber’s home is expected to provide atleast the same level of reliability and service availability as a device typically found in a networked home. (E.g. a Broadband home router, DSL modem etc). Thus, from a service provider’s point of view, the Home eNodeB entity deployment requires the remote management of the device beyond the typical configuration and black box treatment. In other words, a service provider expects the remote management of a Home eNodeB be comparable to currently deployed managed networked devices in homes.

The typical Home eNodeB is expected to have multiple sub-systems that work together. This document proposes that the level of detail in the parameter categories include a generic system and sub-system level information that can be used by service providers.

Typical FemtoCells have multiple HW/SW subsystems and SW stacks on the HeNodeB. Service providers are particularly interested in information at a sub-system level of home networked devices. Hence the parameters in the info model should provide the ability for an insight into the system and subsystems of a Home eNodeB.

The standardization of these parameters is important to service providers from a multi-vendor interoperability perspective.

Start of Section

### 6.1.4	System

System Level Parameter types

| Parameter Type | Description | Notes |
| --- | --- | --- |
| Software Information | Information of all software images loaded/installed | n/a |
| Operational informatio | System operation information and history including information such as the current mode of operation e.g. diag mode, debug mode, release mode (read only), reset history, | n/a |
| LED status | Information related to user side display of LED entities including string description of LED label(if applicable) (read only) | n/a |
| Interface information | Information related to the sub-system interfaces e.g. Processor <-> modem interface, modem<->radio interface etc | n/a |
| System flows | Information of all system flows(running and historical) e.g signalling, traffic, security, PDCP flows etc. | n/a |

### System Level Parameter Details

### 6.1.4.1	Software Information

| Parameter | Description | Notes |
| --- | --- | --- |
| Firmware name and version |  |  |
| OS name and version |  |  |
| Driver/Application list and version | e.g. Middleware/Libraries if applicable, DSP images, FPGA images etc |  |
| Upgrade/Rollback information | History of upgrades/rollbacks of software with timestamps |  |

### 6.1.4.2	Operational Information

| Parameter | Description | Notes |
| --- | --- | --- |
| Mode of operation | This entry informs whether the system is executing in a diagnostic mode,error or normal mode of operation |  |
| Last Boot up time | This parameter provides the last boot up timestamp |  |
| Operational time | This parameter provides the time taken from bootup to the time the system is operationally ready. |  |
| Reboot information | History of restarts/reboots with timestamps and resason & cause/trigger |  |
| Sub-system critical watermark | This parameter is a listing of all sub-system critical watermarks on the running system | Although alarmstake care of reporting critical conditions, this parameter set allows for reading critical watermarks of a device under investigation. |
| Sub-system critical watermark history | This parameter is a historical listing of all sub-system critical watermarks from bootup time | Although alarmstake care of reporting critical conditions, this parameter set allows for reading critical watermarks of a device under investigation. Also allows a service provider to quickly look at critical watermarks on a system under investigation instead of going through a large log file. |

### 6.1.4.3	LED Status

| Parameter | Description | Notes |
| --- | --- | --- |
| LED description | Listing of LED description as strings |  |
| LED state | Listing of LED current state ordered similarly as LED description. |  |

### 6.1.4.4	Interface information

| Parameter | Description | Notes |
| --- | --- | --- |
| Interface description |  |  |
| Interface operational state |  |  |

### 6.1.4.5	System Flows

| Parameter | Description | Notes |
| --- | --- | --- |
| System flow description | Description of currently active flows |  |
| System flow state | State of currently active flows |  |
| System flow history | Historical information of flows , creation time, deletion time, related statistics |  |

End of Section