# 3GPP TSG SA WG5 (Telecom Management) Meeting #105	S5-161102

# 25-29 January 2016, Bratislava (Slovakia)	revision of S5-16xabc

# Source:	Huawei

# Title:	Add requirements for NS lifecycle management

# Document for:	Approval

Agenda Item:	6.5.1.5

# 1	Decision/action requested

The group is asked to discuss and agree on the requirements.

# 2	References

# 3	Rationale

It is discussed in ETSI IFA that NM could indicate NFVO to use existing VNF instance for a new NS instantiation. In order to choose and designate a proper existing VNF instance, NM needs to know the information of exsting VNF instances, e.g. which NS instance a VNF instance is serving, identifier of a VNF instance, etc. And this information could be reported back to NM after instantiation of a NS instance.

# 4	Detailed proposal

### 6.1.2	Requirements for Os-Ma-nfvo

REQ-NFV_LCM_Os-Ma-nfvo-FUN-1	The Os-Ma-nfvo reference point shall have the capability allowing NM to send NS instantiation request based on on-boarded NSD.

REQ-NFV_LCM_Os-Ma-nfvo-FUN-2	The Os-Ma-nfvo reference point shall have the capability allowing NM to send NS instantiation request using existing VNF instances.

REQ-NFV_LCM_Os-Ma-nfvo-FUN-x	The Os-Ma-nfvo reference point shall have the capability allowing NFVO to send information of VNF instances that constitute a NS to NM after NS instantiation.