# 3GPP TSG SA WG5 (Telecom Management) Meeting #119	S5-183212

# 14-18 May 2018, La Jolla (US)

# Source:	China Mobile

# Title:	pCR 28.541 Add GNodeBFunction class attributes

# Document for:	Approval

Agenda Item:	6.4.3

1	Decision/action requested

It is asked to discuss and approve this contribution.

2	References

[1]	3GPP TS 28.541 Management and orchestration of networks and network slicing; NR and NG-RAN Network Resouce Model (NRM); Stage 2 and stage 3 v0.2.0

3	Rationale

Add attribute longitude in <IOC>GNodeBFunction currently defined in [1].

Add attribute latitude in <IOC>GNodeBFunction currently defined in [1].

In NR non-split deployment scenario, the longitude and latitude information of a gNB can be obtained from the GPS module of its gNB-DU. They can represent the location of the gNB.

4	Detailed proposal

1st Modified Section

4.3.1	GNodeBFunction

4.3.1.1	Definition

For more information about the gNB, see 3GPP TS 3GPP TS 38.401 [4].

4.3.1.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBId |  |  |  |  |  |
| longitude | O | M | M | - | M |
| latitude | O | M | M | - | M |

2nd Modified Section

4.4	Attribute definitions

4.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| NRCellDU.administrativeState | It indicates the administrative state of the NRCellDU. It describes the permission to use or prohibition against using the cell, imposed through the OAM services. allowedValues: "Locked", "Shutting down" or "Unlocked" The meaning of these values is as defined in ITU-T Recommendation X.731 [18]. See Append [n] for Relation between the “Pre-operation state of the gNB-DU Cell” and administrative state relevant in case of 2-split and 3-split deployment scenarios. | type: enumeration multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Locked isNullable: False |
| configuredMaxTxPower | This is the maximum possible for all downlink channels, used simultaneously in a cell, added together. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| fDDCenterFrequencyUL | Specifies the channel number for the central UL frequency. The mapping from channel number to physical frequency is described in 3GPP TS 38.104 [12] subclause 5.4.2. allowedValues: This value must be within one of the specified SectorEquipmentFunction.nRFqBands values. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| fDDCenterFrequencyDL | Specifies the channel number for the central DL frequency. The mapping from channel number to physical frequency is described in 3GPP TS 38.104 [12] subclause 5.4.2. allowedValues: This value must be within one of the specified SectorEquipmentFunction.nRFqBands values. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| fDDBandwidthUL | Specifies the frequency difference between the upper edge of the highest used carrier and the lower edge of the lowest used carrier. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| fDDBandwidthDL | Specifies the frequency difference between the upper edge of the highest used carrier and the lower edge of the lowest used carrier. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| localAddress | Local address including IP address used for initialization of the underlying transport. IP address can be an IPv4 or an IPv6 address. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| remoteAddress | Remote address including IP address used for initialization of the underlying transport. IP address can be an IPv4 or an IPv6 address. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBId | It identifies a gNB within a PLMN. See “gNB Identifier (gNB ID)” of subclause 8.2 of 3GPP TS 38.300 [3]). See “Global gNB ID” in subclause 9.3.1.6 of 3GPP TS 38.413 [5]. Editor’s Note: How to support flexible length for gNB ID is stated as TBD in TS 38.413. | type: BitString multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBDUId | It uniquely identifies the DU at least within a gNB. See ‘gNB-DU ID’ in subclause 6.2.2 of 3GPP TS 38.401 [4]. See ‘gNB-DU ID’ in subclause 9.3.1.9 of 3GPP TS 38.473 [8]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBCUName | It identifies the Central Entity of a NR node, see subclause 9.2.1.4 of 3GPP TS 38.473 [8]. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBDUName | It identifies the Distributed Entity of a NR node, see subclause 9.2.1.5 of 3GPP TS 38.473 [8]. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nCGI | It uniquely identifies a NR cell globally. The NCGI is constructed from the PLMN identity the cell belongs to and the NR Cell Identity (NCI) of the cell. See “NR Cell Global identifier (NCGI)” subcluase 8.2 of 3GPP TS 38.300 [3]. | type: BitString (36) multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| nRPCI | This holds the Physical Cell Identity (PCI) of the NR cell. allowedValues: See 3GPP TS 36.211 subclause 6.11 for legal values of pci. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nRTac | This holds the identity of the common Tracking Area Code for the PLMNs. allowedValues: a) It is the TAC or Extended-TAC. b) A cell can only broadcast one TAC or Extended-TAC. See TS 36.300, subclause 10.1.7 (PLMNID and TAC relation). c) TAC is defined in subclause 19.4.2.3 of 3GPP TS 23.003 [13] and Extended-TAC is defined in subclause 9.3.1.29 of 3GPP TS 38.473 [8]. | type: Bitstring multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nSSAI | It represents the types of NSSAI the managed object is capable of supporting, NSSAI is a set of supported S-NSSAI(s), an S-NSSAI is comprised of a SST (Slice/Service type) and an optional SD (Slice Differentiator) field, (Ref. 3GPP TS 23.003 [13]). | type: <<dataType>> multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| pointAArfcnULFDD | The frequency of the lowest usable UL subcarrier indexable on the cell, used as a reference, and indicated on a raster as NR-ARFCN. allowedValues: This value is given by the center frequency of resource element 0 in resource block 0 in the cell and indicated on a raster as NR-ARFCN. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| pointAArfcnDLFDD | The frequency of the lowest usable DL subcarrier indexable on the cell, used as a reference, and indicated on a raster as NR-ARFCN. allowedValues: This value is given by the center frequency of resource element 0 in resource block 0 in the cell and indicated on a raster as NR-ARFCN. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| pointAArfcnTDD | The frequency of the lowest usable subcarrier indexable on the cell, used as a reference, and indicated on a raster as NR-ARFCN. allowedValues: This value is given by the center frequency of resource element 0 in resource block 0 in the cell and indicated on a raster as NR-ARFCN. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rRMPolicy | It represents RRM policy which includes guidance for split of radio resources between multiple slices the cell supports. The RRM policy is implementation dependent. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tDDCenterFrequency | It is the frequency number for the central frequency. See 3GPP TS 38.104 [12] subclause 5.4.2. allowedValues: This value must be within one of the specified SectorEquipmentFunction.nRFqBands values. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tDDBandwidth | Specifies the frequency difference between the upper edge of the highest used carrier and the lower edge of the lowest used carrier. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| longitude | The longitude obtained from the GPS module of a gNB-DU based on World Geodetic System (1984 version) global reference frame (WGS 84). Positive values correspond to degrees east of 0 degrees longitude. allowedValues: -180.0000 to +180.0000 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| latitude | The latitude obtained from the GPS module of a gNB-DU based on World Geodetic System (1984 version) global reference frame (WGS 84). Positive values correspond to the northern hemisphere. allowedValues: -90.0000 to +90.0000 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |

End of Modified Section