3GPP TSG-SA5 Meeting #123	S5-191100

# Montreal,Canada, 21-25 January 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0036 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add Offline charging service selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | OFSBI_CH |  |  |  |  |  | Date: |  |  | 2019-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Nchf interface for 5G data network offline charging service is not specified in TS 32.255. The selection for offline charging service is needed. The offline charging service is different from offline charging method specified in PCF. The sequence for CHF instance selection should be, charging method selection, charging service selection and CHF selection. This rule would be used for SMF (CTF) to select charging service and CHF based on operator’s configuration. Those configurations for charging service selection should be set in SMF, UDM and/or PCF. Furthermore, CHF that stands for charging service might have to register it’s charging service to NRF. SMF may find proper charging service and CHF instance. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This contribution is to add text for offline charging selection for this issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No charging for online charging service and offline charging service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.2, 5.1.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change to TS 32.255

5.1.2	Requirements

The following are high-level charging requirements specific to the packet domain, derived from the requirements in TS 22.115 [101], TS 22.261 [102], TS 23.501 [200], TS 23.502 [201] and TS 23.503 [202].

-	The SMF shall support converged online and offline charging.

-	The SMF shall support offline charging only.

-	The SMF shall support PDU session charging using service based interface.

-	The SMF shall support network slice instance charging.

-	The SMF shall collect charging information per PDU session for UEs served under 3GPP access and non-3GPP access.

-	Every PDU session shall be assigned a unique identity number for billing purposes per PLMN. (i.e. the Charging Id).

-	Data volumes on both the uplink and downlink directions shall be counted separately. The data volumes shall reflect the data as delivered to and forwarded from the user.

-	The charging mechanisms shall provide the date and time information when the PDU session starts.

-	The SMF shall be capable of handling the Charging Characteristics. Charging Characteristics can be specific to a subscription or subscribed DNN.

-	The SMF may be capable of identifying data volumes, elapsed time or events for individual service data flows (flow based charging). One PCC rule identifies one service data flow.

-	SMF shall allow reporting of the service or the detected application usage per rating group or per combination of the rating group and service id. This reporting level can be activated per PCC rule.

-	The quota management shall be per rating group per PDU session.

-	If there are multiple UPFs for one PDU session, the quota management may be one for all UPFs or separate per UPF and the usage and charging information reporting per UPF.

-	The SMF shall support charging for PDU Session types of IP, Ethernet and Unstructured.

-	In Home Routed scenario, the SMF shall collect charging information per PDU session and, based on Home Operator policy and agreement between Home and Visit Operators, shall be able to collect charging information per Qos Flow for in-bound and out-bound roamers in Home Routed scenario.

-	For interworking between 5GS and EPC, the dedicated PGW-C + SMF shall collect charging information using the same mechanisms as the SMF.

Second change to TS 32.255

5.1.x	Charging service selection

SMF shall support charging service selection. Charging service selection parameters should be given by operator’s configuration. During PDU session establishment, the SMF shall choose charging method (s) and charging service for purpose of the CHF selection.

Charging service selection parameters should be provisioned to SMF as a defaut configuration. The CHF instance shall regisiter charging service profile to NRF. The CHF charging service and CHF profile information should be retrived by the SMF. The charging service selection rules may be suppred by PCC rules for PDU session.

The detail of CHF selection is specified in subclause 5.1.5 in this specification.

The example process to charging service selection and CHF selection is shown in figure 5.1.x-1.

Figure 5.1.x-1: Example process for SMF selection charging service and CHF address (es) retriaval