3GPP TSG-SA5 Meeting #126	S5-195398

# Bruges, Belgium, 19th Aug 2019 - 23rd Aug 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0052 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add Netconf-based solution set for provisioning MnS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ONAP3GPP |  |  |  |  |  | Date: |  |  | 2019-08-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | ONAP Controller is a potential consumer of 3GPP provisioning MnS.It is required that a new NETCONF-based solution set be specified for provisioning MnS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new NETCONF-based solution set for provisioning MnS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 12.1.X (new), A.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

1st Change

2	References

-	The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.526: "Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Procedures".

[3]	3GPP TS 28.541: "Management and orchestration ; 5G Network Resource Model (NRM); Stage 2 and stage3".

[4]	ITU-T Recommendation X.733 (02/92): "Information technology - Open Systems Interconnection - Systems Management: Alarm reporting function".

[5]	3GPP TS 28.531: "Management and orchestration of networks and network slicing; Provisioning; Stage 1".

[6]	3GPP TS 28.554: "Management and orchestration of networks and network slicing; 5G End to end Key Performance Indicators (KPI); performance measurements and assurance data".

[7]	3GPP TS 22.261: "Technical Specification Group Services and System Aspects; Service requirements for the 5G system; Stage 1".

[8]	3GPP TS 23.501: "Technical Specification Group Services and System Aspects; System Architecture for the 5G System; Stage 2".

[9]	3GPP TS 23.003: "Technical Specification Group Core Network and Terminals; Numbering, addressing and identification".

[10]	ETSI GS NFV-IFA 013 V2.4.1 (2018-02) "Network Function Virtualization (NFV); Management and Orchestration; Os-Ma-nfvo Reference Point - Interface and Information Model Specification".

[11]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[12]	ETSI GS NFV-IFA 015 (V2.4.1): "Network Function Virtualisation (NFV); Management and Orchestration; Report on NFV Information Model".

[13]	3GPP TS 28.533: "Management and orchestration; Architecture framework"

[14]	ITU-T Recommendation X.734 (1992): "Information technology - Open Systems Interconnection - Systems management: Event report management function".

[15]	3GPP TS 32.158: "Management and orchestration; Design rules for REpresentational State Transfer (REST) Solution Sets (SS) "

[16]	3GPP TS 32.302:	 "Telecommunication management; Configuration Management (CM); Notification Integration Reference Point (IRP); Information Service (IS)".

[17]	3GPP TS 32.401: "Telecommunication management; Performance Management (PM); Concept and requirements".

[18]	3GPP TS 28.552: " Management and orchestration; 5G performance measurements".

[19]	3GPP TS 32.401: "Perfomance Measurement (PM); Concept and requirements".

[20]	ISO 8601:2004: "Data elements and interchange formats – Information interchange – Representation of dates and times".

[x]	RFC 6241 “Network Configuration Protocol (NETCONF)”.

2nd Change

12.1.X. NETCONF-based solution set of provisioning

12.1.X.1 Mapping of operations

12.1.X.1.1 Introduction

The IS operations are mapped to SS equivalents according to table 12.1.X.1.1-1

Table 12.1.X.1.1-1: Mapping of 3GPP IS operations to SS equivalents [x]

| IS operation | NETCONF |  | Qualifier |
| --- | --- | --- | --- |
|  | Protocol operation | Parameter operation |  |
| createMOI | <edit-config> | create | M |
| getMOIAttributes | <get-config> | - | M |
| modifyMOIAttibutes | <edit-config> | replace | M |
| deleteMOI | <edit-config> | delete remove (note 1) | M |

Note 1: The "delete" and "remove" parameter operation are used to delete the identified configuration data. If the configuration data does not exist, the reply of "delete" will include an <rpc-error> element with an <error-tag> value of "data-missing". If the configuration data does not exist, the "remove" operation is silently ignored.

12.1.X.1.2 Operation “createMOI”

The IS operation parameters are mapped to SS equivalents according to table 12.1.X.1.2-1 and table 12.1.1X.1.2-2.

Table 12.1.X.1.2-1: Mapping from IS createMOI input parameters to SS equivalents

| IS operation parameter name | SS parameter location | SS parameter name | SQ |
| --- | --- | --- | --- |
| managedObjectClass, | config | mOClass: string | M |
| managedObjectInstance | config | mOInstance: string | M |
| referenceObjectInstance | config | refOInstance: string | O |
| attributeListIn | config | attribute name: string attribute value: any type(SS specific) | M |

Table 12.1.X.1.2-2: Mapping from IS createMOI output parameters to SS equivalents

| IS operation parameter name | SS parameter location | SS parameter name | SQ |
| --- | --- | --- | --- |
| attributeListOut | data | attribute name: string attribute value: any type(SS specific) | M |
| status | data | statuscode: string | M |

12.1.X.1.3 Operation "getMOIAttributes"

The IS operation parameters are mapped to SS equivalents according to table 12.1.X.1.3-1 and table 12.1.X.1.3-2.

Table 12.1.X.1.3-1: Mapping of IS getMOIAttributes input parameters to SS equivalents

| IS operation parameter name | SS parameter location | SS parameter name | SQ |
| --- | --- | --- | --- |
| baseObjectInstance | subtree filter | bOInstance: string | M |
| scope | subtree filter | base: string level: integer | M |
| filter | subtree filter | filtercondition: any type(SS specific) | M |
| attributeListIn | subtree filter | attribute name: string attribute value: any type(SS specific) | M |

Table 12.1.X.1.3-2: Mapping of IS getMOIAttributes output parameters to SS equivalents

| IS operation parameter name | SS parameter location | SS parameter name | SQ |
| --- | --- | --- | --- |
| managedObjectClass, | data | mOClass: string |  |
| managedObjectInstance | data | mOInstance: string |  |
| attributeListOut | data | attribute name: string attribute value: any type(SS specific) | M |
| status | data | statuscode:string | M |

12.1.X.1.4 Operation "modifyMOIAttributes"

The IS operation parameters are mapped to SS equivalents according to table 12.1.X.1.4-1 and table 12.1.X.1.4-2.

Table 12.1.X.1.4-1: Mapping of IS modifyMOIAttributes input parameters to SS equivalents

| IS operation parameter name | SS parameter location | SS parameter name | SQ |
| --- | --- | --- | --- |
| baseObjectInstance | config | bOInstance: string | M |
| scope | config | base: string level: integer | M |
| filter | config | filtercondition: any type(SS specific) | M |
| modificationList | config | attribute identifier: string attribute value: any type(SS specific) modify operator: string | M |

Table 12.1.X.1.4-2: Mapping of IS modifyMOIAttributes output parameters to SS equivalents

| IS operation parameter name | SS parameter location | SS parameter name | SQ |
| --- | --- | --- | --- |
| modificationListOut | data | mOClass: string mOInstance: string attribute name: string attribute value: any type(SS specific) | M |
| status | data | statuscode:string | M |

12.1.X.1.5 Operation "deleteMOI"

The IS operation parameters are mapped to SS equivalents according to table 12.1.X.1.5-1 and table 12.1.X.1.5-2.

Table 12.1.X.1.5-1: Mapping of IS deleteMOI input parameters to SS equivalents

| IS operation parameter name | SS parameter location | SS parameter name | SQ |
| --- | --- | --- | --- |
| baseObjectInstance | config | bOInstance: string | M |
| scope | config | base: string level: integer | M |
| filter | config | filtercondition: any type(SS specific) | M |

Table 12.1.X.1.5-2: Mapping of IS deleteMOI output parameters to SS equivalents

| IS operation parameter name | SS parameter location | SS parameter name | SQ |
| --- | --- | --- | --- |
| deletionList | data | mOClass: string mOInstance: string | M |
| status | data | statuscode:string | M |

3rd Change

A.x	NETCONF schema of provisioning

This clause describes the capabilities of the service in the structure of the NETCONF Specification. The NETCONF document is represented in the XML foramt option.

NETCONF XML:

<!-- createMO Request -->

<rpc message-id="101" xmlns="urn:ietf:params:xml:ns:netconf:base:1.0">

<edit-config>

<target>

<running/>

</target>

<config>

<top xmlns= "http://example.com/schema/1.2/config">

<mO xc: operation="create">

<mOClass>classname</mOClass>

<mOInstance>DN</mOInstance>

<refOInstance>DN</refOInstance>

<attributeListIn>

<mOAttribute>

<attribute name #1> name</ attribute name #1>

<attribute value> value</ attribute value>

</mOAttribute>

<mOAttribute>

<attribute name #2> name</ attribute name #2>

<attribute value> value</ attribute value>

</mOAttribute>

</attributeListIn>

</mO>

</top>

</config>

</edit-config>

</rpc>

<!-- createMO Response -->

<rpc-reply message-id="101" xmlns="urn:ietf:params:xml:ns:netconf:base:1.0">

<data>

<attributeListOut  xmlns= "http://example.com/schema/1.2/config">

<mOAttribute>

<attribute name  #1 > name</ attribute name  #1>

<attribute value> value</ attribute value>

</mOAttribute>

<mOAttribute>

<attribute name #2 > name</ attribute name  #2>

<attribute value> value</ attribute value>

</mOAttribute>

</attributeListOut>

<status>statuscode</status>

</data>

</rpc-reply>

<!-- getMOIAttributes Request -->

<rpc message-id="101" xmlns="urn:ietf:params:xml:ns:netconf:base:1.0">

<get-config>

<source>

<running/>

</source>

<filter type=”subtree”>

<top xmlns= "http://example.com/schema/1.2/config">

<mO>

<bOInstance>DN</bOInstance>

<scope>

<base>value</base>

<level>value</level>

</scope>

<filtercondition>filter condition description</filtercondition>

<attributeListIn>

<attribute name #1> name</ attribute name #1>

<attribute name #1> name</ attribute name #1>

<attribute name #1> name</ attribute name #1>

</attributeListIn>

</mO>

</top>

</filter>

</get-config>

</rpc>

<!-- getMOIAttributes Response -->

<rpc-reply message-id="101" xmlns="urn:ietf:params:xml:ns:netconf:base:1.0">

<data>

<mO  xmlns= "http://example.com/schema/1.2/config">

<mOClass>classname</mOClass>

< mOInstance>DN</mOInstance>

<attributeListOut>

<mOAttribute>

<attribute name  #1 > name</ attribute name  #1>

<attribute value> value</ attribute value>

</mOAttribute>

<mOAttribute>

<attribute name #2 > name</ attribute name  #2>

<attribute value> value</ attribute value>

</mOAttribute>

<attributeListOut>

<status>statuscode</status>

</mO>

</data>

</rpc-reply>

<!-- modifyMOIAttributes Request -->

<rpc message-id="101" xmlns="urn:ietf:params:xml:ns:netconf:base:1.0">

<edit-config>

<target>

<running/>

</target>

<config>

<top xmlns= "http://example.com/schema/1.2/config">

<mO xc: operation="replace">

<bOInstance>DN</bOInstance>

<scope>

<base>value</base>

<level>value</level>

</scope>

<filtercondition>filter condition description</filtercondition>

<modificationList>

<attribute identifier>id</attribute identifier>

<attribute value>value</attribute value>

<modify operator> operator</modify operator>

</modificationList>

</mO>

</top>

</config>

</edit-config>

</rpc>

<!-- modifyMOIAttributes Response -->

<rpc-reply message-id="101" xmlns="urn:ietf:params:xml:ns:netconf:base:1.0">

<data>

<modificationListOut xmlns= "http://example.com/schema/1.2/config">

<mOClass>classname</mOClass>

<mOInstance>DN</mOInstance>

<mOAttribute>

<attribute name  #1 > name</ attribute name  #1>

<attribute value> value</ attribute value>

</mOAttribute>

<mOAttribute>

<attribute name #2 > name</ attribute name  #2>

<attribute value> value</ attribute value>

</mOAttribute>

</modificationListOut>

<status>statuscode</status>

</data>

</rpc-reply>

<!-- deleteMOI Request -->

<rpc message-id="101" xmlns="urn:ietf:params:xml:ns:netconf:base:1.0">

<edit-config>

<target>

<running/>

</target>

<config>

<top xmlns= "http://example.com/schema/1.2/config">

<mO xc:operation="delete">

<bOInstance>DN</bOInstance>

<scope>

<base>value</base>

<level>value</level>

</scope>

<filtercondition>filter condition description</filtercondition>

</mO>

</top>

</config>

</edit-config>

</rpc>

<!-- deleteMOI Response -->

<rpc-reply message-id="101" xmlns="urn:ietf:params:xml:ns:netconf:base:1.0">

<data>

<deletionList xmlns= "http://example.com/schema/1.2/config">

<mOClass>classname</mOClass>

< mOInstance>DN</mOInstance>

</deletionList>

<status>statuscode</status>

</data>

</rpc-reply>

End of 3rd Change