3GPP TSG-SA5 Meeting #126	S5-195072

# Bruges, Belgium, 19th Aug 2019 - 23rd Aug 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.291 | CR | 0083 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-15 CR 32.291 Correction of TriggerCategory and Triggers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile Com. Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SBI_CH |  |  |  |  |  | Date: |  |  | 2019-07-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Mismatched category attribute name in 6.1.6.2.1.7 Type Trigger; Mismatched trigger attribute name in 6.1.6.2.2.15 Type RoamingChargingProfile. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct attribute name in 6.1.6.2.1.7 and 6.1.6.2.2.15 mapping with A.2 Nchf_ ConvergedCharging API definition. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Related SBI Charging implementation will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.1.7, 6.1.6.2.2.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

###### 6.1.6.2.1.7	Type Trigger

Table 6.1.6.2.1.7-1: Definition of type Trigger

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| triggerType | TriggerType | Oc | 0..1 | the events whose occurrence lead to charging event is issued towards the CHF |  |
| triggerCategory | TriggerCategory | M | 1 | This field indicates whether the charging data generated by the NF consumer for the trigger lead to a Charging Event towards the CHF immediately or not. |  |
| timeLimit | DurationSec | OC | 0..1 | Time limit if trigger type is "Expiry of data time limit" |  |
| volumeLimit | Uint32 | OC | 0..1 | Volume limit if trigger type is "Expiry of data volume limit". This attribute is not valid from Nchf_ ConvergedCharging API version v2.0.0 |  |
| volumeLimit64 | Uint64 | OC | 0..1 | Volume limit if trigger type is "Expiry of data volume limit". This attribute replaces the volumeLimit attribute from Nchf_ ConvergedCharging API v2.0.0 |  |
| maxNumberOfccc | Uint32 | OC | 0..1 | Maximum number if trigger type is "Max nb of number of charging condition changes" |  |

Second change

###### 6.1.6.2.2.15	Type RoamingChargingProfile

Table 6.1.6.2.1.15-1: Definition of type RoamingChargingProfile

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| triggers | array(Trigger) | OC | 0..N | Trigger for roaming QBC |  |
| partialRecordMethod | PartialRecordMethod | Oc | 0..1 | method uses for partial record closure |  |

End of change