3GPP TSG-SA5 Meeting #126	S5-195236

# Bruges (Belgium), 19 - 23 August 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR |  | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [Draft CR] Introduce Stage 2 and 3 for the heartbeat management service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T, Deutsche Telekom, Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ONAP3GPP |  |  |  |  |  | Date: |  |  | 2019-08-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Stage 1 for the heartbeat management service has been specified. Stage 2 and Stage 3 are required. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce Stage 2 and Stage 3 for the heartbeat management service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 and Stage 3 for the heartbeat management service would not exist though Stage 1 exists. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.X (new), A.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

# 10	Management services – Stage 2

## 10.X	Heartbeat management service

### 10.X.1	Operations and notifications

#### 10.X.1.1	getHeartbeatPeriod operation

##### 10.X.1.1.1	Description

This operation is invoked by Heartbeat MnS consumer to obtain the current heartbeat period for the Heartbeat MnS provider.

##### 10.X.1.1.2	Input parameters

| Parameter Name | Support Qualifier | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |

##### 10.X.1.1.3	Output parameters

| Parameter name | Support Qualifier | Matching Information / Legal Values | Comment |
| --- | --- | --- | --- |
| heartbeatPeriod | M | HBMnSProvider.heartbeatPeriod |  |
| status | M | ENUM (OperationSucceeded, OperationFailed) |  |

##### 10.X.1.1.4	Results

In case of success, the HBMnSProvider instance heartbeat period value is returned. In case of failure, indication of the failure is provided in the Output parameters.

#### 10.X.1.2	setHeartbeatPeriod operation

##### 10.X.1.2.1	Description

This operation is invoked by the Heartbeat MnS consumer to modify the heartbeat period for the HBMnSProvider.

##### 10.X.1.2.2	Input parameters

| Parameter Name | Support Qualifier | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| heartbeatPeriod | M | HBMnSProvider.heartbeatPeriod | The new value of the HBMnSProvider.heartbeatPeriod |

##### 10.X.1.2.3	Output parameters

| Parameter name | Support Qualifier | Matching Information / Legal Values | Comment |
| --- | --- | --- | --- |
| status | M | ENUM (OperationSucceeded, OperationFailed) |  |

##### 10.X.1.2.4	Results

In case of success, the HBMnSProvider heartbeat period value is modified. In case of failure, indication of the failure is provided in the Output parameters.

#### 10.X.1.3	triggerHeartbeat operation

##### 10.X.1.3.1	Description

This operation is invoked by the Heartbeat MnS consumer to solicit a notifyHeartbeat notification.

After the successful completion of the operation, the Heartbeat MnS provider shall emit the notifyHeartbeat notification as specified in clause 10.X.1.4 immediately. One notification shall be emitted as follows:

a)	one notification to the soliciting Heartbeat MnS consumer, or

b)	one notification to each of the subscribed Heartbeat MnS consumers.

If the operation fails, an indication of the failure shall be provided in the Output parameters and the notification shall not be emitted.

One of the two options above (a or b) shall be chosen depending on system configuration.

Before invoking this operation, the soliciting Heartbeat MnS consumer should make sure it has subscribed to the notifyHeartbeat notification.

##### 10.X.1.3.2	Input parameters

| Parameter Name | Support Qualifier | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| hBMnSConsumerRef | M | DN |  |

##### 10.X.1.3.3	Output parameters

| Parameter name | Support Qualifier | Matching Information / Legal Values | Comment |
| --- | --- | --- | --- |
| status | M | ENUM (OperationSucceeded, OperationFailed) |  |

##### 10.X.1.3.4	Results

In case of success, the HBMnSProvider sends a notifyHeartbeat notification according to option a or b. In case of failure, indication of the failure is provided in the Output parameters.

#### 10.X.1.4	Notification notifyHeartbeat

##### 10.X.1.4.1	Definition

This notification notifies the subscribed consumer(s) that the HBMnSProvider heartbeat period has expired.

##### 10.X.1.4.2	Input parameters

| Parameter Name | Support Qualifier | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| heartbeatPeriod | M | HBMnSProvider.heartbeatPeriod | The value of the HBMnSProvider.heartbeatPeriod |
| triggerFlag | M | ENUM {HBMnSConsumer, HBMnSProvider} | If this notification is triggered by HBMnS consumer positively by invoking "triggerHeartbeat" operation (cf. clause 10.X.1.3), the value of this parameter shall be "HBMnSConsumer". Otherwise, it shall be "HBMnSProvider". |
| hBMnSProviderRef | M | String | If the value of triggerFlag is "HBMnSConsumer", this field has the same value as the input parameter "hBMnSConsumerRef" of the "triggerHeartbeat" operation (cf. clause 10.X.1.3). |

##### 10.X.1.4.3	Triggering event

###### 10.X.1.4.3.1	From-state

stateBeforeHeartbeatNotification.

| Assertion Name | Definition |
| --- | --- |
| stateBeforeHeartbeatNotification |  |

###### 10.X.1.4.3.2	To-state

stateAfterOHeartbeatNotification.

| Assertion Name | Definition |
| --- | --- |
| stateAfterHeartbeatNotification |  |

### 10.X.2	Managed Information

#### 10.X.2.1	Information Object Class definitions

##### 10.X.2.1.1 	HBMnSProvider

###### 10.X.2.1.1.1	Definition

HBMnSProvider is the representation of the entity which provides the heartbeat management service(s) and contains the heartbeat mechanism.

###### 10.X.2.1.1.2	Attribute

| Attribute Name | Support Qualifier |
| --- | --- |
| hBMnSProviderRef | M |
| heartbeatPeriod | M |

###### 10.X.2.1.1.3	Notification Table

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyHeartbeat | M | - |

#### 10.X.2.2	Information attribute definition

##### 10.X.2.2.1	Definition and legal values

| Name | Definition | Legal Values |
| --- | --- | --- |
| hBMnSProviderRef | It identifies uniquely the HBMnSProvider. |  |
| heartbeatPeriod | It specifies the time between two emissions of heartbeat notifications. A value of zero implies there is no heartbeat emission. The unit is minute. | Type: Integral numeric value Range: value range of heartbeat period is from 5min to 60min, 0 is also a legal value. |

Next change

Annex A (normative): 
OpenAPI specification

## A.0	Introduction

This clause describes the capabilities of the service in the structure of the OpenAPI Specification Version 3.0.1 [A9]. The OpenAPI document is represented in the JSON format option.

## A.X	Heartbeat management service

### A.X.1	Generic heartbeat management service

Editor’s note: this is FFS.

### A.X.2	Heartbeat management service for integration with ONAP VES

Editor’s note: this is FFS.

End