3GPP TSG-SA5 Meeting #126	S5-195377

# Bruges, Belgium, 19th Aug 2019 - 23rd Aug 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0125 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modify the definition of Round-trip GTP Data Packet Delay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2019-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The N3 interface is between NG-RAN and UPF. Consequently, in non-split NG-RAN scenarios, the round-trip GTP Data packet delay on N3 interface should be the round-trip packet delay between gNB and UPF while in 2-split ng-RAN scenarios should be the delay between gNB-CU and UPF. In 3-split scenarios, it should be the dealy between gNB-CU-UP and UPF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify the definitions of Round-trip GTP Data Packet Delay on N3 interface. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The measurement of round-trip GTP Data Packet Delay in N3 interface is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First of changes

5.4.1.9	Round-trip GTP Data Packet Delay

a)	This measurement provides a round-trip GTP packet delay on a N3 interface.

b)	DER (n=1).

c)	This measurement is obtained as: the time when receiving a GTP echo reply message from the gNB in non-split NG-RAN deployment scenarios (gNB-CU in 2-split NG-RAN deployment scenarios or gNB-CU-UP in 3-split NG-RAN deployment scenarios)  at UPF’s ingress GTP termination, minus time when sending the associated echo request message to gNB-CU at the UPF’s GTP egress termination. This measurement represents a single measurement and can be repeated regularly or on demand during the lifetime of the target tunnel.

d)	Each measurement is an integer representing the delay in microseconds.

e)	The measurement name has the form GTP.InDataPktPacketDelayN3UPF.

f)	EP_N3.

g)	Valid for packet switched traffic.

h)	5GS.

End of changes