3GPP TSG-SA5 Meeting #126	S5-195378

# Bruges, Belgium, 19th Aug 2019 - 23rd Aug 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0126 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add Packet Drop Rate related measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2019-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SDAP is a new layer for the NG-RAN user plane protocol, located in the upper of PDCP layer. SDAP layer is adapted to the 5G QoS mechanism and holds the function of the mapping of QoS flows and DRBs. Since SDAP is associated with QoS, the packet drop rate of services with different QoS requests can be easily obtained in this layer which is an important indicater to evaluate the reliability of the network. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add measurements related to Packet Drop Rate. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The measurements related to Packet Drop Rate of SDAP will not be monitored. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.x.1(new), A.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First of changes

5.1.1.X	Packet Drop Rate

5.1.1.X1	SDAP Packet Drop Rate in DL

- This measurement provides the rate of SDAP SDU packets which are not successfully received by RAN network system. It is a measurement shows the network reliability.  Only user-plane traffic (DTCH) and SDAP SDUs received by SDAP are considered.  The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3).

Unit: number of discarded packets per received packets * 106

- SI.

- This measurement is obtained as: 1000000*Number of missing SDAP SDU packets, representing packets that are not delivered to egress interface, of a data radio bearer, divided by total number of SDAP SDU packets (including the missing SDAP SDU packets) of a bearer, starting from the sequence number of the first packet delivered by UE SDAP to gNB-CU-UP until the sequence number of the last packet. If transmission of a packet might continue in another cell, it shall not be included in this count. Separate counters are optionally maintained for mapped 5QI (or QCI for NR option 3).

- Each measurement is an integer value representing the number of discarded packets per received packets * 106. The number of measurements is equal to one. If the optional QoS level measurements is performed, the measurements are equal to the number of mapped 5QIs.

- The measurement name has the form DRB. SDAPPacketDropRateDl or optionally DRB. SDAPPacketDropRateDl.QOS, where QOS identifies the target quality of service class.

- GNBCUUPFunction.

NRCellCU.

- Valid for packet switched traffic.

- 5GS.

- One usage of this measurement is for reliability assurance within integrity area (user plane connection quality). NRCellCU measurement applies only for 2-split deployment.

Second of changes

A.X	RAN network reliability measurement

SDAP Packet Drop Rate in DL

SDAP is a new layer for the NG-RAN user plane protocol, located in the upper of PDCP layer. SDAP layer is adapted to the 5G QoS mechanism and holds the function of the mapping of QoS flows and DRBs.

Since SDAP is associated with QoS, the packet drop rate of services with different QoS requests can be easily obtained in this layer which is an important indicater to evaluate the reliability of the network.

End of changes