3GPP TSG-SA5 Meeting #126	S5-195380

# Bruges, Belgium, 19th Aug 2019 - 23rd Aug 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0026 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add Analytical KPI of User Data Suppression Level |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2019-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | User data volume in downlink demanded by end users grows rapidly in 5GS with the development of diverse communication services.This KPI reveals in which degree the end users’ requests are suppressed in order to help operators to performe the network planning or network optimization. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add analytical KPI of User Data Suppression Level. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The problem of user data suppression will not be monitored. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4, 6.x.1(new), A.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First of changes

4	End to end KPI concept and overview

The following KPI categories are included in the present document:

-	Accessibility (see the definition in [3]).

-	Integrity (see the definition in [3]).

-	Utilization

-	Analytics

For future update of the document it will also include:

-	Retainability (see the definition in [3]).

-	Availability.

-	Mobility.

Second of changes

6.X	Analytics KPI

6.X.1	User Data Volume Suppression level in downlink

a)	The level of user data volume suppression in downlink consumed by end users.

b)	This KPI describes the severity of the data volume suppression. It is used to evaluate in which degrees the end users’ demands for data volume are unsatisfied.

c)	This KPI is an integer which shows the suppression level of user data in downlink. The KPI is derived from analysing the measurements related with data volumes, end users IP throughput and resource utilizations. The range of the KPI values depends on the implementation.

d)	DRB.SuppLevel= analytics (RRU.PrbTotDl, RRU.PrbTotDlDist.BinX, DRB.UEThpDl, DRB.UEThpDlDist.BinX, DRB.PdcpSduVolumeDL_Filter)

e)	Measurement names used for the user data volume suppression analysing

| Performance Measurements Category | Input performance measurements | Source | Description |
| --- | --- | --- | --- |
| UE throughput | DRB.UEThpDl, DRB.UEThpDlDist.BinX | PM data | The IP throughput of end users in downlink, see clause 5.1.1.3 of TS 28.552. |
| Radio resource utilization | RRU.PrbTotDl, RRU.PrbTotDlDist.BinX | PM data | The usage of downlink physical radio resource utilization of the network, see clause 5.1.1.2 of TS 28.552. |
| PDCP Data Volume | DRB.PdcpSduVolumeDL_Filter | PM data | The number of PDCP data volume transmitted in downlink, see clause 5.1.2.1 and 5.1.3.6 of TS 28.552. |

f)	NG-RAN

g)	Analytics

h)	Time interval (microsecond)

i)	Mean

Third of changes

A.X	Use Case for Analytics KPI

Use case for User Data Suppression

User data volume in downlink demanded by end users grows rapidly in 5GS with the development of diverse communication services. This KPI reveals in which degree the end users’ requests are suppressed in order to help operators to perform the network planning or network optimization.

End of changes